# Major non-compliance: engineering

Table 1: Major impact of the authorised activity

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Any authorisation condition | Non-compliance causes or is likely to cause: - Category 1 or 2 environmental event – this includes significant morphological impact (e.g. erosion, instability). - Breach of water standard. | - Environmental events guidance. - Relevant water standards: - Groundwater monitoring, investigation or modelling demonstrates significant and sustained adverse trend. | Demonstration that the cause or likely cause of the harm has ceased or been resolved. This can include monitoring and modelling. |
| Descriptive conditions No significant: - Iridescence / sheen; - Discolouration; - Deposition of solids; - Increased foaming; or - Microbiological growth; outside of site. | Non-compliance causes or is likely to cause Category 1 or 2 environmental events. | - Site inspection. - Environmental events guidance. | Demonstration that the cause or likely cause of the harm has ceased or been resolved and the visual is now compliant. |

Table 2: Scope of site

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Location | Engineering activities being undertaken at a location not specified in authorisation. This is a site-specific assessment that will be based on scale of location breach, the waterbody size and sensitivity. Guidance on what constitutes non-compliance versus major non-compliance will be developed. | Site inspection - NGR check. | Demonstration that the cause of the breach is resolved and mitigation actions implemented. |
| Timescales | Undertaking works during non-permitted periods e.g. working during periods when fish spawning and emergence of juveniles. | Site inspection or evidence from a third party e.g. date stamped photos or video. | Cessation of activities. |
| Design drawings | Designs (conditions or drawings) not followed leading to increased flood or drought risk, barrier to fish passage, impact on other infrastructure and / or other water users. | - Site inspection e.g. comparison of ‘as built’ compared to an approved design drawing. - Investigation by inspecting officer. | Site specific assessment or remote evidence submitted e.g. video evidence that required mitigations have been implemented to resolve the impacts of the design breaches. |

Table 3: Understanding of authorised activities

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Requirement to report incidents (either in operation of plant or environmental events). | Unless specified otherwise in the authorisation, failure to notify SEPA within 24 hours of an event or incident that has caused or could cause an adverse impact upon the environment or human health. Failure to submit an incident report within 14 days. | - Operator incident notification received within timescale (by phone call or email). - Operator incident report submitted in accordance with authorisation conditions. - Assessment of operator incident report and the timescale in which it was submitted. “Incident” means any of the following situations: accidents; malfunction, breakdown, or failure of plant or techniques; or any event such as force majeure or emergency actions that results or is likely to result in a breach of any condition of the authorisation. | - Date notification is received. - Date incident report is received. - The operator has notified SEPA of the incident or environmental event and has provided a report of adequate quality. |

Table 4: Overall management of authorised activities

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Compliance with authorisation conditions | Multiple non-compliance (same or different conditions) that cumulatively equate to major non-compliance. The major non-compliance will be recorded against the condition that places the responsibility on the Authorised Person to ensure compliance with the conditions of the authorisation. | Having regard to condition-specific guidance where this exists and environmental events guidance and events records. - More than or equal to four Category 3 events within a 365-day period that have been caused by a breach of the same condition. - Unless otherwise stated within the site authorisation, multiple non-compliance breaches may be considered collectively as major non-compliance depending on the particular circumstances. SEPA's assessment will be subject to additional governance checks to ensure national consistency in decision-making. | Site specific assessment and discussion |
| Completion, restoration, mitigation or clean up conditions | Completion, restoration, mitigation or clean-up conditions not complied with. Examples: - Silt mitigation failure causes extensive silt pollution i.e. equates to Category 1 or 2 environmental event threshold. | Investigation by inspecting officer. | Site specific assessment and discussion. |

Table 5: Wider legal environmental requirements and financial non-compliance

EPAS is designed to provide a more accurate picture of legal environmental compliance. We will therefore assess compliance of the following situations under EPAS as it is important a performance rating reflects this.

| Contravention | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Undertaking a regulated activity without authorisation | Undertaking a regulated activity without authorisation, for example: - Unauthorised bank re-profiling at site authorised for sediment removal. - Unauthorised embankments created at site authorised for sediment removal. | - Site inspection. | Unauthorised activity ceases, and, where appropriate, authorisation is varied to include the activity or appropriate authorisation obtained. |
| Environmental event at an authorised site that is unrelated to the authorised activity | Causes a: - Category 1 or 2 environmental event. - Breach of water or air quality standard. - Significant and sustained adverse trend for groundwater. For example: - Unauthorised engineering activity causes extensive silt pollution. - Hydrocarbon pollution associated with on-site fuel storage. - Biosecurity hazards caused with inappropriate disposal of invasive species when carrying out authorised bank reprofiling. | - Environmental events guidance. - Groundwater monitoring / investigation shows, or modelling demonstrates, significant and sustained adverse trend. | Demonstrating the cause or likely cause of the harm has ceased or been resolved. This can include monitoring and modelling. |
| Non-payment of annual charges | Escalation to debt collection or if debt goes into the next financial year. (Non-compliance is when payment is more than 90 days late.) | - Monthly debtors report. | Payment of annual charges. (Payment plan agreed with SEPA is equivalent to a compliance recovery plan.) |

<Report date here (month, year)>

For information on accessing this document in an alternative format or language, please contact SEPA by email or telephone 0300 099 6699.

If you are a user of British Sign Language (BSL), the Contact Scotland BSL service gives you access to an online interpreter, enabling you to communicate with us using sign language.

Adequate quality means containing the required information to enable the assessment of compliance with authorisation conditions.