WAS-G-DEF-06

Version 2.0 – June 2025

End-of-waste for compost

Contents

Purpose	2

End-of-waste criteria	2

Waste acceptance	2

Use of compost	3

Record keeping	3

Loss of product status	4

Annex 1 – Wastes acceptable to produce compost	5

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk

## Purpose

This guidance sets out the end-of-waste criteria for compost produced from waste in open and in-vessel processes.

## End-of-waste criteria

When sold and dispatched from the site to the end-user for a certain use, compost meeting the following criteria will cease to be waste:

- Compost is produced from source segregated biowastes and/or source segregated biodegradable materials listed in Annex 1 only.

- The composting process and any compost produced is certified to the standards contained in BSI PAS100:2018 Specification for Composted Materials and the Additional Scheme Rules for Scotland.

- Plastic >2mm must not be present at levels above 0.06% by weight in air dried compost.

- The certification process must be carried out by a third party accredited by the United Kingdom Accreditation Service to carry out this certification.

- The compost must meet PAS100:2018 without having to be blended with any other materials including other composts, digestates, materials, products or additives.

- The compost must not require any further processing or recovery operations. Use as a ‘raw material’ in a manufacturing/blending process is acceptable providing the certified compost is not mixed with waste.

## Waste acceptance

The producer must have in place ‘waste acceptance criteria’ to ensure that only wastes listed in Annex 1 are accepted for composting.

Steps must have been taken to exclude contamination from the feedstock. This includes invasive plant species such as Giant Hogweed, Japanese Knotweed, Himalayan Balsam and toxic species such as Ragwort and Yew.

Remove packaging that is not independently certified as industrially or home compostable (or both). Packaging that is independently certified as compliant with at least one of the following can go forward for composting:

- EN 13432.

- EN 14995.

- ASTM D6400.

- EN 17427.

- AS 5810-2010.

- NF T51-800.

- TUV Austria’s certification requirements for home compostable packaging under their “OK compost HOME scheme”.

Only untreated wood is allowed. Wood and wood-derived wastes impregnated with preservatives, painted, or with any non-biodegradable layer is not permitted.

## Use of compost

Compost must only be used in the right quantity, at the right time of year and with the right frequency of application to deliver the desired crop need or agricultural benefit. The use of the compost must comply with all other regulatory controls and in accordance with best practice. For use in agriculture, such requirements include:

- Diffuse Pollution General Binding Rule 18: Fertiliser storage and application.

- Prevention of Environmental Pollution from Agricultural Activity Code of good practice (PEPFAA code).

- Nitrate Vulnerable Zones Regulations (NVZ regulations).

## Record keeping

Producers must retain records of all testing and of each sale of compost. This should include the following:

- Date of supply.

- Customer’s name, contact details and nature of business.

- Quantity supplied by weight/volume.

- The specification with which the digestate complies.

- A statement that the digestate was produced in compliance with this guidance.

## Loss of product status

Compost meeting these end-of-waste criteria will become waste again if at any stage:

- It is discarded, or the holder intends to or is required to discard it.

- It is applied in excess of soil and crop requirements.

- It is stored indefinitely with little prospect of being used.

This applies to anyone holding stores of compost, not just producers.

## Annex 1 – Wastes acceptable to produce compost

| EWC Code | Description |
| --- | --- |
|  | Wastes from agriculture, horticulture, aquaculture, forestry, hunting and fishing, food preparation and processing |
| 02 01 01 | Sludges from washing and cleaning − vegetables, fruit and other crops |
| 02 01 02 | Animal tissue waste |
| 02 01 03 | Plant tissue waste - including straw, other crop residues, riverine vegetation and spent growing media based on plant issues such as compost derived from source-segregated biodegradable waste, peat and bark. |
| 02 01 06 | Animal faeces, urine and manure - including spoiled fully biodegradable bedding |
| 02 01 07 | Waste from forestry – green waste and plant tissue only |
| 02 01 99 | Wastes not otherwise specified – spent mushroom compost or discarded mushrooms from commercial mushroom growing only |
|  | Wastes from the preparation and processing of meat, fish and other foods of animal origin |
| 02 02 01 | Sludges from washing and cleaning, peeling, centrifuging and separation including wash waters and sludges from secondary food processing or the cook chill sector |
| 02 02 02 | Animal tissue waste – may include blood and animal gut contents |
| 02 02 03 | Material unsuitable for consumption or processing - may include gut contents, shells and shellfish wastes. Includes sludges from gelatine production |
| 02 02 04 | Sludges from on-site effluent treatment |
|  | Wastes from fruit, vegetables, cereals, edible oils, cocoa, coffee, tea and tobacco preparation and processing, conserve production, yeast and yeast extract production, molasses preparation and fermentation |
| 02 03 01 | Sludges from washing, cleaning peeling, centrifuging and separation from food processing only |
| 02 03 04 | Materials unsuitable for consumption or processing - biodegradable waste only. Allowed only if no chemical additives or toxin residues present |
| 02 03 05 | Sludges from on-site effluent treatment or processing sludge from production of edible fats and oils including seasoning residues |
|  | Wastes from sugar processing |
| 02 04 03 | Sludges from on-site effluent treatment - biological sludge only |
|  | Wastes from the dairy products industry |
| 02 05 01 | Materials unsuitable for consumption or processing - including raw milk |
| 02 05 02 | Sludges from on-site effluent treatment |
|  | Wastes from the baking and confectionary industry |
| 02 06 01 | Materials unsuitable for consumption or processing -biodegradable waste only |
| 02 06 03 | Sludges from on-site effluent treatment |
|  | Wastes from the production of alcoholic and non-alcoholic beverages (except coffee, tea and cocoa) |
| 02 07 01 | Wastes from washing, cleaning and mechanical reduction of raw materials – biodegradable wastes from the processing of the raw materials used in the production of alcoholic and non-alcoholic beverages (except coffee, tea and cocoa) |
| 02 07 02 | Wastes from spirits distillation – spent grains, hops and whisky filter sheets and cloths, yeast and yeast like residues, sludge from production process, or malt husks, malt sprouts, yeasts and yeast-like residues only |
| 02 07 04 | Material unsuitable for consumption or processing – biodegradable wastes from the processing of the raw materials used in the production of alcoholic and non-alcoholic beverages (except coffee, tea and cocoa) |
| 02 07 05 | Sludges from on-site effluent treatment – sludges from the production of alcoholic and non-alcoholic beverages (except coffee, tea and cocoa) |
|  | Wastes from wood processing and the production of panels and furniture, pulp, paper and cardboard |
| 03 01 01 | Waste bark and cork - virgin timber only |
| 03 01 05 | Sawdust, shavings, cuttings, wood and particle board other than those in 03 01 04 only – virgin timber only |
|  | Wastes from pulp, paper, and cardboard production and processing |
| 03 03 01 | Waste bark and wood – virgin timber only |
| 03 03 10 | Fibre rejects – virgin timber only |
| 03 03 11 | Sludges from on-site effluent treatment other than those mentioned in 03 03 10. Only allowed if not mixed with or does not contain, de-inking sludge. |
|  | Wastes from the textile industry |
| 04 01 01 | Hair from hide processing |
| 04 02 10 | Organic matter from natural products such as grease and wax – biodegradable waste only |
| 04 02 21 | Wastes from unprocessed textile fibres – biodegradable waste only |
|  | Waste from the manufacture, formulation, supply and use of plastics, synthetic rubber and man-made, fibres |
| 07 02 13 | Waste plastic - unused and uncontaminated excess production certified to a relevant compostable standard only. |
|  | Waste from the manufacture, formulation, supply and use of plastics, synthetic rubber and man-made, fibres |
| 15 01 01 | Paper and cardboard packaging (excluding veneers, plastic coatings or laminates) certified to a relevant compostable standard. |
| 15 01 02 | Plastic packaging – certified to a relevant compostable standard |
| 15 01 03 | Wooden packaging – virgin timber only |
| 15 01 05 | Composite packaging – certified to a relevant compostable standard |
| 15 01 09 | Textile packaging – made entirely from biodegradable fibres only |
| 15 02 03 | Absorbents, filter materials and cloths from the production of alcoholic and non-alcoholic beverages other than those mentioned in 15 02 02 made from compostable material only – hops and whisky filter sheets and cloths made from compostable material only |
|  | Wastes not otherwise specified in the list |
| 16 03 06 | Organic wastes other than those mentioned in 16 03 05 – wool fleece only (excludes hides and skins) |
| 16 10 02 | Aqueous liquid wastes other than those mentioned in 16 10 01 - Milk and Dairy waste milk from agricultural premises only. - Digestate or liquor from an anaerobic process that accepts only waste input types allowed by this guidance. - Liquor or leachate from a composting process that accepts waste input types listed in this guidance only and in compliance with Animal By-Products Regulations. - Untreated wash waters from cleaning fruit and vegetables on farm only |
|  | Soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 06 | Dredging spoil other than those mentioned in 17 07 05 - riverine vegetation only (no associated dredged mineral material). HACCP assessment must consider pollutants that may be present and determine adequate risk control is feasible. |
|  | Wastes from physico-chemical treatments of waste |
| 19 02 03 | Premixed wastes composed only of non-hazardous wastes - only if derived solely from input types allowed by this guidance and remains segregated from, and uncontaminated by any other waste type. |
| 19 02 06 | Sludges from physico/chemical treatment other than those mentioned in 19 02 05 - only if derived solely from physical treatment and/or pH adjustment of input types allowed by this guidance and remains segregated from, and uncontaminated by, any other waste type. |
|  | Wastes from aerobic treatment of solid wastes |
| 19 05 01 | Non-composted fraction of municipal and similar wastes - from composting process that accepts only wastes as listed in this guidance and made up of previously sanitised batches only |
| 19 05 02 | Non-composted fraction of animal and vegetable waste- from composting process that only accepts wastes as listed in this guidance and made up of previously sanitised batches only |
| 19 05 03 | Off-specification compost - from composting process that only accepts wastes as listed in this guidance and made up of previously sanitised batches only. This includes oversize material resulting from screening such compost. |
|  | Wastes from anaerobic treatment of solid wastes |
| 19 06 03 | Liquor from anaerobic treatment of municipal waste - from a process that accepts wastes listed in the anaerobic digestion end-of-waste guidance and made up of previously pasteurised and stabilised batches only. Wastes shall not contain wastes derived from mechanical biological treatment facilities. |
| 19 06 04 | Digestate from anaerobic treatment of municipal waste - from a process that only accepts wastes listed in the anaerobic digestion end-of-waste guidance, independently certified as complying with BSI PAS 110 and the Scottish Scheme rules. Includes previously pasteurised and stabilised batches only and in compliance with Animal and Plant Health Agency approval. Wastes shall not contain wastes derived from mechanical biological treatment facilities. |
| 19 06 05 | Liquor from anaerobic treatment of animal and vegetable waste from a process that only accepts wastes listed in the anaerobic digestion end-of-waste guidance independently, certified as complying with BSI PAS 110 and the Scottish Scheme rules. Includes previously pasteurised and stabilised batches only and in compliance with Animal and Plant Health Agency approval. Wastes shall not contain wastes derived from mechanical biological treatment facilities. |
| 19 06 06 | Digestate from anaerobic treatment of animal and vegetable waste from a process that only accepts wastes listed in the anaerobic digestion end-of-waste guidance independently, certified as complying with BSI PAS 110 and the Scottish Scheme rules. Includes previously pasteurised and stabilised batches only and in compliance with Animal and Plant Health Agency approval. Wastes shall not contain wastes derived from mechanical biological treatment facilities. |
|  | Wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 01 | Paper and cardboard (excluding veneers or plastic coatings) certified to a relevant compostable standard |
| 19 12 12 | Other wastes (including mixtures of materials) from mechanical treatment of wastes other than those mentioned in 19 12 11 - only if derived solely from input types allowed by this guidance and remains segregated from, and uncontaminated by, any other waste type |
|  | Municipal waste (household waste and similar commercial, industrial and institutional wastes) including separately collected fractions |
| 20 01 01 | Paper and cardboard (excluding non-biodegradable coatings or preservatives such as veneers, plastic coatings or laminates) certified to a relevant compostable standard |
| 20 01 08 | Biodegradable kitchen and canteen waste |
| 20 01 25 | Edible oil and fat |
| 20 01 39 | Plastics certified to a relevant compostable standard |
|  | Garden and park wastes (including cemetery waste) |
| 20 02 01 | Biodegradable waste - Manure, garden waste, green waste, horticultural waste, plant tissue, parks and garden waste, hedge and tree trimmings, grass cuttings and leafy materials. Waste types in this section allowed if biodegradable material only, with no chemical additives and no toxic residues present. Excludes road sweeping, gully waste and dog faeces. The waste must not contain invasive non-native species such as Japanese Knotweed or toxic species such as Yew or ragwort. |
|  | Other municipal wastes |
| 20 03 01 | Mixed municipal waste - separately collected biodegradable wastes otherwise allowed by this guidance only. If former foodstuffs are packaged, packaging must be certified to a relevant compostable standard. |
| 20 03 02 | Waste from markets allowed only if source segregated biodegradable fractions only. If former foodstuffs are packaged, packaging must be certified to a relevant compostable standard. |