# Major non-compliance: industrial activities

Table 1: Major impact of the authorised activity

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Any authorisation condition. | Non-compliance causes or is likely to cause: - category 1 or 2 environmental event. - breach of water or air quality standard. - significant and sustained adverse trend for groundwater. | - Environmental events guidance. - Determine if any environmental standards have been breached: - Groundwater monitoring, investigation or modelling demonstrates significant and sustained adverse trend. | Demonstration that the cause or likely cause of the harm has ceased or been resolved. This can include monitoring and modelling. |
| Descriptive conditions. No significant: - iridescence / sheen; - discolouration; - deposition of solids; - increased foaming; or - microbiological growth; in receiving waters. Conditions covering the control of nuisance, including but not limited to odour, noise, dust, litter, aerosols and the presence of vermin. | Non-compliance causes or is likely to cause a category 1 or 2 environmental event. | - Environmental events guidance. - Inspection of the ‘Pollution Control’ or ‘Nuisance’ Schedule of the Authorisation - SEPA’s odour guidance. - Noise: Environmental events guidance. | - Demonstration that the cause or likely cause of the harm has ceased or been resolved, and the visual impact is now compliant. - Operator conducts monitoring / modelling to indicate compliance. |
| Single tier numeric limits set to achieve environmental standards. | More than twice the limit specified in authorisation. Exceptions: - pH: limit exceeded by more than 1 unit. - Temperature: exceeded by more than 5°C. - Any breach of a treated effluent emissions limit to water for upper-tier only conditions. - Landfill leachate head limit exceeded by more than 1m. - Landfill perimeter borehole Methane concentration between 5% and 15%. | - SEPA monitoring data. - Operator self-monitoring. - Regulatory Method (WAT-RM-40) - Guidance on the management of landfill gas | Major non-compliance remains until next compliant sample. |
| Two-tier numeric limits set to achieve environmental standards (set according to a specified statistical distribution). | - Any breach of upper-tier limit - Number of planned samples exceeding lower tier limit is greater than the maximum number of permitted exceedances specified in the look up table. | Assess against authorisation conditions and look up table. - SEPA monitoring data. - Operator self-monitoring. - Regulatory Method (WAT-RM-40) | - Upper tier: when the next planned sample result is compliant. - Lower tier: when number of planned samples exceeding lower tier limit is less than the maximum number of permitted exceedances in the previous 12 months - specified in the look up table. |
| Annual numerical limits. | Any breach of the limit specified in the authorisation. | Assess against limit in authorisation, using: - SEPA monitoring data. - Operator monitoring data. - SEPA guidance. | Major non-compliance will apply from the date of the breach until the end of the relevant calendar year. |

Table 2: Scope of site

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| All conditions relating to the authorised place, person or activities. | Authorised place, person or activity not operating as described in the Authorisation. For example: - Deposition of waste outside the authorised area. - Deposition of waste outside the authorised landfill cell containment area. - Incorrect discharge location. | - Site inspection. - Review of the on-site records. - SEPA investigative monitoring data. | - Activity undertaken in line with the authorisation or authorisation obtained to expand scope of existing activity. - Authorisation is transferred to a fit and proper person. |
| Limit on the amount of waste that can be accepted, stored or treated on a site. | Exceeding the limit that is authorised for acceptance, storage or treatment on site where the authorised person is unable to manage the material in an effective manner – is the cause or likely cause of a category 1 or 2 environmental event. | - Site inspection, including on-site records review - Data return review | Amount of waste stored on site and time that it is stored reduced to within condition limit. |
| Type of waste that can be accepted | - Accepting unauthorised waste. - Allowance made for some unauthorised waste entering site and being dealt with in accordance e.g. via quarantine procedures. Only where this does not happen, is this deemed major non-compliance. | - Site inspection, including on-site records review. - Data return review. | - Unauthorised waste has been removed from site or is dealt with according to procedure. - Where that is not possible, information submission that includes: - Corrective actions and reasons for accepting unauthorised wastes - Assessment of the likely impact - Evidence that the waste is no longer being accepted - SEPA will assess submission and ascertain if enough information is provided to resolve compliance. |

Table 3: Understanding of authorised activities, general industrial activities.

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Requirement to report incidents / events (either in operation of plant or environmental events). | Unless specified otherwise in the authorisation, failure to notify SEPA within 24 hours of an event that has caused or could cause an adverse impact upon the environment or harm to human health. Failure to submit a report within 14 days or the report does not comply with the information requirements specified in the authorisation. | - Operator notification received within timescale. - Operator incident report submitted in accordance with authorisation conditions. - Assessment of operator incident report against incident / event conditions in authorisation and environmental events guidance. | The operator has notified SEPA of the incident / event and has provided a report of adequate quality. |
| Requirement to submit data (records, reports, data). | Failure to make a data return (including nil returns) of adequate quality within 30 days of the required submission date. | - SEPA will assess the submission to confirm it has been received within the correct timescale. - If an operator is near the compliance limit, information should be included about how this will be managed going forward. | - Required data submission. - Or if no data available, MNC will be resolved on receipt of an information submission that includes: - Corrective actions/reasons for missing information. - Assessment of likely impact from missing data. - Evidence that the records are now being recorded and kept. - SEPA will assess submission and ascertain if enough information is provided to resolve compliance. |
| Requirement to keep records. | - Failure to provide upon request information required by the authorisation. - Information submitted is not true and accurate. - Records are not kept for the minimum duration. | No information provided. | - Required data submission. - Or if no data available, MNC will be resolved on receipt of an information submission that includes: - Corrective actions and reasons for missing information. - Assessment of the likely impact from missing data. - Evidence that the records are now being recorded and kept. - SEPA will assess submission and ascertain if enough information is provided to resolve compliance. |

Table 4: Overall management of authorised activities, general industrial activities.

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Compliance with authorisation conditions. | Multiple non-compliance (same or different conditions) that cumulatively equate to major non-compliance. | Having regard to condition-specific guidance where this exists and / or environmental events guidance and events records. - Greater than or equal to four category 3 events within a 365-day period that have been caused by a breach of the same condition. - Unless otherwise stated within the site authorisation, multiple non-compliance breaches may be considered collectively as major non-compliance depending on the particular circumstances. SEPA's assessment will be subject to additional governance checks to ensure national consistency in decision-making. | - Demonstration that the cause or likely cause of the harm has ceased / been resolved. This can include monitoring / modelling. - Remedial actions will be specific for each non-compliance. However, evidence must be provided that all of the conditions identified as cumulatively equating to major non-compliance are now compliant. |
| Management of site, technical competence infrastructure, equipment or training. | Non-compliance causes or is likely to cause a category 1 or 2 environmental event: - Includes commissioning requirements, training, staff resource, maintenance and breakdowns, management of change, procedures. - Authorised activities not managed or supervised by a designated technically competent person. - Implementation of an Environmental Management System that does not consider BAT at all stage. - Subject to any other authorisation condition, infrastructure or equipment required by the authorisation not installed or not working for extended periods of time (e.g. more than 3months). - Monitoring equipment not calibrated in accordance with authorisation requirements. | - Based on produced guidance or as decided by SEPA. - BAT reference documents - BREFs + BATCs - UK BAT Implementation Guidance. - Best practice guidance where available e.g. Technical Guidance notes, EA guidance etc. - WST-G-002: Guidance regarding provision and assessment of technically competent management at waste management facilities | - Management system in place to control the site activities in accordance with the authorisation. - Activities have ceased or reduced to levels that no longer cause or likely to cause a category 1 or 2 environmental event. - Evidence provided that site equipment or infrastructure is installed and operating in accordance with the authorisation. |
| Management plans. | Management plan not submitted, or when they are submitted, they are of very poor quality or content | Using available guidance i.e. latest versions of SEPA Odour Guidance for Odour, BS4142 and H3 for Noise, BREF documents, Landfill management plan guidance - Management plan does not cover all potential scenarios that have potential to occur, and the site officer deems need addressed i.e. a vermin management plan at a waste site does not include flies, or if it does, the measures proposed are not. | When an acceptable management plan is submitted. |
| Financial provisions. | Failure to maintain adequate financial provisions until the authorisation is surrendered. | Assessment to ensure adequate financial provisions are being made to cover all authorisation obligations. - WST-G-031: Financial provision for non-landfill waste management | Demonstration that adequate financial provision is being maintained by the authorised person. |
| Acceptance of waste. | Failure to implement appropriate pre-acceptance audit procedures of waste on receipt and / or at the point of disposal. Failure to provide and maintain a calibrated weighbridge at the authorised place. Failure to record and weigh all waste accepted at the authorised place using the weighbridge. Failure to comply with: - The criteria and procedures for the acceptance of waste at Landfills (Scotland) Direction 2005. | - Assessment by document review with reference to AMSG: Guidance for the storage and treatment of healthcare waste. Pre-Acceptance Audit must be satisfactory. - Inspection of weighbridge records. - Refer to Authorised Wastes & Quantities Schedule of authorisation and Landfill site management plan guidance. The Landfill Regulation Team will use waste acceptance criteria themed inspection checklists to assess compliance. | - Demonstration that pre-acceptance procedures are being implemented. - Evidence of a calibrated weighbridge certificate provided. |
| Any BAT requirement (as defined by SEPA). | Failure to comply with an authorisation condition requiring the implementation of BAT (as defined by SEPA). | - BAT reference documents - BREFs + BATCs - UK BAT Implementation Guidance. - SEPA decision based on Sector Comparison and SEPA Guidance here; Industrial activities ¦ Beta ¦ SEPA ¦ Scottish Environment Protection Agency - Best practice guidance where available e.g. Sector and Technical Guidance notes, EA Guidance etc. | Implement BAT in accordance with the authorisation. |
| Monitoring. | - Monitoring not undertaken for a period of more than 30 days from the required date. - Deviation from monitoring requirements of their authorisation. The extent of the deviation will determine if the breach is considered major non-compliance. | - SEPA will review the operators monitoring to assess extent of deviation. - Available Standards, Guidance and Procedures e.g. UKAS, MCERTS | Information submission that includes: - Corrective actions/reasons for missing information - Assessment of the likely impact from missing data - Evidence that the monitoring is now being carried out SEPA will assess submission and ascertain if enough information is provided to resolve compliance. |

Table 5: Wider legal environmental requirements and financial non-compliance

EPAS is designed to provide a more accurate picture of legal environmental compliance.  Under EPAS we will therefore assess compliance of the following situations as it is important a performance rating reflects this. In addition to below, our intent is to develop major non-compliance criteria for duty of care and special waste regulation contraventions and start to use these at the same time.

| Contravention | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Undertaking a regulated activity without authorisation. | Undertaking a regulated activity without authorisation. For example: - Unauthorised direct discharge into groundwater. - Unauthorised discharge to surface water. - Unauthorised Directly Associated Activities. | SEPA will check if an activity is being carried out that is not covered by an appropriate authorisation. | Unauthorised activity ceases, or authorisation is varied to include the activity, or appropriate authorisation is obtained. |
| Environmental event at an authorised site that is unrelated to the authorised activity. | Causes: - category 1 or 2 environmental event. - breach of water or air quality standard. - significant and sustained adverse trend for groundwater. Examples relevant to industrial activities: - hydrocarbon pollution unrelated to authorised activity – diesel tank on site that is used to fuel vehicles ruptures. - slow contamination of borehole required by authorisation from unrelated vehicles. | - Environmental events guidance. - Determine if any environmental standards have been breached: - Local environmental standard e.g. spatial standard breach. - Groundwater monitoring / investigation shows, or modelling demonstrates, significant and sustained adverse trend. Note: Modelling can be used to predict area of impact, and/or test if it breaches spatial standards. | Demonstration that the cause or likely cause of the harm has ceased or been resolved. This can include monitoring and modelling. |
| Failure to comply with requirements of the UK POPs Regulations. | - Failure to dispose of or recover POPs-contaminated waste in such a way as to ensure that the POPs content is destroyed or irreversibly transformed. - Undertaking any disposal or recovery operations that may lead to recovery, recycling, reclamation or reuse on their own of the POPs-contaminated wastes. | Assessment against Guidance on the management of Waste Upholstered Domestic Seating (WUDS) containing Persistent Organic Pollutants (POPs) and the overarching requirements of the UK POPs Regulations |  |
| Non-payment of annual charges. | Escalation to debt collection or if debt goes into the next financial year. Major non-compliance equals escalation to debt collection or if debt goes into next financial year. (Non-compliance is when payment is more than 90 days late). | Monthly debtors report. | Payment of annual charges. (Payment plan agreed with SEPA is equivalent to a compliance recovery plan.) |

# Annex 1: Major non-compliance specific to landfill activities (in addition to the Industrial Activities general document above)

Table 3: Understanding of authorised activities, additional landfill specific authorisation conditions.

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Financial provision. | - Submissions to demonstrate the maintenance of adequate financial provisions are not provided to SEPA within 30 days of the date specified in the authorisation. - Failure to maintain adequate financial provisions until the authorisation is surrendered. - Non-renewal of bond as per frequency specified in performance agreement. | - Assess submission of annual financial certificate and three yearly confirmation of the financial certificate. - Assessment to ensure adequate financial provisions are being made to cover all authorisation obligations including aftercare and closure. - Bond should be renewed as per frequency and at quantum specified in associated performance agreement. technical-guidance-note-estimate-of-amount-of-financial-provision-for-landfill.pdf | - Annual and three yearly submissions submitted that demonstrate compliance. - Demonstration that adequate financial provision is being maintained by the authorised person. - Demonstration that the bond has been renewed in accordance with the provisions of the performance agreement between the authorised person and SEPA. |

Table 4: Overall management of authorised activities, additional landfill specific authorisation conditions.

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Conditions covering containment, capping and restoration. | - Construction Quality Assurance (CQA) Plans or Reports are not submitted, or where they are submitted they are of very poor quality or content. - Failure to provide a CQA Report prior to the deposition of waste in any new cell. - Failure to provide a CQA Report within 30 days from the date specified in the Authorisation, for capping, leachate well and landfill gas infrastructure. - Capping not applied within three months of the date specified in the authorisation. - Restoration Management Plan (RMP) not submitted or is not representative of on-site activities or when they are submitted, they are of very poor quality or content. - Failure to restore the landfill in accordance with a SEPA agreed RMP. - Undertaking restoration without having a SEPA agreed RMP. | - SEPA will assess the submission against relevant authorisation conditions using landfill engineering guidance documents. - Site inspection - SEPA will assess the quality of RMP against WST-G-57 guidance. - Site inspection to assess potential to cause an impact upon the environment or human health. - A review of documents to confirm - Correct waste types. - Excessive waste volumes. | - Provision of compliant CQA documentation. - Compliant capping system installed at the site. - Provision of compliant RMP. - Restoration is undertaken in accordance with the agreed RMP. |
| Requirement to extend the landfill gas (LFG) management system following the capping of a cell or phase. | Failure to extend the landfill gas management system within six months of capping a cell or phase. | Site inspection and review of CQA documents. | - Confirmation that the LFG management system has been installed and is operational. - CQA documentation has been assessed as complaint (where relevant). |
| Conditions covering the management of asbestos waste. | Operations that could lead to the release of asbestos fibres. Major non-compliance but not necessarily a category 1 or 2 environmental event. | Site inspection: Technical Guidance Note: The disposal in landfills for non-hazardous waste of asbestos, wastes . | Operations have ceased or reduced to levels that no longer cause or likely to cause a category 1 or 2 environmental event. |

<Report date here (month, year)>

For information on accessing this document in an alternative format or language, please contact SEPA by email or telephone 0300 099 6699.

If you are a user of British Sign Language (BSL), the Contact Scotland BSL service gives you access to an online interpreter, enabling you to communicate with us using sign language.

Additional criteria specific to landfill can be viewed in Annex 1, table 3.

Adequate quality means containing the required information to enable the assessment of compliance with authorisation conditions.

Additional MNC criteria that are specific to landfill can be viewed in Annex 1, table 4.