October 2026

2025 WASTE DATA QUALITY REPORT

WASTE LANDFILLED IN SCOTLAND

Table of Contents

Quality Report for Waste Landfilled in Scotland 2025	2

Introduction	2

Revisions Policy	3

Reporting	3

EWC Codes	5

Biodegradable municipal waste	5

EWC Coded 19 12 12 waste	6

Feedback	9

Appendix 1	10

Datasets used in the 2025 methodology	10

Scottish licensed/permitted waste site returns	10

Appendix 2	11

European Waste Catalogue	11

European Waste Catalogue List of Waste (EWC 2000)	11

European Waste Catalogue for Statistics (EWC-Stat)	11

EWC codes which comprise Municipal Waste and biodegradability factors	13

Appendix 3	15

Glossary	15

Appendix 4	16

Acronyms	16

#### An Official Statistics Publication for Scotland

#### Published on 27th October 2026 9:30 AM

## Quality Report for Waste Landfilled in Scotland 2025

## Introduction

This report describes the methodologies used to produce summary data on waste landfilled in Scotland for the 2025 calendar year. The report should be used alongside the 2025 Waste landfilled in Scotland official statistics commentary document and associated data tables.

The 2025 data are presented as follows:

- The Waste landfilled in Scotland and associated data tables are presented in the Waste landfilled in Scotland commentary document (and associated data tables). This narrative describes the major trends and provides an interpretation of the data. They are located on SEPA’s web site.

- Scotland’s Environment Waste Discover Data tool presents the waste from all sources (WFAS) data in an interactive and visual format and is found on Scotland’s Environment web. This tool covers the total waste managed, whether it be waste from households, waste from construction and demolition, or waste from commerce and industry. It includes the waste landfill dataset.

The WFAS Discover tool is updated once yearly, in about March, when all data for Scottish waste generated and managed, of which waste landfilled in and outwith Scotland is a subset, becomes available. The waste landfilled in Scotland statistics are released about six months before the annual update of the WFAS Discover tool. This is because the landfill data comes primarily from one discrete dataset and, in line with the Code of Practice for Statistics which provides that statistics should be released as soon as they are considered ready, can be prepared and published earlier than the WFAS.

Appendix 1 provides a fuller description of the source dataset(s) included in the Waste landfilled in Scotland statistics

Appendix 2 provides a summary of the coding of waste using the European Waste Catalogue (EWC) and European Waste Catalogue for Statistics (EWC-Stat), which are used throughout this document. It also provides a table of EWC codes that comprise municipal waste, for the purpose of reporting biodegradable municipal waste landfilled.

Appendix 3 provides a provides a glossary of terms.

Appendix 4 provides a list of acronyms.

### Revisions Policy

Revisions could occur for various reasons, including when data from third parties are unavailable or provisional at the time of publishing or if there are subsequent methodological improvements or refinements. The figures are accurate at the time of publication. However, the data may be updated if further revisions are necessary. Normally these revisions will be published concurrent with the next release. Where there have been changes in methodology for the waste data tables, the complete dataset is revised for all years to ensure that comparisons between years are valid.

## Reporting

This section describes how we report waste landfilled in Scotland. All landfill data are sourced from permitted waste quarterly data returns submitted to SEPA by landfill site operators. A list of operational Scottish landfill sites is maintained quarterly. In 2025, there were 36 operational landfill sites in Scotland (Table 1).

The number of landfill sites that are actively disposing of waste to landfill varies from year to year. The number of operational landfill sites has decreased steadily between 2005 and 2025, consistent with a decrease in waste landfilled from 7.1 million tonnes in 2005 to 1.6 million tonnes in 2025.

Table 1. The number of landfill sites in Scotland that have contributed to the landfill dataset

| Year | Number of landfills |
| --- | --- |
| 2005 | 129 |
| 2006 | 117 |
| 2007 | 110 |
| 2008 | 83 |
| 2009 | 81 |
| 2010 | 74 |
| 2011 | 66 |
| 2012 | 65 |
| 2013 | 68 |
| 2014 | 70 |
| 2015 | 62 |
| 2016 | 57 |
| 2017 | 55 |
| 2018 | 49 |
| 2019 | 45 |
| 2020 | 41 |
| 2021 | 43 |
| 2022 | 42 |
| 2023 | 42 |
| 2024 | 40 |
| 2025 | 36 |

Data on waste disposed to landfill in Scotland were taken from the waste data returns dataset which is managed by SEPA. Data submissions were collected, processed and checked by SEPA staff with the assistance of various automation and visualisation tools. SEPA carried out quality assurance of the dataset by comparing individual site data with previous quarters/years, assessing the correct use of ‘landfill’ as a management method in Tables B and C4, reviewing the consistency of EWC codes with the description of waste provided by the operator, and checking for missing data.

### EWC Codes

Waste data are supplied as EWC codes, which are aggregated into final EWC-Stat reporting categories. Waste types are separated into non-hazardous/hazardous using European Waste Catalogue (EWC) codes.

### Biodegradable municipal waste

Biodegradable Municipal Waste (BMW) is the fraction of municipal waste that will degrade within a landfill, giving rise to landfill gas emissions, primarily methane. It includes, amongst other materials, food waste, green waste, paper and cardboard.

The BMW of waste sent to landfill is calculated based on the EWC code of the waste. A percentage biodegradability (“BMW factor”) has been determined for all waste types sent to landfill, ranging from 100% for materials such as paper and food, 50% for materials such as textiles and furniture, and 0% for inert materials like tyres and metals (Appendix 2 ). These factors were originally devised via a working group between Defra and the devolved regions of the UK for the purpose of reporting BMW sent to landfill under the EU landfill directive (Council Directive 1999/31/EC). Although no longer reporting to the EU under this directive, the BMW sent from landfill is routinely published by Defra in its UK Statistics on Waste.

It should be noted that this measure differs from that for the 2025 BMW landfill ban in Scotland.
The 2025 landfill ban applies to the whole amount of waste containing BMW, not just the BMW fraction. For example, 100 tonnes of 20 03 01 coded waste, which has a biodegradability factor of 63%, will contribute 63 tonnes of BMW landfilled for the 2020 BMW landfill target. However, the 2025 landfill ban for municipal BMW will apply to the entire 100 tonnes of this waste.

For all waste landfilled, the BMW factor was applied to the tonnes of waste for each EWC code.
This gives the total biodegradable waste landfilled by waste type. This was undertaken for all waste codes that fall into the category of Municipal waste, being all waste coded under EWC Chapter 20, selected codes under Chapter 15, and Chapter 19 where the source prior to treatment is deemed to be municipal. The full list of EWC codes which comprise the definition of municipal waste is listed in Appendix 2.

### EWC Coded 19 12 12 waste

EWC coded 19 12 12 wastes - residues from the sorting of waste - may be produced from municipal waste sources; for example, the sorting of mixed residual waste to pick out recycling material, or from non-municipal waste sources; for example, the sorting of construction waste for forward recycling. A methodology is therefore required to estimate the amount of 19 12 12 waste that is produced from municipal sources.

The amount of municipal waste that contributed to the EWC coded 19 12 12 sorting residues landfilled was estimated from the proportion of net municipal waste and net non-municipal waste inputs from sites across Scotland that produced the sorting residue wastes (see Table 3 below).

To do this, the following approach was used: - The 19 12 12 “produced” at Scottish waste sites from municipal waste (MW) sources was estimated from waste data returns submitted by the population of Scottish waste sites.

- The MW inputs and non-MW inputs were summated across the population of waste sites that recorded EWC coded 19 12 12 wastes sent off site. Only inputs that were recorded as brought onto site specifically for waste treatment were included in the analysis. A ratio of MW inputs to non-MW inputs was determined from this summary.

- For each waste site, net 19 12 12 inputs (outputs minus inputs) were recorded, subtracting any outputs sent outwith Scotland, and any outputs recorded as sent to incineration (as these were not landfilled). If a negative number resulted, it was assumed zero tonnes of 19 12 12 waste was produced at the site.

- The net 19 12 12 produced was summated across all waste management sites, then multiplied by the ratio of MW inputs to provide an estimate of 19 12 12 produced from MW and 19 12 12 produced from non-MW.

- The percentage of 19 12 12 produced from MW sources over non-MW sources (see Table 3 below) was applied to the 19 12 12 waste recorded as landfilled for the year.

For 2019 and 2020, where there was an incomplete dataset following a cyber-attack on SEPA in December 2020, the ratio of MW inputs over non-MW outputs was based on the average of 2017 – 2018 and 2021. For 2019, a summary of Scottish waste landfilled by EWC code was recovered following the cyber-attack and the BMW could be calculated by applying the ratio on the EWC codes. For 2020, although the full set of Scottish data returns was not available, the subset of landfill returns was available in which to apply the ratio.

Table 2. Estimation of 19 12 12 produced from municicipal sources

| Year | 19 12 12 produced from MW sources (Tonnes) | 19 12 12 produced from non-MW sources (Tonnes) | Percentage 19 12 12 produced from MW sources |
| --- | --- | --- | --- |
| 2007 | 195,000 | 248,000 | 0.440 |
| 2008 | 300,000 | 297,000 | 0.503 |
| 2009 | 190,000 | 260,000 | 0.422 |
| 2010 | 258,000 | 179,000 | 0.590 |
| 2011 | 262,000 | 211,000 | 0.554 |
| 2012 | 404,000 | 186,000 | 0.684 |
| 2013 | 376,000 | 243,000 | 0.607 |
| 2014 | 380,000 | 257,000 | 0.597 |
| 2015 | 447,000 | 274,000 | 0.620 |
| 2016 | 499,000 | 243,000 | 0.672 |
| 2017 | 505,000 | 289,000 | 0.636 |
| 2018 | 553,000 | 330,000 | 0.627 |
| 2019 | not available | not available | 0.656 |
| 2020 | not available | not available | 0.656 |
| 2021 | 855,000 | 358,000 | 0.705 |
| 2022 | 535,000 | 360,000 | 0.598 |
| 2023 | 370,000 | 235,000 | 0.611 |
| 2024 | 268,000 | 242,000 | 0.526 |
| 2025 | 316,000 | 267,000 | 0.542 |

### Feedback

We welcome feedback on the data from all users including how and why the data are used. This helps us to understand the value of the statistics to external users. Please contact us at waste.data@sepa.org.uk.

## Appendix 1

### Datasets used in the 2025 methodology

Data on waste landfilled in Scotland were compiled from one source: licensed/permitted waste site returns (waste data returns).

### Scottish licensed/permitted waste site returns

Individual PPC permitted sites submit quarterly licensed/permitted waste site returns (waste data returns) to SEPA via email or post. The waste data return form consists of Table B (Waste inputs to site), Table C (Waste treated on site), Table C4 (Waste landfilled or incinerated after treatment on-site – landfill or incineration sites only) and Table D (Waste sent off site). A copy of the return form can be downloaded from the SEPA website.

The quantity of waste landfilled is measured as the total tonnage of waste reported in Tables B and C4 of the return form with the management method of LF (landfilled). Waste data are supplied in the return form as EWC codes which are aggregated into final EWC-Stat reporting categories. Waste type descriptions are separated into non-hazardous/hazardous using EWC codes.

## Appendix 2

### European Waste Catalogue

Throughout this document reference is made to both the European Waste Catalogue (EWC) list of wastes and European Waste Catalogue for Statistics (EWC-Stat). A brief explanation of each is given below, along with links to further information. This section also provides a table of EWC codes that comprise municipal waste for the purpose of reporting biodegradable municipal waste landfilled.

### European Waste Catalogue List of Waste (EWC 2000)

The EWC 2000 is a harmonised, non-exhaustive list of waste types established by the European Commission (2000/532/EC) as amended in 2015 (2014/955/EU). The list is used to categorise wastes based on a combination of what they are, and the process or activity that produces them.

The list is divided into 20 chapters, most of which are industry-based, although some are based on materials and processes. Each chapter is represented by a two-digit code between 01 and 20 and comprises one or more sub-chapters. Individual waste types are detailed in the sub-chapters and are assigned a six-digit code that comprises two digits for the chapter, two for the sub-chapter and two to the waste type.

Hazardous wastes are designated by entries where the EWC code is terminated by an asterisk (*).

The use of EWC codes to describe waste on waste transfer notes in Scotland has been statutory since April 2004. Most statutory waste data returns received by SEPA, including licensed/permitted site returns, exempt activity returns, and special waste consignment notes require waste to be classified according to the EWC 2000 (as amended).

### European Waste Catalogue for Statistics (EWC-Stat)

The EWC-Stat is a (mainly) substance-oriented statistical classification of waste established by the European Commission (2004/574/EC). The EWC-Stat contains thirteen categories, each represented by a two-digit code between 01 and 13. These are sub-divided into individual waste types.

A table of equivalence allows wastes coded in the EWC 2000 (as amended) to convert into the EWC-Stat. However, because of the way the coding system operates, it is not possible to do the reverse conversion.

### EWC codes which comprise Municipal Waste and biodegradability factors

Table 3. The biodegradability of the European Waste Catalogue (EWC) codes that are counted as Municipal Waste.

| EWC code | Biodegradability factor | EWC code | Biodegradability factor |
| --- | --- | --- | --- |
| 20 01 01 | 1 | 20 03 99 | 0.63 |
| 20 01 02 | 0 | 19 01 02 | 0 |
| 20 01 08 | 1 | 19 01 11* | 0 |
| 20 01 10 | 0.5 | 19 01 12 | 0 |
| 20 01 11 | 0.5 | 19 01 13* | 0 |
| 20 01 13* | 0 | 19 01 14 | 0 |
| 20 01 14* | 0 | 19 01 15* | 0 |
| 20 01 15* | 0 | 19 01 16 | 0 |
| 20 01 17* | 0 | 19 01 17* | 0 |
| 20 01 19* | 0 | 19 01 18 | 0 |
| 20 01 21* | 0 | 19 01 99 | 0 |
| 20 01 23* | 0 | 19 04 01 | 0 |
| 20 01 25 | 1 | 19 04 02* | 0 |
| 20 01 26* | 1 | 19 04 03* | 0 |
| 20 01 27* | 0 | 19 04 04 | 0 |
| 20 01 28 | 0 | 19 05 01 | 0.63 |
| 20 01 29* | 0 | 19 05 03 | 0.63 |
| 20 01 30 | 0 | 19 06 03 | 0 |
| 20 01 31* | 0 | 19 06 04 | 0.2 |
| 20 01 32 | 0 | 19 06 05 | 0 |
| 20 01 33* | 0 | 19 06 06 | 0.2 |
| 20 01 34 | 0 | 19 12 01 | 1 |
| 20 01 35* | 0 | 19 12 02 | 0 |
| 20 01 36 | 0 | 19 12 03 | 0 |
| 20 01 37* | 1 | 19 12 04 | 0 |
| 20 01 38 | 1 | 19 12 05 | 0 |
| 20 01 39 | 0 | 19 12 06* | 1 |
| 20 01 40 | 0 | 19 12 07 | 1 |
| 20 01 41 | 0 | 19 12 08 | 0.5 |
| 20 01 99 | 0.63 | 19 12 09 | 0 |
| 20 02 01 | 1 | 19 12 10 | 0.5 |
| 20 02 02 | 0 | 19 12 11* | 0.5 |
| 20 02 03 | 0 | 19 12 12 | 0.63 |
| 20 03 01 | 0.63 | 15 01 01 | 1 |
| 20 03 02 | 0.63 | 15 01 02 | 0 |
| 20 03 03 | 0.51 | 15 01 05 | 0.74 |
| 20 03 04 | 0.2 | 15 01 06 | 0.63 |
| 20 03 06 | 0.1 | 15 01 07 | 0 |
| 20 03 07 | 0.5 | 15 01 09 | 0.5 |

Notes:

The fraction of 19 12 12 coded waste considered municipal waste is calculated on a site by site basis, in order to account for municipal waste inputs that contribute to the formation of the 19 12 12 waste.

An EWC code with an asterisk is a hazardous waste.

## Appendix 3

### Glossary

EWC Code

a six-digit code listed in the European Waste Catalogue. These codes are used to identify and classify waste into different categories. They are structured as three pairs of numbers, each representing chapters, sub-chapters, and individual entries. For example, 12 01 06*.

Hazardous Waste

waste with hazardous properties which may render it harmful to human health or the environment. Hazardous waste is also called Special Waste in Scotland, as defined in the Special Waste Regulations 1996 .

PPC Permit

a permit issued by SEPA under the Pollution Prevention and Control (Scotland) Regulations 2012.

Waste from all sources (WFAS)

waste that originates from commercial and industrial sources, construction and demolition sources as well as from household sources.

## Appendix 4

### Acronyms

EWC

European Waste Catalogue

EWC-Stat

European Waste Catalogue for Statistics

SEPA

Scottish Environment Protection Agency

For information on accessing this document in an alternative format or language please contact SEPA by email at equalities@sepa.org.uk.

If you are a user of British Sign Language (BSL) the Contact Scotland BSL service gives you access to an online interpreter enabling you to communicate with us using sign language. https://contactscotland-bsl.org