Version 2.0 August 2025

SEPA Guidance:

The disposal of asbestos wastes in non-hazardous waste landfills

This guidance has been updated to meet accessibility standards and to replace certain references to legislation with references to the Environmental Authorisations (Scotland) Regulations 2018. It has not been reviewed beyond this. We are aware that sections of this guidance may need to be updated, and this work will be completed in due course.

# Contents

1.0	Background	2

2.0	Asbestos Wastes	3

2.1 	Containment	3

2.2 	Criteria for Disposal in Non-hazardous Sites	4

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk

# Background

The Landfill (Scotland) Regulations 2003, (Landfill Regulations), set out those wastes which may be accepted in the different classes of landfill. Landfills will be classified as Hazardous, Non-hazardous or Inert.

Regulation 12. (3) and (4), set out below, details the wastes that may be accepted in a landfill for non-hazardous waste:

12. (3) The operator of a landfill for non-hazardous waste shall ensure the landfill is only used for landfilling –

- municipal waste;

- non-hazardous waste of any other origin which fulfils the waste acceptance criteria in paragraphs 1 and 3(b) of Schedule 2; and

- stable, non-reactive hazardous waste (such as that which is solidified with leaching behaviour equivalent to that of non-hazardous waste referred to in sub-paragraph (b) and which fulfils the waste acceptance criteria in paragraphs 1 and 3(a) of Schedule 2.

(4) Where hazardous waste of the type described in paragraph (3)(c) is disposed of at a landfill for non-hazardous waste, the operator shall ensure it is not deposited in cells used or intended to be used for the disposal of biodegradable non-hazardous waste.

This document provides guidance on the additional measures required for asbestos wastes as SEPA considers they present specific hazards requiring special measures to be taken.

# Asbestos Wastes

SEPA accepts that asbestos waste can be determined to meet the definition and criteria of a stable non-reactive hazardous waste suitable for disposal in a non-hazardous landfill provided it is landfilled in a separate, sufficiently self-contained cell.

## 2.1 	Containment

The important requirements for separation given below are that in a non-hazardous site, asbestos waste can be landfilled in a separate cell if the cell is sufficiently self-contained. The asbestos cell can contain no other waste materials other than those used for the purposes of covering the asbestos. With respect to the management of landfill gas and leachate from the biodegradable wastes, it is important to note that, in order to prevent the uncontrolled release of asbestos fibres, no drilling should take place through the asbestos. The following principles ensure that sufficient separation is provided:

- No physical contact with any other wastes except suitable inert wastes used as cover materials.

- The purpose of any engineered barriers is to physically separate the waste types.

- The design and location of the cell should minimise the need for leachate management within the asbestos cell.

- There should be limited contact of landfill gas and leachate from the biodegradable wastes with the asbestos wastes and

- The asbestos waste cell must not compromise the landfill gas and leachate management of the biodegradable waste cells.

## 2.2 	Criteria for Disposal in Non-hazardous Sites

Asbestos wastes can be disposed of untreated in separate cells of non-hazardous sites provided the following procedures are adhered to:

- Only waste asbestos and non-hazardous construction materials containing, or contaminated, with asbestos can be accepted in the cell.

- The asbestos must either be rigid asbestos bound by a binding agent or fibrous asbestos. All asbestos wastes shall be packaged where appropriate in accordance with the appropriate approved codes of practice and guidance documents.

- In order to avoid dispersion of fibres, the zone of deposit shall be covered daily with suitable inert material and, if the waste consists of rigid asbestos that is not packed or for any other reason asbestos is not properly contained, it shall be regularly sprinkled. The waste shall be covered immediately to a depth of at least 250mm. Final cover should be not less than two metres of suitable material and asbestos waste shall not be within 2 metres of the edge or surface of the landfill after final completion of restoration works. For all these purposes, suitable material should be used, comprising incombustible, granular material free from any objects capable of disrupting the waste or any packing.

- Asbestos waste to be segregated from non-hazardous waste by at least 2m of suitable material.

- A final top cover shall be placed on the landfill/cell in order to avoid the dispersion of fibres. and

- No works shall be carried out on the landfill/cell that could lead to a release of fibres (e.g. drilling of holes).

This guidance has been updated to meet accessibility standards and to replace certain references to legislation with references to the Environmental Authorisations (Scotland) Regulations 2018. It has not been reviewed beyond this. We are aware that sections of this guidance may need to be updated, and this work will be completed in due course.

## Disclaimer

This guidance is based on the law as it stood when the guidance was published.

Whilst every effort has been made to ensure the accuracy of this guidance, SEPA gives no warranty, covenant or undertaking (express or implied) regarding the fitness for purpose of, or any error, omission or discrepancy in this guidance. Reliance on its contents and the contents of any websites that are linked to or from this guidance is entirely at the user’s own risk. SEPA is not liable for any loss or damage that may come from using this guidance. This includes:

- any direct, indirect and consequential losses

- any loss or damage caused by civil wrongs, breach of contract or otherwise.

SEPA reserves the right to depart from this guidance and take appropriate action as it considers necessary or appropriate. Operators are responsible for ensuring that they are compliant with the law. If necessary, independent legal / specialist advice should be sought.