WAT-G-108

Version 1.0, March 2026

EASR Guidance: Domestic oil spills

Contents

0

1.	Purpose and scope	2

2.	Reasons for oil tank leaks	2

3.	Who is responsible	2

4.	What should be done in the event of a spill or leak?	2

5.	Determining clean-up requirements	4

5.1	What to do for minor spills	5

5.2	What to do for larger spills	5

6.	Ensuring it doesn’t happen again	5

7.	Useful contacts	7

Disclaimer	7

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk

# Purpose and scope

This guidance sets out advice for householders on how to deal with small scale oil spills, such as kerosene and diesel from domestic heating oil tanks.

# Reasons for oil tank leaks

Oil tanks can leak for a variety of reasons. The most common are:

- Failures of the tank body. Metal tanks can rust through, often at their bases. Plastic tanks can split due to inherent defects or age/wear-and-tear.

- Damage to equipment on the tank such as sight gauges.

- Damage to or wear-and-tear failures of fuel feed lines.

- Failure of components at the boiler end of the system, such as flexible hoses.

- Spills when oil is delivered.

# Who is responsible

Anyone who uses, stores, fills, transports or manufactures oil, has a responsibility to ensure the substance is contained and used in a way that won’t result in a release to the environment. They also have a responsibility to clean-up any spills that occur and deal with any consequences of the release.

# What should be done in the event of a spill or leak?

If you suspect an oil leak at your home, you should act quickly and contact your insurance company. Don’t put off taking action or assume the problem will go away. The quicker the leak can be dealt with, the less oil will be lost and further damage can be minimised. If your insurance doesn’t provide adequate cover, you might find it helpful to contact the International Spill Accreditation Scheme (ISAS).

If you have a spill, follow these steps:

- If there’s a strong smell of oil in your home, ventilate the area by opening windows and doors and contact the environmental health department at your local authority for advice.

- Keep children and pets away from any spills.

- Avoid getting oil on your skin and clothing.

- Wash your hands and don’t smoke, eat or drink when or after you are in contact with the oil.

- Switch off your oil supply at the tank.

- Try to find out where the leak is coming from.

- Immediately try to stop it at the source. Put a bucket under dripping oil to catch it (don’t use containers that will be used to store food for humans or animals).

- Prevent spilled oil from spreading and, in particular, prevent it from getting into drains and watercourses. Use absorbing material such as earth, sand, cat litter or commercial products.

- If there’s an ongoing leak from the tank, try to stem the flow. For example, you may be able to use sealant to temporarily repair a metal tank. For plastic tanks, try rubbing a bar of soft soap into the split.

- Try to establish how much oil has been lost. Check the level on the tank and think about how much you use and when you last had a delivery.

- Never use detergents or a hose to wash the spill away.

- Store anything with oil on it or soaked into it in containers that don’t leak until it can be correctly and legally disposed of.

- If the oil has or could enter drains or the water environment (groundwater, ponds, burns, rivers, lochs, estuaries or coastal waters), you must contact SEPA. If it’s likely to affect a public water supply, you must contact Scottish Water. If it’s likely to affect private water supplies, you must contact your local authority (environmental health).

- If your water supply or pipework may have been affected, don’t drink the water.

- Arrange for any remaining fuel in the tank to be removed by a fuel supplier. Don’t try to do this yourself and don’t store oil in a building, shed or vehicle.

- Check with your insurer whether they or you should arrange for an engineer to repair or replace your tank or pipework. Damaged plastic tanks can’t be repaired.

- Keep a record of any actions you take.

# Determining clean-up requirements

If the spill has contaminated the ground, the soil will need to be cleaned up along with any floating oil and any affected waters. Practical questions at the outset:

- Is there a strong smell of oil indoors?

- Has the oil spread away off your land, under your house or down a drain?

- Are there any burns, ponds, lochs, drains or private water supplies in the immediate area that could be contaminated by the oil spill?

- Are you at all unsure about the extent of the oil spill?

If all the answers to the above four questions are no and you can be sure that the spill is recent, small and limited to shallow soil, then this is a minor spill. Section 5.1 provides advice that you should follow unless your insurance company, SEPA or local authority instructs you otherwise.

However, if any of the answers are yes or you aren’t sure about the extent of the spill, then you may have a larger, more complicated scenario to deal with. A larger spill of oil has the potential to contaminate a lot more soil and groundwater. Section 5.2 provides advice. In this case, both soil and groundwater need to be assessed and possibly cleaned up. This is likely to involve specialist environmental contractors that may be appointed by your insurers.

## 5.1	What to do for minor spills

If it is a clearly confirmed minor spill you should check the soil, working out from the source of spill to find out where the oil now is. If limited to a few buckets worth of soil you should dig up oily and stained soils. Remove all oil. Dispose of soil correctly. You should record the actions taken and take photos and keep records of any soil sample results.

## 5.2	What to do for larger spills

Clean-up works will generally only be carried out by specialist environmental contractors. If your building and contents policy permits, your insurance company will appoint a specialist contractor on your behalf. However, if you don’t have the necessary insurance cover, you will need to appoint a specialist contactor yourself. Unless your spill is a minor spill, we don’t advise you to attempt to clean-up the oil yourself, given the potential health risks and the specialist nature of remediation and waste disposal.

You or your contractor will need to contact us for advice on the safe disposal and/or treatment of contaminated soils or waters.

It’s important that appropriate steps are taken to check that the ground and/or water has been adequately cleaned up. The extent and type of checks will depend on the extent of the spill and its impact, and any regulatory requirements. For example, for small spills that haven’t spread or contaminated the water environment, a documented visual and odour inspection to confirm that all contamination has been removed may be sufficient. In the case of larger spills, confirmatory investigations should be undertaken by specialist contractors and in accordance with UK best practice.

# Ensuring it doesn’t happen again

By following some basic rules, you can minimise the risk of another spill. For example:

- Ensure you comply with The Environmental Authorisations (Scotland) Regulations water GBR 27, the storage of oil on premises used as a private dwelling (except where the premises is a vehicle or vessel).

- Keep your tank and fittings well maintained.

- Always be present when your tank is filled.

- If you replace your tank, ensure it is placed away from high risk locations, above ground (if possible) and installed by an OFTEC registered or other reputable installation contractor.

- Ensure your tank is compliant with local building control regulations.

- Carry out monthly visual inspections of your tank. See the Oil Care website for what to look for.

- Investigate if the level of oil in your tank drops suddenly, if your boiler starts cutting out or if you detect any unexplained odours or ground movement.

- Obtain a copy of your insurer’s guide to dealing with oil spill emergencies and keep it close to your boiler or tank.

- Obtain a domestic oil spill kit for use in emergencies.

- Ensure above ground tanks and pipework are inspected by a suitably qualified professional at least once a year, usually at the same time your boiler is checked and serviced.

- Ensure underground tanks are leak tested at the recommended intervals (three to five yearly for underground tanks).

- If your underground tank is more than 20 years old, ensure it is leak tested every year and consider replacing it with an above ground tank.

- Make sure you insurance covers oil spills.

If you notice a drop in oil level, don’t assume that oil has been stolen etc, as leaks of underground fuel lines can often be ongoing for some time, prior to oil contamination becoming visible.

Further guidance is available in Get to know your oil tank and Dealing with spills: PPG 22.

# Useful contacts

- SEPA Pollution Hotline number - 0800 80 70 60

- Scottish Water customer helpline - 0800 0778778.

- Local authorities – Contact details for each of the Scottish local authorities are available on the Scottish Government website.

- International Spill Accreditation Scheme (ISAS).

# Disclaimer

Whilst every effort has been made to ensure the accuracy of this guidance, SEPA gives no warranty, covenant or undertaking (express or implied) regarding the fitness for purpose of, or any error, omission or discrepancy in this guidance. Reliance on its contents and the contents of any websites that are linked to or from this guidance is entirely at the user’s own risk. SEPA is not liable for any loss or damage that may come from using this guidance. This includes:

- any direct, indirect and consequential losses

- any loss or damage caused by civil wrongs, breach of contract or otherwise

SEPA reserves the right to depart from this guidance and take appropriate action as it considers necessary or appropriate. Operators are responsible for ensuring that they are compliant with the law. If necessary, independent legal / specialist advice should be sought.