# Major non-compliance: water resources

Table 1: Major impact of the authorised activity

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Any authorisation condition. | Non-compliance causes or is likely to cause: - Category 1 or 2 environmental event. - Breach of water standard. - Significant and sustained adverse trend for groundwater. | - Environmental events guidance. - Groundwater monitoring, investigation or modelling demonstrates significant and sustained adverse trend. | Demonstration that the cause or likely cause of the harm has ceased or been resolved. This can include monitoring and modelling. |
| Descriptive conditions. No significant: - Iridescence / sheen; - Discolouration; - Deposition of solids; - Increased foaming; in receiving waters. | Non-compliance causes or has potential to cause Category 1 or 2 environmental events. | - Site inspection. - Environmental events guidance. | Demonstration that the cause or likely cause of the harm has ceased or been resolved and the visual impact is now compliant. |
| Abstraction numeric limits. | - Daily abstraction rates: limits exceeded by more than 20%. - Combined abstraction rates (as specified in the authorisation): limits exceeded by more than 20%. - Instantaneous abstraction rate: limits exceeded by more than 20%. - 25 or more exceedances of less than 20% of daily limit in a 12-month period. Exception: seasonal licences of 180 days or less equals 14 exceedances. - 25 or more exceedances of less than 20% of combined abstraction rate (as specified in the authorisation) in a 12-month period. - Breach of yearly / seasonal abstraction limits. Note: assessment of yearly / seasonal limits will be made independently from daily limits. | - Assess against authorisation limit using: - Operator data. - SEPA monitoring data e.g. gauged flows. - Site inspections - data examined during site inspections and / or gauged flows. - Default assessment of compliance will be made on daily limits and assessment on instantaneous flows will be made if there is no daily limit authorised. | - Evidence provided demonstrates compliance e.g. evidence of new meter or recalibration of existing meter. and / or - ​SEPA monitoring and inspection demonstrate compliance. |
| Abstraction water scarcity conditions. | Breach of any temporary restriction on abstraction during a water scarcity event. | - Assess against Notice limit or water scarcity condition. - Assess against authorisation limit using: - Operator data. - SEPA monitoring data e.g. gauged flows. - Site inspections - data examined during site inspections and / or gauged flows. | - ​Evidence provided by the Operator demonstrates compliance e.g. evidence of new meter or recalibration of existing meter. and / or - ​SEPA monitoring and inspection demonstrate compliance. |
| Impoundment: mitigation flows (e.g. compensation flows, hands-off flows and freshets). | - Mitigation flows are less than 80% of the required flows in the authorisation. - 25 or more breaches of mitigation less than 20% (i.e. between 80% and less than 100% of the required flows in the authorisation). - No reduction in number of breaches for seasonal licences as mitigation flows need to be provided at all times. - Compensation structure deliberately obstructed Freshets: - Delivering less than 80% of required volume - Non-delivery of freshet, where not agreed with SEPA in advance Non-compliance will be recorded if: - Delivery period late - Flows delivered 80% to less than 100% of volume | Assess against authorisation limit using: - Operator data. - SEPA monitoring data e.g. gauged flows. - Site inspections - data examined during site inspections and / or gauged flows. | - ​Evidence provided by the operator demonstrates compliance e.g. evidence of new meter or recalibration of existing meter. and / or - ​SEPA monitoring and inspection demonstrate compliance. |
| Return abstracted water – temperature. | Any breach greater than 5°C. | On site measurement or assessment of cooling water data required by licence. | ​Operator data demonstrates compliance via cooling water data provided to SEPA and / or SEPA monitoring. |

Table 2:  Scope of site

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Provision of fish pass and screens. | - Fish pass or screen not provided. - Fish pass or screen not maintained or not operated to provide passage of migratory fish. - Fish pass or screens must not cause harm to fish. - Fish pass must not prevent passage of migratory fish. | - Presence or absence of fish pass and / or screen. - Ecological surveys. | Provision of fish pass and / or fish screens. |
| Impoundment: height and capacity of impoundment. | The height of impoundment and / or the capacity of impounded volume exceeded. | Against limit in authorisation via data returns or from data examined during site inspections. | ​Impoundment is amended to licenced limits or variation application approved containing any required mitigation measures. |
| Groundwater. | Depth of borehole exceeded or located in the wrong place, causing an adverse impact on the water environment. | On site measurement for depth and location and / or evidence in drilling logs. | ​Borehole is amended to licenced limits or variation application approved containing any required mitigation measures. |
| Return abstracted water – scour. | Return of abstracted water shall not cause significant scour that leads to a failure of environmental standards. | - On site assessment including visual assessment, site measurements. - Desk top review of historic records and satellite photography and information contained in SEPA records such as application documents. | ​Review of operator evidence and / or site inspection. This evidence may include photographs, hydromorphological reports and site inspection. |

Table 3: Understanding of authorised activities

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Requirement to report incidents (either in operation of plant or environmental events). | Unless specified otherwise in the authorisation, failure to notify SEPA within 24 hours of an event that has caused or could cause an adverse impact upon the environment or human health. Failure to submit an incident report within 14 days. | - Operator notification received within timescale – by phone call or email. - Operator incident report submitted in accordance with authorisation conditions. | ​The operator has notified SEPA of the incident / event and has provided a report of adequate quality. |
| Requirement to submit data (records, report, data). | Failure to make a data return (including nil returns) of adequate quality within 30 days of the required submission date. | - Less than 30 days late equals ‘non-compliant’. - More than 30 days late equals ‘Major non-compliant’. - Where data is missing: - Less than 30 days of missing data equals ‘Compliant’. - More than 30 days of missing data equals ‘Major non-compliant’. | - ​Required data submission. - ​Or if no data available, MNC will be resolved on receipt of an information submission that includes: - ​Corrective actions and reasons for missing information. - ​Assessment of potential impact from missing data. - ​Evidence that the records are now being recorded and kept. - ​SEPA will assess submission and ascertain if enough information is provided to recover compliance. |
| Requirement to keep records. | - Failure to provide upon request information required by the authorisation. - Information submitted is not true and accurate. - Records are not kept for the minimum duration. | - Review of records, data returns and submission. - Site inspection. - Incident investigation. | - ​Required data submission. - ​Or if no data available, MNC will be resolved on receipt of an information submission that includes: - ​Corrective actions/reasons for missing information. - ​Assessment of potential impact from missing data. - ​Evidence that the records are now being recorded and kept. - ​SEPA will assess submission and ascertain if enough information is provided to recover compliance. |

Table 4: Overall management of authorised activities

| Authorisation condition | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Compliance with authorisation conditions. | - Multiple non-compliance (same or different conditions) that cumulatively equate to major non-compliance. - The major non-compliance will be recorded against the condition that places the responsibility on the Authorised Person to ensure compliance with the conditions of the authorisation. | Having regard to condition-specific guidance where this exists and/or environmental events guidance and events records. - Greater than or equal to four Category 3 events within a 365-day period due to the same cause at the same asset or location. Unless otherwise stated within the site authorisation, multiple non-compliance breaches may be considered collectively as major non-compliance depending on the particular circumstances. SEPA's assessment will be subject to additional governance checks to ensure national consistency in decision-making. | ​Evidence provided that the issues have been resolved and action taken to prevent reoccurrence. |
| Operation and calibration of measuring and monitoring equipment. | Repeated failure to carry out required maintenance and calibration of monitoring equipment and data recording equipment to demonstrate true and accurate data. | - Inspection of calibration and maintenance records. - Review method of deriving returns data is fit for purposes i.e. pump rates, back calculated from power generation etc. - Assess comments associated with data quality issues. | ​Evidence provided that equipment issues resolved and operating as it should, including evidence of calibration checks where appropriate. |

Table 5: Wider legal environmental requirements and financial non-compliance

EPAS is designed to provide a more accurate picture of legal environmental compliance. Under EPAS we will therefore assess compliance of the following situations as it is important a performance rating reflects this.

| Contravention | Major non-compliance | How will SEPA assess / supporting guidance | Return to compliance examples |
| --- | --- | --- | --- |
| Undertaking a regulated activity without authorisation. | Undertaking a regulated activity without authorisation. E.g. drilling and operation of borehole on any authorised site or installation of impoundment without authorisation (temporary or otherwise). | During inspections of authorised and unauthorised sites. | ​Unauthorised activity ceases and where appropriate authorisation is varied to include the activity or appropriate authorisation obtained. |
| Environmental event at an authorised site that is unrelated to the authorised activity. | Causes a: - Category 1 or 2 event; or - Downgrading of annual water or air classification; or - Significant and sustained adverse trend for groundwater. Note: do not need evidence a classification site is downgraded, could rely on modelling to predict area of impact, and / or test if it breaches spatial standards. Previous guidance to assess major for descriptive conditions equals equivalent to a Category 1 or 2 environmental event. | - Environmental events guidance. - Groundwater monitoring, investigation, or modelling demonstrates significant and sustained adverse trend. Examples relevant to water resources: - Sediment management in abstraction infrastructure (not covered in the authorisation activities) results in the release of solids to the water environment. | Demonstration that the cause or likely cause of the harm has ceased or been resolved. This can include monitoring and modelling. |
| Non-payment of annual charges | Escalation to debt collection or if debt goes into the next financial year. (Non-compliance is when payment is more than 90 days late.) | - Monthly debtors report. | Payment of annual charges. (Payment plan agreed with SEPA is equivalent to a compliance recovery plan.) |

<Report date here (month, year)>

For information on accessing this document in an alternative format or language, please contact SEPA by email or telephone 0300 099 6699.

If you are a user of British Sign Language (BSL), the Contact Scotland BSL service gives you access to an online interpreter, enabling you to communicate with us using sign language.

Note: not maintained or operated excludes operational breakdown notified to SEPA and not exceeding 30 days in any 12-month period. This would be non-compliance, not major non-compliance.

Adequate quality means containing the required information to enable the assessment of compliance with authorisation conditions.