Version 3.0 July 2026

SPRI-G-002

Scottish Pollutant Release Inventory (SPRI) Reporting guidance: Larger scale activities

# Contents

1.	Introduction	3

1.1 Scope of the guidance note	3

1.2	Access to further information	4

1.3 Disclosing information you provide	4

1.4 Feedback	4

1.5 Useful references	4

2. Petroleum sector guidance	6

2.1  Air emissions	6

2.2 Emissions to water and wastewater	25

2.3 Off-site waste transfers	27

2.4 Useful references	28

Reference Tables – UKPIA Pollution Inventory guidance for refineries	29

3 Incineration sector guidance	52

3.1 Air emissions	52

3.2 Emissions to water and wastewater	57

3.3 Off-site waste transfers	59

Reference Tables – Incineration source factors	59

Emissions to air	60

4 Paper and wood production and processing sector guidance	72

4.1  Air emissions	73

4.2 Emissions to water and wastewater	75

4.3 Off-site waste transfers	78

5 Cement and lime sector guidance	79

5.1 Air emissions	80

5.2 Emissions to water and wastewater	83

5.3 Off-site waste transfers	83

Reference Tables – Cement source factors	84

Reference Tables – Lime source factors	89

6 Production and processing of metals sector guidance	92

6.1 Air Emissions	93

6.2 Emissions to water and wastewater	99

6.3 Off-site waste transfers	101

Reference Tables – Ferrous metals source factors	103

Reference Tables – Non-ferrous metals source factors	114

7 Combustion sector guidance	129

7.1 Air pollutants	130

7.2 Emissions to water and wastewater	159

7.3 Off-site waste transfers	160

7.4 Useful references	161

8 Chemical sector guidance	162

8.1 Emissions to air	163

8.2 Emissions to water and wastewater	169

8.3 Off-site waste transfers	172

9 Glossary	173

Disclaimer	182

This guidance has been updated to meet accessibility standards and to replace certain references to legislation with references to the Environmental Authorisations (Scotland) Regulations 2018 and changes due to Brexit. It has not been reviewed beyond this.

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk

# Introduction

This Guidance Note provides information to assist you in preparing submissions to the Scottish Pollutant Release Inventory (SPRI). It has been developed through a process of consultation between industry stakeholders and ourselves. It is part of a range of non-statutory guidance and tools produced to assist in the completion of the annual SPRI return. Information on the range of guidance notes is available on the SPRI website.

By providing a series of guidance notes, we seek to ease the burden on industry and to raise the quality of SPRI data. In addition, the increasing use of SPRI data in policy-making and for comparisons on a sector and national basis has led to a need for increased consistency in SPRI data and an improved understanding of uncertainty.

## 1.1 Scope of the guidance document

This document contains guidance on specific industry sectors and should be used in conjunction with the SPRI operator guidance: Release Estimation Techniques (RET) (SPRI-G-003) to ensure that you meet the reporting requirements of the UK-PRTR. The SPRI Reporting General Operator Guidance (SPRI-G-004) includes a more prescriptive indication of methods of measurement and calculation and a requirement to report certain releases to land while the RET Guidance (SPRI-G-003) contains information on determining the most appropriate RETs to calculate the annual SPRI emission values. Data on transboundary shipments of hazardous and non-hazardous waste taking place during the reporting year must also be reported to meet the UK PRTR requirements

Note that you are not required to include any emissions due to historic activities (for example, from contaminated land) in your SPRI return unless the original contamination is related to an ongoing activity.

Emissions reported need only relate to operations of the permitted activity (direct emissions from point and fugitive sources) or from the UK PRTR activity (where it differs from the permit or is not a permitted activity) and from unauthorised or unplanned events termed “accidental” emissions (e.g. resulting from equipment failure).

For accidental emissions you should not include any emissions resulting from routine maintenance, such as the release associated with recharging cooling fluids.

## 1.2 Access to further information

This document does not provide detailed information on suggested measurement and monitoring techniques as this is dealt with extensively in other guidance documents that we publish. References to monitoring guidance and other sources of reference information are included in the relevant sections of this document.

## 1.3 	Feedback

In order to improve the quality of our guidance, we would like to receive feedback from you on your experiences in using this document. Such feedback will be taken into account in the preparation of other guidance documents in this series, and in the updating of this document.

Feedback should be submitted to the SPRI team by email: SPRIAdminstration@sepa.org.uk

## 1.4 	Useful references

The following are general references and further information you may find useful. Any references specific to a particular sector will be contained in the relevant section of this guidance note.

- Scottish Pollutant Release Inventory webpage on the SEPA website

- Environmental Authorisations (Scotland) Regulations 2018

- SEPA SPRI General Operator Guidance (SPRI-G-004) on the SEPA website

- SEPA SPRI Guidance on Release Estimation Techniques (RET) (SPRI-G-003) on the SEPA website

- SPRI Schedule to notice on the SEPA website

- UK-PRTR webpages on the UK government website

# 2. 	Petroleum sector guidance

This Section covers activities that are regulated as “Gasification, liquefaction and refining activities” under the Environmental Authorisation (Scotland) Regulations 2018 (EASR). The Environmental Authorisation (Scotland) Regulations 2018 (EASR) revoke the Pollution Prevention and Control (Scotland) Regulations 2012 (PPC) and provides an integrated environmental approach to the regulation of certain industrial activities, which is now applied to schedules 20 and 26 emissions activities under EASR. This means emissions to air, water (including discharges to sewer) and land, plus a range of other environmental effects, must be considered together.

“Gasification, liquefaction and refining activities” were formerly included within the scope as Part A prescribed/listed activities regulated under chapter 1, section 1.2 of the PPC Regulations. They are now found in EASR schedule 20 part 4 chapter 1 paragraphs 1 – 4 and schedule 26 part 3 chapter paragraphs 2 – 4.

## 2.1 Air emissions

The main air emissions from a refinery are CO2, CO, SOx, NOx, VOC and particulate matter (dust and associated heavy metals). They arise typically from sources such as stacks of process furnaces and boilers, regenerators (fluidised catalytic cracking units), individual items such as valves and pump seals and to a lesser extent from flares and incinerator stacks. Reference Tables - UKPIA Pollution Inventory guidance for refineries contains references to UKPIA’s “Pollution Inventory – Guidance for Refineries” document, which covers the major categories of refinery emission sources. The document is split into three categories of pollutant; those that will almost certainly be emitted in quantities above the reporting threshold, those emitted but generally in quantities below the reporting threshold and those that are unlikely to be emitted by refineries.

The list overleaf provides a brief summary of the main pollutants released by a refinery and should be taken as a guide only, and you should verify that there are no other pollutants emitted from your installation.

#### List 1 – Main air pollutants emitted by refineries and their main sources

Carbon dioxide (CO2)

- Process furnaces.

- Boilers.

- Gas turbines.

- Fluidised catalytic cracking regenerators.

- CO boilers.

- Flare systems.

- Incinerators.

Carbon monoxide (CO)

- Process furnaces.

- Boilers.

- Gas turbines.

- Fluidised catalytic cracking regenerators.

- CO boilers.

- Flare systems.

- Incinerators.

- Sulphur recovery units.

Nitrogen oxides (Nox)

- Process furnaces.

- Boilers.

- Gas turbines.

- Fluidised catalytic cracking regenerators.

- CO boilers.

- Flare systems.

- Incinerators.

- Coke calciners.

Particulate matter (including PM10)

- Process furnaces.

- Boilers.

- Gas turbines.

- Fluidised catalytic cracking regenerators.

- CO boilers.

- Incinerators.

- Coke calciners.

Sulphur oxides (SOx)

- Process furnaces.

- Boilers.

- Gas turbines.

- Fluidised catalytic cracking regenerators.

- CO boilers.

- Flare systems.

- Incinerators.

- Sulphur recovery units.

- Coke calciners.

Volatile Organic Compounds (VOCs)

- Storage and handling facilities.

- Gas separation units.

- Oil/water separation systems.

- Fugitive emissions (valves, flanges, etc.), vents, flare systems.

### 2.1.1 Emission sources

These emissions are exhausted via a stack or vent; i.e. a single point source into the atmosphere. Abatement equipment; e.g. scrubbing units, fabric filters (bag house) can be incorporated into the exhaust system prior to discharge to atmosphere. Point source emissions will be the most significant emission source for combustion activities taking place within refineries as well as occurring from refinery process sources (e.g. vacuum distillation, catalytic cracking, sulphur recovery, etc.).

### 2.1.2 Fugitive emissions

Fugitive emissions are those that are not released from a point source such as a stack. Examples of fugitive emissions from refineries include VOC emissions from piping systems, wastewater systems, storage tanks, loading and unloading systems and storage and handling. Although the release from each individual source may be small, diffuse VOC emission sources such as pumps, compressors, valves and flanges may contribute 20-50% of the total VOC emissions due to the large number of such sources in a refinery. Only fugitive emissions that leave the site need to be reported to the SPRI. Whilst contained spills would therefore not need to be reported, you should report vapour emissions that may have dispersed.

Estimation of fugitive VOC emissions is dealt with below in Section 2.3.4.

(a) Combustion source factors

See list 1 in section 2.1 above for refinery combustion source factors.

Emission factors are provided for the estimation of combustion products with the exception of SO2, for which the usually preferred RET is mass balance – based on the sulphur content of the fuel (see Example 1).

Emissions of certain trace elements can be estimated using mass balance, when fuel composition data is available. If such data is unavailable, default emission factors can be used.

Emissions of NOx from oil and gas-fired boilers and furnaces that do not use CEMS for annual mass emissions can be calculated from the fuel burn and NOx factors which have been agreed with us on a site-specific basis.

(b) Refinery process source factors

See list 1 in section 2.1 above for refinery process source factors.

Fuel analysis and process stream data

The use of fuel analysis and process stream data to determine emissions is similar to the use of emission factors.

The basic equation used, for example, in fuel analysis emission calculations is:

Equation 1

E 	= 	Qf x [Op Hours] x (PCf/100] x (MWp/EWf)

Where:

E is the		Emission rate of pollutant (kg/yr).

Qf is the		Fuel/feedstock/raw material use (kg/hr).

PCf is the 		Pollutant concentration in the fuel/feedstock/raw material (%).

Op Hours is the 	Operating hours per year (hr/yr).

MWp is the 		Molecular weight of pollutant as emitted after combustion/processing.

EWf is the 		Elemental weight of pollutant as present in fuel/feedstock/raw material.

Where the pollutant concentration in the fuel/feedstock/raw material is consistent over the averaging period (i.e. one year), Equation 1 can be rewritten as:

###### E 	= 	M x [PCf/100] x (MWp/EWf)

Where:

E is the 	Emission rate of pollutant (kg/yr).

M is the 	Mass of fuel/feedstock/raw material used in one year (kg/hr).

PCf is the 	Pollutant concentration in the fuel/feedstock/raw material (%).

MWp is the 	Molecular weight of pollutant as emitted after combustion/processing.

EWf is the 	Elemental weight of pollutant as present in fuel/feedstock/raw material.

The following example shows how to estimate annual emissions using Equation 1.

###### Example 1

This example shows how SO2 emissions can be calculated from oil combustion, based on fuel analysis results and fuel flow information. It is assumed that the facility operates using oil for 150 hours per year and that abatement of SO2 does not occur.

Qf =		2000 kg/yr

PCf = 		1.17%

MWp = 	64

EWf = 	32

Op Hours = 	1500 hr/yr

E = Qf x PCf x (MWp/EWf) x [Op Hours]

E = [(2000) x (1.17/100) x (64/32) x 150] kg/yr

E = 7.02 x 103 kg/yr

Equation 1 can also be used for volatile elements such as fluorine and chlorine as well as trace metallic pollutants, although some of these species are retained in the plant, either in the ash or in abatement equipment (see below).

When using Equation 1, you should be aware that the amount of pollutants present in the fuel or process stream can vary significantly.

### 2.1.3 Fugitive VOC emissions

For the purposes of this note fugitive VOC emissions from refineries can be split into four categories:

- Process fugitives.

- Tank farm fugitives.

- Loading/unloading fugitives and

- Drainage and effluent system fugitives.

Guidance for estimation of the above releases for UK refineries can be found in two separate protocols produced by the Energy Institute. The first can be used to produce estimates of annual emissions of NMVOCs including fugitive VOC releases (VOC Protocol) the second provides a methodology (Speciation Protocol), in conjunction with total VOC releases, for determining the fractional speciation of hydrocarbon emissions from oil refineries.

###### (a) 	Process fugitives

Two methods for calculating process fugitives are presented in the VOC Protocol.

The first method uses the USEPA protocol for equipment leak estimates. This protocol assumes knowledge of the number of valves/flanges/seals on a refinery and takes a tiered approach to estimating emissions.

- Tier 1 applies average emission factors based upon the process service.

- Tier 2 applies average emission factors based on leak/no leak criteria. This requires the use of VOC monitoring equipment to measure threshold VOC concentrations at each fitting.

- Tier 3 applies emission correlations based on actual VOC concentration methods determined at each fitting.

###### (i) 	Tier 1 – Average emissions factors

Where no screening values are available for particular equipment types, the “average emissions factors” presented in this section should be used. This methodology involves applying the following generic equation to estimate emissions from all sources in a stream, for a particular equipment type:

Equation 2

###### E 	= 	F x WF x N

Where:

E is 	the emission rate of VOC from all sources grouped in a particular equipment type and service (kg/hr) (e.g. values in light liquid service.).

F is 		the average emission factor for the particular equipment type (from Table 1.).

WF is	 	the average weight fraction of VOC in the stream.

N is 	the number of pieces of equipment grouped in the relevant category according to equipment type, service and weight fraction of VOC.

Although the average emission factors are in terms of VOCs, the equation still requires an input regarding the weight fraction of VOC in the process stream (i.e. WF) to account for any non-organic compounds. For example, if the stream contains water vapour, you will need to account for this in your calculations. Example 2 illustrates the application of this methodology.

How to calculate your fugitive VOC emissions:

###### Step 1: Develop an inventory of the number and service type of fugitive sources

The number and service type (gas/vapour, light liquid and heavy liquid) of each equipment type in the refinery must be determined. Service type definitions are given below:

- Gas/vapour – material in a gaseous state under operating conditions.

- Light liquid – material in a liquid state in which the sum of the concentrations of individual constituents with a vapour pressure over 0.3 kilopascals (kPa) at 20oC ³ 20% by weight (wt %) and

- Heavy liquid – material that does not fall under either of the above two definitions.

Step 2: Group the inventory into ‘streams’

In order to simplify calculations, it is recommended that the equipment/service mode combinations identified in Step 1 (e.g. valves in gas service) be grouped into ‘streams’ according to the approximate weight fraction of VOCs (i.e. WF) in each stream.

A further simplification may be to group areas of the refinery according to the ‘average’ weight fraction of VOCs in the process streams. It will be necessary to take account of the various service modes for each equipment type (e.g. gas, light liquid etc) contained within that area.

Another approach may be to take the conservative assumption that all streams are approximately 100% VOCs, thereby making WF = 1.

Step 3: Note operational hours

For the specific equipment category defined by the above two steps, the number of operational hours needs to be estimated.

Step 4: Use emission factors to estimate emission rates

Use the relevant emission factors and the equation given above to calculate the emissions from each equipment type. These emissions should then be added to derive a total emission rate for all equipment pieces quantified using this methodology.

Table 1 below gives the emission factors required to estimate emissions using the steps discussed above.

###### Table 1: Average emission factors for process fugitives

| Equipment type | Service | Emission factor (kg/hr/source) |
| --- | --- | --- |
| Connectors | Gas | 2.50 x 10-4 |
| Connectors | Light liquid | 2.50 x 10-4 |
| Connectors | Heavy liquid | 4.34 x 10-5 |
| Flanges | Gas | 2.50 x 10-4 |
| Flanges | Light liquid | 2.50 x 10-4 |
| Flanges | Heavy liquid | 4.68 x 10-5 |
| Compressor seals | Gas | 0.636 |
| Pump seals | Light liquid | 0.114 |
| Pump seals | Heavy liquid | 3.49 x 10-3 |
| Valves | Gas | 0.0268 |
| Valves | Light liquid | 0.0109 |
| Valves | Heavy liquid | 9.87 x 10-5 |
| Open-ended lines | All | 2.30 x 10-3 |
| Pressure relief valves | Gas | 0.16 |
| Sampling connections | All | 0.015 |
| Other | Heavy liquid | 5.18 x 10-5 |

###### Note: The emission factors given above are for non-methane VOCs only.

The following example shows the application of the average emission factor approach.

Example 2

A particular section of the refinery has 300 valves, 200 of these are in gas service (Step 1).

Within this smaller group of valves in gas service, it is ascertained that 100 valves are, on average, 80 weight percent NMVOCs, 10% methane, and 10% water vapour (Step 2).

It is estimated that this group of valves operates for 5500 hours per year (Step 3).

The appropriate emission factor for valves in gas service is 0.0268 kg/hr/source (from Table 1) (Step 4).

Emissions from this group of valves is thus estimated by using the following parameters:

F = 	0.0268

WF = 	0.8

N = 	100

The final emission estimate for the group of 100 valves specified above is approximately 11,900 kg of NMVOCs/year.

The previous steps would be repeated again for the remaining 200 valves that were not included in the above estimate for that section of the refinery. Similarly, emissions need to be calculated from other potential fugitive emission sources in that section, followed by the next refinery “section” etc. until fugitive emissions from the entire refinery have been quantified.

###### (ii) 	Tier 2 – Leak/no-leak method

Screening using a portable monitoring device is required for this methodology. This method relies on a “leak”/”no-leak” criteria. The following steps should be taken.

Step 1: Measure leaks from fugitive sources

A leak is typically defined and recorded if a screening value of >10,000 ppmv is returned by an appropriate calibrated monitoring instrument. Therefore, the emission factor chosen from Table 2 will depend on whether the component tested returns a pass (i.e. reading 10,000 ppmv), or fail (i.e. reading <10,000 ppmv) result.

Step 2: Estimate the VOC emission rate

Emissions are estimated for each of the equipment types listed using the following equation:

Equation 3

###### E 	= 	(FG x NG) + (FL x NL)

Where:

E is 	the VOC emission rate for the equipment type (kg/hr).

FG is	the emission factor for sources of screening values ≥10,000 ppmv (kg/hr/source).

NG is 	for the particular equipment type of concern, the number of sources with screening values ≥10,000 ppmv.

FL is 	the emission factor for sources of screening values <10,000 ppmv (kg/hr/source.).

NL is 	for the particular equipment type of concern, the number of sources with screening values ≥10,000 ppmv

###### Step 3: Note operational hours

For the specific equipment pieces screened, the annual number of operational hours needs to be estimated. This is required to derive annual emissions based on the hourly emission rates.

###### Step 4: Determine total VOC emissions

If all process fugitive sources have been screened, total VOC emissions from all sources can be determined by adding emission rates from each individual equipment component.

Table 2 on the next page presents the emission factors required to estimate emissions using the steps discussed above.

###### Table 2: Leak/no leak emission factors for process fugitives

| Equipment type | Service | Leak (≥10,000 ppmv) emission factor (kg/hr) | No leak (<10,000 ppmv) emission factor (kg/hr) |
| --- | --- | --- | --- |
| Connectors/flanges | All | 0.0375 | 0.00006 |
| Valves | Gas | 0.2626 | 0.0006 |
| Valves | Light liquid | 0.0852 | 0.0017 |
| Valves | Heavy liquid | 0.00023 | 0.00023 |
| Pump seals | Light liquid | 0.437 | 0.012 |
| Pump Seals | Heavy liquid | 0.3885 | 0.0135 |
| Compressors | Gas | 1.608 | 0.0894 |
| Pressure relief valves | Gas | 1.691 | 0.0447 |
| Open-ended lines | All | 0.01195 | 0.0015 |

###### (iii) 	Tier 3 Correlation equations

This method can only be used if screening values (ppmv) are obtained through a fugitive leak screening programme. The required screening value (SV) data is collected using an appropriate calibrated monitoring instrument.

The following points are important to note when using this methodology:

- Emission estimates are for ‘total organic carbon (TOC) and, as a consequence, a correction must be made to convert the estimates to NMVOCs (i.e. to exclude methane).

- These emission factors are on a ‘per source’ basis and

- Each individual screening value must be entered into the correlation equation to predict emissions for an equipment piece. It is important not to average screening values and then enter the average value into the correlation equation to estimate emissions.

The following steps should be followed to determine fugitive emissions using the correlation equation approach.

Step 1: Measure leaks from fugitive sources

For each piece of equipment tested, the recorded screening value will fall into one of three categories. The correct estimation methodology must then be used for each category as follows:

- For ‘zero’ readings (i.e. no emission detected), Step 2 should be consulted to estimate emissions.

- For screening values between the lower and upper detection limits of the monitoring device, Step 3 should be used and

- For values greater than the upper detection limit of the monitoring device (i.e. a ‘pegged’ emission reading), Step 4 should be used.

Step 2: Use of zero default factors

If no emissions are detected (i.e. below detection limit), then the ‘Default Zero Emission Rate’ emission factors are used (see Table 3), unless the lower detection limit of the monitoring device is >1 ppmv. In this case, half the detection limit is used.

Step 3: Use of correlation factors to determine leak

If screening values (SV) are determined through testing (i.e. measured level is between the lower and upper detection limits), then the ‘Correlation Equations’ presented in Table 3 are to be used to determine the leak from each relevant component tested.

Step 4: Use of pegged emission rate

If pegged screening values are detected, the ‘Pegged Emission Rate’ emission factors presented in Table 3 are used.

Step 5: Correct TOC readings to VOC estimate

Once emissions have been estimated from each source, the emissions must be converted from TOCs to VOCs. To do this, additional information is required on the approximate weight % of VOCs and TOCs in the process streams from which the emissions originate. This is then combined with the emission estimate for each equipment component as follows:

Equation 4

EVOC = 	ETOC x (WPVOC/WPTOC)

Where:

EVOC is	the VOC emission rate from the equipment (kg/hr)

ETOC is	the TOC emission rate from the equipment (kg/hr) calculated using the emission factors for correlations from Table 3

WPVOC is	the concentration of VOC in the equipment in weight (%)

WPTOC is	the concentration of TOC in the equipment in weight (%)

If a number of equipment pieces can be grouped because they share the same process stream and thus have similar VOC/TOC ratios, TOC emissions can be added for this equipment group prior to performing Step 5, thereby reducing the number of calculations required.

###### Step 6: Note operational hours

For the specific equipment pieces tested, you should estimate the annual number of operational hours. This information is required to derive annual emissions based on the hourly emission rates.

Step 7: Determine total VOC emissions

If all process fugitive sources have been tested, total VOC emissions from all sources can be determined by adding the emissions from each individual equipment component. Please see Table 3 below.

###### Table 3: Correlation equations for process fugitive emissions

| Equipment type | Default zero emission rate (kg/hr) | Pegged emission rate (kg/hr) 10,000 ppmv | Pegged emission rate (kg/hr) 100,000 ppmv | Correlation Equation (kg/hr) |
| --- | --- | --- | --- | --- |
| Connector | 7.5 x 10-6 | 0.028 | 0.03 | leak = 1.53 x 10-6 (SV) 0.735 |
| Flange | 3.1 x 10-7 | 0.085 | 0.084 | leak = 4.61 x 10-6 (SV) 0.703 |
| Valve | 7.8 x 10-6 | 0.064 | 0.14 | leak = 2.29 x 10-6 (SV) 0.746 |
| Open-ended line | 2.0 x 10-6 | 0.03 | 0.079 | leak = 2.20 x 10-6 (SV) 0.704 |
| Pump seal | 2.4 x 10-5 | 0.074 | 0.16 | leak = 5.03 x 10-5 (SV) 0.610 |
| Other | 4.0 x 10-6 | 0.073 | 0.11 | leak = 1.36 x 10-5 (SV) 0.589 |

The second method is a crude estimation which assumes that fugitive VOC emissions from plant and pipework represent 0.03wt% of the actual material processed in the refinery (including all thermally processed material but not product blend stocks).

Thus annual process fugitives = 0.0003 x annual refinery throughput (tonnes).

This method should be used in the absence of available data on the number of valves, flanges, pump seals, etc. for the refinery under consideration.

###### (a) 	Tank farm fugitives

For a detailed coverage of estimation of emissions from both floating and fixed roof tanks refer to Section 2 of the VOC Protocol.

###### (b) 	Loading/unloading fugitives

For a detailed coverage of estimation of emissions from both controlled and uncontrolled loading operations refer to Section 3 of the VOC Protocol.

###### (c) 	Drainage and effluent system fugitives

For a more detailed coverage of estimation of emissions from drains, separators, air flotation units and biological treatment facilities refer to Section 1.9 of the VOC Protocol.

### 2.1.4 Speciation of VOC emissions

The VOC Speciation Protocol provides a methodology for determining the fractional speciation of hydrocarbon emissions from oil refineries. The results are reported as the mass fraction of the annual hydrocarbon emissions providing individual annual mass release data for the following SPRI substances:

- Benzene.

- Toluene.

- Xylene (all isomers).

- 1,3-butadiene.

- 1,2-dichloroethane and

- 1,1,2,2-tetrachloroethane.

## 2.2 	Emissions to water and wastewater

Water discharges from refinery processes can be contaminated by a variety of substances. The main ones and their main sources are illustrated in List 2 which should be taken as a guide only, and you should verify that there are no other pollutants emitted from the process.

#### List 2 - Main water pollutants generated by refineries

Phenols

- Distillation units.

- Visbreaker.

- Catalytic cracking.

- Ballast water.

Organic chemicals (TOC)

- Distillation units.

- Hydro treatment.

- Visbreaker.

- Catalytic cracking.

- Hydro cracking.

- Spent caustic.

- Storage tanks water heel drainings.

Benzene, toluene, xylene (BTX)

- Distillation units.

- Hydro treatment.

- Visbreaker.

- Catalytic cracking.

- Storage tanks water heel drainings.

All of the following determinands below are generally measured directly from the effluent discharge unless otherwise indicated.

#### List 3 – Determinands measured in refinery effluent discharges

- Arsenic

- Benzene - API factor of 6g/l applied to process water

- Cadmium

- Chromium

- Copper

- Ethyl benzene

- Lead

- Mercury

- Nickel

- Nitrogen – Total

- Phenols

- Total organic carbon

- Toluene - API factor of 1µg/l applied to process water

- Xylene - API factor of 0.25µg/l applied to process water

- Zinc

### 2.2.1 Emission sources

More generally, emissions to water arise from the following sources:

- Process water, steam and washwater. These waters become contaminated with process fluids, dissolved gases and apart from oil, will also have taken up hydrogen sulphide, ammonia and phenols. The process water is treated in several steps before discharge to the environment.

- Cooling water, once-through or circulating systems. This stream is theoretically free of oil. However, leakage into once-through systems, even at low concentrations, can result in significant mass losses because of the large volume of water involved.

- Rainwater from process areas. This type of water has not been in contact with the process fluids, but it comes from rainfall on surfaces which are possibly oil-contaminated. This water is typically treated prior to discharge to the environment.

The resulting discharge of the above substances depends on the ‘in process’ preventative measures (good housekeeping, re-use) and the presence and technical standards of wastewater treatment facilities.

## 2.3 	Off-site waste transfers

### 2.3.1 Relevant wastes

The amount of waste generated by refineries is small if it is compared to the amount of raw materials and products that they process. Oil refinery waste normally covers three categories of materials:

- Oily and non-oily sludges. These originate from various sources such as crude and product tank bottoms, desalters, alkylation units, boiler feed water preparation, biotreaters, the cleaning of heat exchanger bundles and equipment, oil spills and soil remediation. In terms of volume oily sludges represent the largest waste category from refineries. Bio-sludge production only takes place if a refinery operates a biotreater;

- Other wastes are produced from many of the refining processes, petroleum handling operations and wastewater treatment. Both hazardous and non-hazardous wastes are generated. Spent catalysts originate from the reformers, catalytic crackers, hydro crackers, hydrodesulphurisation and hydro treating units; and

- Non-refining wastes; e.g. domestic, demolition and construction wastes.

- Any transfer of waste off-site to a third party is covered by the Duty of Care provisions of the Environmental Protection Act 1990. This includes the requirement to describe the waste and record the quantity. You should therefore use data generated in compliance with Duty of Care requirements to complete the SPRI return.

- If tankered waste is to be further processed, such as, dewatering of oily waste by a specialist waste contractor out with your site, it should be recorded within this Section.

###### Transboundary shipments of hazardous waste

For transboundary movements of hazardous waste (outwith the United Kingdom), the name and address of the recoverer or the disposer of the waste and the actual recovery or disposal site have to be reported.

## 2.4 	Useful references

These are additional useful references relevant to this sector.

- Protocol for the estimation of petroleum refinery process plant fugitive VOC emissions (The Energy Institute)

- USEPA. Protocol for Equipment Leak Emission Estimates November 1995, 453/R-95-017

# Reference Tables – UKPIA Pollution Inventory guidance for refineries

This guidance document was produced by UKPIA’s Refinery Emissions Working Group and was developed through a review of relevant industry guidance; In particular, information from the US Environment Protection Agency’s Air Chief program, the American Petroleum Institute and the Canadian Petroleum Products Institute.

This document is designed to be an aid for compiling Pollution Inventory (PI) submissions from petroleum refineries, and covers the major categories of emission sources; process emissions, combustion emissions, wastewater, etc.

This table addresses all the pollutants which we consider potentially emitted from a typical oil refinery.

### Emissions to Air

Table 4: Category 1 - These emissions are almost certainly produced by refineries in volumes above the reporting threshold (i.e. returns are expected for these substances)

| Determinand | Threshold (Kg) | Method | Emission source | Reference | Factors and guidance | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Ammonia | 1,000 | Calculation | FCCU Regen | AP42 | FCCU (uncontrolled): 0.155 kg/1000L | When CO boiler is present NH3 emissions from FCCU are considered negligible (see CPPI 9.3.4). |
| Ammonia | 1,000 | Calculation | SRU to stack | Industry experience | 1 ppm in SRU flue gas | Sour water stripper overhead gas to the sulphur recovery unit is included in this calculation. |
| Ammonia | 1,000 | Calculation | SWS to flare | Industry experience | Flared SWS gas x 0.002 |  |
| CO | 100,000 | Calculation | Combustion | AP42 | RFG: 1.34 E0 kg/1000m3 (0.082 lb/MBTU) RFO: 6.00 E-1 kg/1000L (0.033 lb/MBTU) | AP42: suggestion of formation in sulphur recovery unit but no mention of factors. |
| CO | 100,000 | Measurement | FCCU Regen | On-site analyser | Measurement |  |
| CO2 | 10,000,000 | Calculation | Site combustion and process | ETS | Guidance from ETS |  |
| Methane | 10,000 | Calculation | Combustion | AP42 | RFO (industrial boiler): 1.20 E-1 kg/1000L NG: 3.68 E-2 kg/1000m3 | Emission source is unburnt residuals in heater flue gases. |
| Methane | 10,000 | Calculation | Fugitive | Industry experience | RFG flow x Me content x 0.03% |  |
| Methane | 10,000 | Calculation | FCCU regen | Industry experience | 4.9 E-3 kg/1000m3 exhaust gas flow |  |
| Methane | 10,000 | Calculation | Incomplete flare combustion | Industry experience |  |  |
| Chlorine | 10,000 | Estimation (engineering judgement) | Reformers | Industry experience | BRT from catalyst regeneration | Losses of free Cl from cooling towers (used as biocide) considered negligible on basis that Cl will be dissolved in water drops. Chlorine used in regeneration of catalyst (reactivation of active sites). |
| Dinitrogen oxide (nitrous oxide) | 10,000 | Calculation | Combustion | EEQ Corinair | RFG: 2.16 E-2 kg/1000m3 (22 g/million grams) | EMEP/EEA air pollutant emission inventory guidebook - 2013 ¦ Publications ¦ European Environment Agency (EEA) |
| Dinitrogen oxide (nitrous oxide) | 10,000 | Calculation | Combustion | AP42 | NG (uncontrolled): 3.52 E-2 kg/1000m3 NG (low-NOx burner): 1.02 E-2 kg/1000m3 RFO: 1.32 E-2 kg/1000L |  |
| Dinitrogen oxide (nitrous oxide) | 10,000 | Estimation (engineering judgement) | Process | Industry experience | N/A (no longer applicable) for non-combustion plant |  |
| 1,2 di-chloroethane | 1,000 | Measurement | Site | IP Speciation Protocol | Based on speciation surveys |  |
| Hydrogen sulphide | 1,000 | Calculation | SRU to flare | CPPI | 0.5% of measured H2S content in gas flared (98% combusted, 1.5% reacted with CO) | CPPI: “For process units which are subject to strict fugitive emission control, for example HF, the emissions may be considered negligible and assumed to be zero.” eCFR :: 40 CFR 49.4178 -- VOC emissions control requirements for fugitive emissions. |
| NOx | 100,000 | Calculation | Combustion | EA | Site specific factors agreed for combustion plant |  |
| NOx | 100,000 | Calculation/ Measurement | FCCU regen | EA | Site specific EA agreed factors or emission analyser |  |
| NOx | 100,000 | Calculation | FCCU regen | AP42 | 2.04 E-1 kg/1000L cracker feed |  |
| NOx | 100,000 | Calculation | Flare | Industry experience | 0.068 lbs/MMBtu gas flared (HHV) |  |
| NOx | 100,000 | Calculation | SWS to flare (stoichiometric conversion of combusted ammonia to NOx) | Industry experience |  |  |
| Particulate matter | 10,000 | Calculation | Combustion | AP42 | RFO: 1.1S + 0.39 kg/1000L (where S=%sulphur in fuel) RFG/NG: 1.22 E-1 kg/1000m3 |  |
| Particulate matter | 10,000 | Measurement | FCCU regen | BS 3504: 1983 | Continuous anaylser or spot analysis | CPPI “PM emissions from catalytic cracking units stem primarily from catalyst fines entrained in the exhaust gas from the catalyst regenerator.” |
| PM10 | 1,000 | Calculation | Combustion | AP42 | RFO: 86% of total particulate RFG/NG: 100% of total particulate |  |
| PM10 | 1,000 | Calculation | FCCU | CPPI | PM10: 70% of total particulate |  |
| Sulphur oxides | 100,000 | Calculation | Combustion | Industry experience | Fuel sulphur content x flow rate |  |
| Sulphur oxides | 100,000 | Measurement | FCCU regen | EA | Continuous analyser or spot analysis |  |
| Sulphur oxides | 100,000 | Calculation | FCCU regen | AP42 | 1.41 E0 kg/1000L of FCC feed |  |
| Sulphur oxides | 100,000 | Calculation | SRU to stack | Industry experience | Based upon sulphur unit efficiency (% sulphur not removed) |  |
| Sulphur oxides | 100,000 | Calculation | Flare | Industry experience | Flared gas sulphur content x flow rate |  |
| Toluene | 10 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | In-house data based on site speciation protocol |  |
| NMVOC | 10,000 | Measurement/Calculation | Site/Fugitive | IP estimation protocol | Modified by measured data as it becomes available from LDAR programmes |  |
| PAHs (Total) | 50 | Calculation | Combustion | API/WSPA | RFO: Sum of Borneff Six. 1.39 E-6 kg/1000L NG: Sum of Borneff Six: 1.10 E-7 kg/1000m3 | https://ww2.arb.ca.gov/california-air-toxics-emission-factor Borneff Six PAHs are; benzo(a)pyrene, indeno(1,2,3-cd)pyrene, benzo(g,h,i)perylene, benzo(b)fluoranthene, benzo(k)fluoranthene and fluoranthene. From 2006 the SPRI Schedule requires reporting of the annual emissions of listed individual PAH compounds (with individual reporting thresholds) rather than a PAH total. |
| PAHs (Total) | 50 | Calculation | FCCU | CARB | 9.3 E-8 kg/1000L fresh feed (6.71 E-6 lbs/MBar) | https://ww2.arb.ca.gov/california-air-toxics-emission-factor |
| Benzo(a)pyrene | 1 | Calculation | Combustion | AP42 | RFO: 1.05 E-7 kg/1000L NG: 9.60 E-9 kg/1000m3 |  |
| Benzo(a)pyrene | 1 | Calculation | FCCU | CARB | 4.0 E-9 kg/1000L fresh feed (2.60 E-7 lbs/MBar) |  |
| Benzene | 1,000 | Measurement/Calculation | Combustion | Industry experience | N/A (no longer applicable) for combustion plant |  |
| Benzene | 1,000 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | Based upon site speciation survey |  |
| 1,3-butadiene | 100 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | Based upon site speciation survey |  |
| Butene | 1,000 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | Based upon site speciation survey |  |
| Ethylene | 1,000 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | Based upon site speciation survey |  |
| Pentene | 1,000 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | Based upon site speciation survey |  |
| Propylene | 10,000 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | Based upon site speciation survey |  |
| Tetrachloroethane | 10 | Measurement/Calculation | Site/Fugitive | IP speciation protocol | Based upon site speciation survey |  |
| Xylene | 1,000 | Measurement/Calculation | Site/Fugitive | IP speciation P Protocol | Based upon site speciation survey |  |
| Metals |  | Calculation | Combustion | AP42 | Combustion of RFO | Emission source is unburnt residuals in heater flue gases. |
| Nickel | 10 | Calculation | Combustion | AP42 | RFO: 1.01 E-2 kg/1000L |  |
| Nickel | 10 | Calculation/ Measurement | FCCU | Industry experience | Cat cracker particulates x 584 ppm |  |
| Vanadium | 10 | Calculation | Combustion | AP42 | RFO: 3.82 E-3 kg/1000L |  |
| Vanadium | 10 | Calculation/ Measurement | FCCU | Industry experience | Cat cracker particulates x 1053 ppm |  |
| Zinc | 100 | Calculation | Combustion | AP42 | RFO: 3.49 E-3 kg/1000L |  |

Table 5: Category 2 - These emissions may be produced by refineries but are generally below the reporting threshold. Further clarification may be required (i.e. further clarification may be required by reporting sites)

| Determinant | Threshold (Kg) | Method | Emission source | Reference | Factors and guidance | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Carbon Disulphide | 1,000 | Estimation (engineering judgement) | SRU to stack | Industry consensus | N/A or BRT | AP42: suggestion of formation in sulphur recovery unit but no mention of factors. https://www.epa.gov/air-emissions-factors-and-quantification/ap-42-compilation-air-emissions-factors-stationary-sources |
| Hydrogen Chloride | 1,000 | Estimation (engineering judgement) | Reformer | Industry experience | BRT from catalyst regeneration | Chlorine used in regeneration of catalyst (reactivation of active sites). |
| Hydrogen Chloride | 10,000 | Estimation (engineering judgement) | Combustion | Industry experience | BRT | CPPI: “For process units which are subject to strict fugitive emission control, for example HF, the emissions may be considered negligible and assumed to be zero.” |
| Fluorine and inorganic compounds as HF | 1,000 | Estimation (engineering judgement) | HF alkylation | HF detection equipment | BRT when undetected |  |
| HCFCs | 1,000 | Calculation | Process refrigerants | Industry experience | Estimate from fridge units’ make-up rate |  |
| Tetrachloroethylene | 1 | Estimation (engineering judgement) | Site/Fugitive | Industry experience | Industry consensus BRT or N/A |  |
| Trichloroethylene | 100 | Estimation (engineering judgement) | Site/Fugitive | Industry experience | Industry consensus BRT or N/A |  |
| Arsenic | 1,000 | Calculation | Combustion | AP42 | RFO: 1.58 E-4 kg/1000L |  |
| Mercury | 1 | Calculation | Combustion | AP42 | RFO: 1.36 E-5 kg/1000L |  |
| Selenium | 100 | Calculation | Combustion | AP42 | RFO: 8.20 E-5 kg/1000L |  |

Table 6: Category 3 - These substances are unlikely to be produced by refineries (i.e. no returns are expected for these substances)

| Determinand | Threshold (Kg) | Method | Emission source | Reference | Factors and guidance |
| --- | --- | --- | --- | --- | --- |
| Dioxins | 0.00001 | Estimation (engineering judgement) | Combustion | IP/ENTEC | Industry consensus indicates refineries not a source of dioxin emissions – assume BRT |
| Beryllium | 1 | Calculation | Combustion | AP42 | RFO: 3.34 E-6 kg/1000L |

Barnes, “UK Oil Refining and Atmospheric Emissions of Dioxins” (2004) and ENTEC, “Development of UK Cost Curves for Abatement of Dioxin Emissions to Air” (2003).

Note: Substances listed in the SPRI Schedule but not included in these tables are considered to be not applicable to refinery emissions.

### Releases to the water environment and transfers in wastewater

Wherever possible data should be net of incoming cooling water quality, however where this would result in a negative return (i.e. removal of pollutants from incoming waters) a BRT return should be made.

Table 7: Category 1 - These emissions are almost certainly produced by refineries in volumes above the reporting threshold (i.e. returns are expected for these substances)

| Determinand | Threshold | Method | Emissions source | Reference | Factors and guidance |
| --- | --- | --- | --- | --- | --- |
| Benzene | 10 | Measurement/Calculation | Effluent treatment facilities |  | Effluent quality measured data – BRT if bio-treated |
| Benzene | 10 | Calculation | Process water | API | 6 µg/litre process water |
| Toluene | 10 | Measurement/Calculation | Effluent treatment facilities |  | Effluent quality data |
| Toluene | 10 | Calculation | Process water | API | 1 µg/litre process water |
| Xylene | 10 | Measurement/Calculation | Effluent treatment facilities |  | Effluent quality data |
| Xylene | 10 | Calculation | Process water | API | 0.25 μg/litre process water |
| Nitrogen | 50,000 | Measurement |  |  | Effluent quality data (14/17 since ammoniacal) |
| Phenols | 20 | Measurement |  |  | Effluent quality data (72/94 for "as C") |
| TOC | 50,000 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Arsenic | 5 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Cadmium | 1 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Chromium | 20 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Copper | 20 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Lead | 20 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Mercury | 0.1 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Nickel | 20 | Measurement | Effluent treatment facilities |  | Effluent quality data |
| Zinc | 100 | Measurement | Effluent treatment facilities |  | Effluent quality data |

Note: Substances listed in the SPRI Schedule but not included in these tables are considered to be not applicable to refinery emissions.

# 3 	Incineration sector guidance

This Section covers activities that are regulated as “Gasification, liquefaction and refining activities” under the Environmental Authorisation (Scotland) Regulations 2018 (EASR). The Environmental Authorisation (Scotland) Regulations 2018 (EASR) revoke the Pollution Prevention and Control (Scotland) Regulations 2012 (PPC) EASR and PPC before it provides an integrated environmental approach to the regulation of certain industrial activities, which is now applied to schedules 20 and 26 emissions activities under EASR. This means emissions to air, water (including discharges to sewer) and land, plus a range of other environmental effects, must be considered together.

“Gasification, liquefaction and refining activities” were formerly included within the scope as Part A prescribed/listed activities regulated under chapter 1, section 1.2 of the PPC Regulations. They are now found in EASR schedule 20 part 4 chapter 1 paragraphs 1 – 4 and schedule 26 part 3 chapter paragraphs 2 – 4.

For the purposes of this document, activities can be broadly separated into two categories;

Waste incineration and non-waste incineration activities.

Plants excluded from waste incineration categorisation include those burning only vegetable waste from agriculture and forestry, wood waste, animal carcasses and radioactive waste amongst others. These excluded plants are not subject to waste incineration mandatory monitoring requirements if it has been demonstrated in the application for an authorisation that it was not Best Available Techniques (BAT) so to do. Essentially, the waste incinerators covered in this note are:

- Municipal Waste Incinerators (MWI).

- Clinical Waste Incinerators (CWI).

- Hazardous (Chemical) Waste Incinerators (HWI).

- Sewage Sludge Incinerators (SSI).

- Drum Incinerators (DI) and

- Animal Remains Incinerators (ARI).

## 3.1 Air emissions

Incineration installations subject to the Industrial Emissions Directive (IED) are normally required to be continuously monitored for NOx (provided that ELVs are set), total particulate matter, CO, TOC, HCl, HF and SO2. Under certain circumstances, continuous monitoring of HF, HCl and SO2 may not be required and periodic monitoring can be substituted. The specific requirements for continuous or periodic monitoring are laid down in the authorisation. The permit also specifies frequencies for periodic monitoring for heavy metals, dioxins and furans, dioxin-like PCBs and PAHs. The frequencies required by the IED are the minimum frequencies for such periodic measurements, but the permit may specify higher frequencies. Additionally, the permit may set additional site-specific monitoring requirements depending upon the local circumstances, which may include, where Selective Non-catalytic Reduction (SNCR) abatement is in operation, N2O and/or NH3 releases depending on the reductant used.

The main air emissions from waste incineration activities are shown in List 4. The List should be taken as a guide only, and you should verify that there are no other pollutants emitted from your installation. See Reference Tables – Incineration source factors for a summary of currently used RETs and guidance on a more comprehensive list of potential releases of substances to air from various sources.

#### List 4 - Main air pollutants emitted by incineration activities and their main sources

Cadmium main sources

- Flue gas in MSW incinerators from batteries, accumulators, paints and plastics.

- Hazardous wastes including effluent treatment sludges and drummed waste from metal plating works.

Carbon monoxide main sources

- Flue gas as a result of incomplete combustion of waste; e.g. if spontaneously evaporating or rapid-burning substances are present, or when combustion gas mixing with the supplied oxygen is poor.

Carbon dioxide main sources

- Flue gas as a result of complete combustion of organic material. Between 0.7 and 1.7 tonnes of CO2 is generated per tonne of MSW combusted.

Dioxins and furans

- Flue gas as a result of re-combination reaction of carbon, oxygen and chlorine.

- Also found in boiler ash, bottom ash, fly ash and sorbents.

Hydrogen chloride

- Flue gas from wastes containing chlorinated organic compounds or chlorides.

Hydrogen fluoride

- Flue gas from fluorinated plastic or fluorinated textiles in MSW and a variety of fluorinated compounds in HWI.

Mercury

- Flue gas from MSW containing batteries, thermometers, dental amalgam, fluorescent tubes or mercury switches.

- In HWI from coking plant tars, waste from chlorine alkaline electrolysis, caustic oil sludge from refineries and other chemicals containing mercury.

- Also found in bottom ash, fly ash and sorbents. In SSI, Hg from sewage (especially dental amalgam).

Metals (Heavy metals and compounds other than Hg and Cd) Sb, As, Pb, Cr, Cu, Mn, Ni, V and Zn.

- Flue gas as particulate matter usually as metal oxides and chlorides. Also found in bottom ash, fly ash and sorbent.

Ammonia

- Flue gas where SNCR is used and NH3 is used as a reducing agent. This arises through overdosing.

Nitrogen oxides

- Flue gas as both thermal and fuel NOx.

- In MSW incinerators the proportion of thermal NOx is very relevant with each type of grate having an inherent background NOx level. It is often more significant than fuel NOx.

Nitrous oxide

- Principally from SNCR. Modern MSW incinerators have low combustion originated N2O but, depending on reagent, significant emissions resulting from SNCR.

Polycyclic aromatic hydrocarbons (PAHs)

- Flue gas as products of incomplete combustion. Also found in bottom ash, fly ash and sorbents.

Dioxin-like Polychlorinated biphenyls (PCBs)

- Flue gas from most municipal waste streams and some industrial wastes. Higher concentrations in some hazardous waste streams.

- Also found in bottom ash and APC residues.

Particulate matter (including PM10)

- Flue gas as fine ash from the incineration process entrained in the gas flow.

- Also fugitive releases of dust from waste storage areas.

Sulphur dioxide

- Flue gas where sulphur is present in the waste stream. Common sources of sulphur in some waste streams are: waste paper; plaster board (calcium sulphate) and sewage sludges.

Volatile Organic Compounds (VOCs)

- Flue gas from incineration of organic waste. Also fugitive releases from waste storage areas.

Note: The term incineration includes hazardous waste, clinical waste, municipal, animal carcass and sewage sludge incineration.

### 3.1.1 Emissions sources

These emissions are exhausted via a stack or vent; i.e. a single point source into the atmosphere. Abatement equipment; e.g. scrubbing units, fabric filters (bag house) can be incorporated into the exhaust system prior to discharge to atmosphere. Point source (main stack) emissions are usually the most significant emission source (in terms of annual mass releases) for combustion activities taking place within a variety of furnace types at incineration activities.

### 3.1.2 Fugitive emissions

Fugitive emissions are those that are not released from a point source such as a stack.

Fugitive dust and VOC emissions will be minimised where combustion, flue gas treatment processes and plant in general (including storage areas) operate at negative pressure.

Some examples of the potential sources of fugitive emissions to air are:

- The loading and unloading of transport containers;

- Storage areas (e.g. bays, stockpiles, etc.).

- Transferring material between vessels (e.g. silos, volatile liquids such as SLF).

- Conveyor systems.

- Pipe work and ductwork systems (e.g. pumps, valves, flanges).

- Abatement equipment by-pass.

- Accidental loss of containment from failed plant and equipment.

- Oil and ammonia storage tanks.

- Poor building containment and extraction.

- Use of poorly-sealed waste charging systems.

Only fugitive emissions that leave the installation need to be reported to the SPRI. Whilst contained spills would therefore not need to be reported, you should report vapour emissions that may have dispersed.

Waste feedstock analysis and process stream data

The use of waste feedstock analysis and process stream data to determine emissions is similar to the use of emission factors.

For MSW incineration, analysis of fuel is not used for calculation of emissions. Either analysis is not carried out, is too variable, is too unreliable or cannot be related to emissions.

The basic equation used, for example, in waste feedstock analysis emission calculations is the same as Equation 1 and should be used in the same way (as illustrated in Example 1):

Equation 1 can also be used for volatile elements such as fluorine and chlorine as well as trace metallic pollutants, although some of these species are retained in the plant, either in the ash or in abatement equipment and appropriate retention factors applied.

When using Equation 1, you should be aware that the amount of pollutants present in the waste feedstock or process stream can vary significantly.

Fugitive emissions

Methodologies and factors used to determine fugitive emissions are referenced in Reference Tables – Incineration source factors

## Emissions to water and wastewater

Water discharges from incineration processes arise from the air abatement equipment (e.g. wet scrubbers), storm water, cooling water, boiler blow-down, incoming waste handling areas, ash and other residue handling areas, accidental emissions of raw materials, products or waste materials, firefighting and from on-site effluent treatment. List 5 illustrates the main pollutants emitted to water and their main sources. List 5 should be taken as a guide only, and you should verify that there are no other pollutants emitted from the process, including in association with suspended particulate matter. See Reference Tables – Incineration source factors for methodologies used to determine emissions to water.

#### List 5 - Main water pollutants generated by incineration activities

Dioxins and furans

- Scrubber liquor.

- Releases from ash quenching.

Ethylene dichloride

- Scrubber liquor.

- Releases from ash quenching.

Metals

- Waste storage.

- Scrubber liquor.

- Releases from ash quenching.

Polycyclic aromatic hydrocarbons (PAHs)

- Scrubber liquor.

- Releases from ash quenching.

Suspended solids

- Raw material preparation.

- Storage and handling.

- Scrubber liquor.

- Releases from ash quenching.

Total organic carbon (TOC)

- Waste storage.

- Scrubber liquor.

- Releases from ash quenching.

The resulting discharge of the previous substances depends on the ‘in process’ preventative measures (e.g. good housekeeping, re-use) and the presence and technical standards of wastewater treatment facilities.

Notwithstanding the above, you should consider all emission sources to water and characterise the flows and emission concentrations from each source.

## 3.3 	Off-site waste transfers

### 3.3.1 Relevant wastes

In general the waste streams from incineration facilities comprise:

- Bottom ash (approx. 25% by weight and 10% by volume of input for a modern MWI).

- Fly ash:

- APC residues (commonly combined with fly ash and then approx. 2.5% by weight of waste input for a modern MWI).

- Rejected feedstock wastes (chemical or physical incompatibility e.g. large objects).

- Recovered waste fractions; e.g. steel and aluminium extracted from ash.

- Road tanker waste removed from the site during the reporting year.

Transboundary shipments of hazardous waste

For transboundary movements of hazardous waste (out with the United Kingdom), the name and address of the recoverer or the disposer of the waste and the actual recovery or disposal site have to be reported.

# Reference Tables – Incineration source factors

The Reference Tables are designed to be an aid for compiling SPRI submissions from incineration processes, and covers the major categories of emission sources; combustion emissions, fugitive emissions, wastewater, etc. It also includes emission factors where applicable.

The table below addresses all the pollutants which we consider potentially emitted from typical incineration processes.

### Emissions to air

Table 8: Major categories of emission sources

| Determinand | Method | Emission source | Literature reference | Factors and guidance | Notes |
| --- | --- | --- | --- | --- | --- |
| Benzene | Calculation | Waste Combustion |  |  |  |
| Benzo(a)pyrene | Measurement | Waste Combustion | BS ISO 11338 | BS ISO 11338 for extractive monitoring. |  |
| Butadiene | Calculation | Waste Combustion |  |  |  |
| CO | Measurement | Waste Combustion | CEN/ISO/ MCERTS | Continuous and periodic monitoring carried out for all incinerators covered by IED. BS EN14181 for CEMS. Standard Reference Method BS EN15058. (ISO12039) formerly relevant. |  |
| CO | Calculation | MSW Combustion | Environment Agency (2000) | 0.197 kg/t of MSW combusted. |  |
| CO | Calculation | SSW Combustion | USEPA 1995 | 1.1 kg/t of SSW combusted (fluidised bed). |  |
| CO | Calculation |  | USEPA 1995 | 15.5 kg/t of SSW combusted (multiple hearth). |  |
| CO |  | CW Combustion | USEPA 1995 | 1.48 kg/t of CW combusted (controlled air). |  |
| CO2 | Calculation | Waste Combustion | ETS |  |  |
| CH4 | Calculation | MSW Combustion | Environment Agency (1997) | 0.0008 kg/t of MSW combusted. |  |
| CH4 | Calculation | SSW Combustion | USEPA 1995 | 0.39 kg/t of SSW combusted in multiple hearth (controlled). |  |
| Dioxins and Furans | Measurement | Waste Combustion | BS EN1948 | Periodic monitoring required for IED incinerators. |  |
| HCl | Measurement | Waste Combustion | MCERTS/BS EN | BS EN14181 for CEMs and BS EN1911 (or demonstrated equivalent) for periodic monitoring carried out for all incinerators covered by IED. |  |
| HCl | Calculation | MSW Combustion | USEPA 1996 | 3.2 Kg/t of MSW combusted. |  |
| HCl | Calculation | SSW Combustion | USEPA 1995 | 0.05 Kg/t of SSW combusted on fluidised bed (controlled). |  |
| HCl | Calculation | CW Combustion | USEPA 1995 | 16.8 Kg/t of CW combusted (uncontrolled). |  |
| Fluorine and inorganic compounds – as HF | Measurement | Waste Combustion | ISO/USEPA | BS EN14181 for CEMs and USEPA Method 26A for periodic monitoring. Not all IED incinerators are required to monitor HF continuously. |  |
| NH3 | Measurement | SNCR NOx abatement | MCERTS certified equipment | Continuous or periodic monitoring may be required depending upon BAT assessment. US EPA Method 26. VDI 2461 or ASTM D6348-03 potentially relevant to periodic monitoring. |  |
| NMVOCs | Measurement | Waste Combustion | MCERTS/BS EN 12619 | CEM and extractive monitoring required under IED. BS EN14181 for CEMS. |  |
| NMVOCs | Calculation | MSW Combustion | Environment Agency (2000) | 0.0308 kg/t of MSW combusted. |  |
| NMVOCs | Calculation | SSW Combustion | USEPA 1995 | 0.84 kg/t of SSW combusted in multiple hearth (uncontrolled). |  |
| NMVOCs | Calculation | Fugitive VOC Releases |  |  |  |
| NOx | Measurement | Waste Combustion | CEN/ISO/MCERTS | Continuous monitoring carried out for all incinerators covered by IED. BS EN14181. Standard Reference Method BS EN14792. (BS ISO 10849 for CEM & BS ISO 11564/ISO 10849 for extractive monitoring of former relevance. |  |
| NOx | Calculation | MSW Combustion | Environment Agency (2000) | 1.37 kg/t of MSW combusted. |  |
| NOx | Calculation | SSW Combustion | USEPA 1995 | 2.5 kg/t of SSW combusted in multiple hearth (uncontrolled). |  |
| NOx | Calculation | SSW Combustion | USEPA 1995 | 0.88 kg/t of SSW combusted in fluidised bed (uncontrolled). |  |
| NOx | Calculation | CW Combustion | USEPA 1995 | 1.78 kg/t of CW combusted in controlled air (uncontrolled). |  |
| N2O | Measurement | Waste Combustion |  | Extractive monitoring required for IED. (ASTM D6348-03, ISO 10849, VDI2469-1). |  |
| N2O | Calculation | MSW combustion | IPCC (2000) | 0.03 kg/t of MSW combusted. |  |
| N2O | Calculation | SSW combustion | IPCC (2000) | 0.8 kg/t of SSW combusted. |  |
| PAHs | Measurement | Waste Combustion | BS ISO | BS ISO11338 for extractive monitoring. | From 2006 the SPRI requires reporting of the annual emissions of listed individual PAH compounds (with individual reporting thresholds) rather than a PAH total. |
| PCBs | Measurement | Waste Combustion | CEN/BS EN | BS EN1948 for dioxin-like PCBs extractive monitoring. |  |
| Total Particulate Matter | Measurement | Waste Combustion | BS ISO/BS EN | Continuous monitoring carried out for all incinerators covered by IED. BS EN13284-2 for CEMs and BS EN13284-1 SRM. |  |
| Total Particulate Matter | Calculation | MSW Combustion | USEPA 1996 | 12.6 kg/t of MSW combusted (uncontrolled). |  |
| Total Particulate Matter | Calculation | SSW Combustion | USEPA 1995 | 233 kg/t of SSW combusted (uncontrolled). |  |
| Total Particulate Matter | Calculation | CW Combustion | USEPA 1995 | 2.33 kg/t of CW combusted (uncontrolled). |  |
| PM10 (also PM2.5) | Calculation | MSW Combustion | (Environment Agency 2000) | 0.022 kg/t of MSW combusted. |  |
| PM10 (also PM2.5) | Calculation | SSW Combustion |  | 0.075 kg/t of SSW combusted. |  |
| PM10 (also PM2.5) | Calculation | CW Combustion | CORINAIR (1999) | 0.27 kg/t of CW combusted. |  |
| SOx | Measurement | Waste Combustion | CEN/ISO/MCERTS | BS EN14181/BS 6069 (ISO 7935) for CEMs and BS EN14791 (SRM). BS ISO11632/BS ISO 6069 alternatives for extractive monitoring. |  |
| SOx | Calculation | MSW Combustion | (EA 2000) | 0.076 kg/t of MSW combusted |  |
| SOx | Calculation | SSW Combustion | USEPA 1995 | 2.3 kg/t of SSW combusted in multiple hearth (venturi scrubber) |  |
| SOx | Calculation | SSW Combustion | USEPA 1995 | 0.15 kg/t of SSW combusted in fluidised bed (uncontrolled) |  |
| SOx | Calculation | CW Combustion | USEPA 1995 | 1.09 kg/t of CW combusted in controlled air (uncontrolled) |  |
| As | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Cd | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Cu | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Cr | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Hg | Measurement | Waste Combustion | BS EN | BS EN13211 for extractive monitoring (or in series with other metals by BS EN14385 subject to adaptation). |  |
| Mn | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Ni | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Pb | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Sb | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| V | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring. |  |
| Zn | Measurement | Waste Combustion | BS EN | BS EN14385 for extractive monitoring | Although not specifically validated for zinc BS EN14385 will be applicable in most cases (see Environment Agency for England and Wales “Metals” Method Implementation Document). |

### Releases to the water environment and transfers in wastewater

Wherever possible data should be net of incoming cooling water quality, however where this would result in a negative return (i.e. removal of pollutants from incoming waters) a BRT return should be made.

The list of reference methods is not exhaustive, other analytical methods may be appropriate.

Table 9: List of reference methods

| Determinand | Method | Emission source | Literature reference | Factors and guidance |
| --- | --- | --- | --- | --- |
| As | Measurement | Effluent Treatment Facilities | BS EN ISO 11969 | Monthly flow proportional sampling. |
| Cd | Measurement | Effluent Treatment Facilities | BS EN ISO 17294 | Monthly flow proportional sampling. |
| Chlorides (as total Cl) | Measurement | Effluent Treatment Facilities | Chloride in Waters. Sewage and Effluents 1981 SCA blue book. |  |
| Chloroform | Measurement | Effluent Treatment Facilities | BS EN ISO 10301 |  |
| Cr | Measurement | Effluent Treatment Facilities | BS EN ISO 11885 BS EN ISO 17294 | Monthly flow proportional sampling. |
| Cu | Measurement | Effluent Treatment Facilities | BS EN ISO 11885 BS EN ISO 17294 | Monthly flow proportional sampling. |
| Ethylene dichloride | Measurement | Effluent Treatment Facilities | BS EN ISO 10301 |  |
| Hg | Measurement | Effluent Treatment Facilities | BS EN 13506 | Monthly flow proportional sampling. |
| Naphthalene | Measurement | Effluent Treatment Facilities | BS ISO 17993 USEPA Method 0610 |  |
| Ni | Measurement | Effluent Treatment Facilities | BS EN ISO 11885 BS EN ISO 17294 | Monthly flow proportional sampling. |
| PAHs | Measurement | Effluent Treatment Facilities | BS ISO 17993 USEPA method 0610 |  |
| Pb | Measurement | Effluent Treatment Facilities | BS EN ISO 11885 BS EN ISO 17294 | Monthly flow proportional sampling. |
| PCBs | Measurement | Effluent Treatment Facilities | USEPA methods 0680, 1668 |  |
| TOC | Measurement | Effluent Treatment Facilities |  | Continuous monitoring. |
| Tributyltin | Measurement | Effluent Treatment Facilities | BS ISO 18073:2004 |  |
| Zinc | Measurement | Effluent Treatment Facilities | BS EN ISO 11885 BS EN ISO 17294 | Monthly flow proportional sampling. |

Benzo(a)pyrene, benzo(b)fluoranthene, benzo(k)fluoranthene, indeno(1,2,3-cd)pyrene.

# 4 	Paper and wood production and processing sector guidance

This Section covers activities that are regulated as “Paper, pulp and panel manufacturing activities” under the Environmental Authorisation (Scotland) Regulations (EASR). The Environmental Authorisation (Scotland) Regulations (EASR) revoke the Pollution Prevention and Control (Scotland) Regulations 2012 (PPC)and provides an integrated environmental approach to the regulation of certain industrial activities, which is now applied to schedules 20 and 26 emissions activities under EASR. This means emissions to air, water (including discharges to sewer) and land, plus a range of other environmental effects, must be considered together.

“Paper, pulp and panel manufacturing activities” were formerly included within the scope as Part A prescribed/listed activities regulated under chapter 6, section 6.1 of the PPC Regulations. They are now found in EASR schedule 20 part 4 chapter 6 paragraph 29.

## 4.1 	 Air emissions

### 4.1.1 Relevant pollutants

The main air emissions from paper and pulp activities are shown in Table 10. The table should be taken as a guide only, and you should verify that there are no other pollutants emitted from your installation.

Table 10: Guide to main pollutants emitted by paper and wood production and processing activities to air

| Main air pollutants | Main sources - Paper | Main sources - Pulp |
| --- | --- | --- |
| CH4 | Effluent treatment plant. | Effluent treatment plant. |
| Chloroform | Bleaching, effluent treatment plant. | Recycled fibre pulping and de-inking, bleaching with chlorine dioxide or sodium hypochlorite. |
| CO | Papermaking, effluent treatment plant. |  |
| CO2 | Effluent treatment plant. | Neutral sulphite semi-chemical, with recovery. Effluent treatment plant. |
| Dioxins and furans |  | Neutral sulphite semi-chemical, with recovery. |
| Formaldehyde | From resins used in papermaking. |  |
| NOx | Effluent treatment plant. | Neutral sulphite semi-chemical, with recovery. Effluent treatment plant. |
| PAHs |  | Neutral sulphite semi-chemical, with recovery. Effluent treatment plant. |
| PM | Finishing/converting operations. | Neutral sulphite semi-chemical, with recovery. |
| SOx |  | Neutral sulphite semi-chemical, with recovery. Neutral sulphite semi-chemical, without recovery. Sulphite without recovery, unbleached. |
| VOCs | Drying processes. |  |

Note: Combustion process releases are not covered in this table.

### 4.1.2 Emission sources

These emissions are exhausted via a stack or vent, i.e. a single point source into the atmosphere. Abatement equipment, e.g. cyclones for removal of PM can be incorporated into the exhaust system prior to discharge to atmosphere.

### 4.1.3 Fugitive emissions

Fugitive emissions are those that are not released from a point source such as a stack. Fugitive emissions are not considered to be significant in the paper and wood production and processing industry.

Some examples of the potential sources of fugitive emissions to air are:

- Storage areas (e.g. bays, stockpiles, lagoons, etc.);

- Transferring material between vessels;

- Conveyor systems;

- Pipe work and ductwork systems (e.g. pumps, valves, flanges);

- Abatement equipment by-pass;

- Accidental loss of containment from failed plant and equipment;

- Open vessels (in particular the effluent treatment plant);

- Spillages; and

- Poor building containment and extraction.

Only fugitive emissions that leave the installation need to be reported to the SPRI. Whilst contained spills would therefore not need to be reported, you should report vapour emissions that may have dispersed.

## 4.2 	Emissions to water and wastewater

### 4.2.1 Relevant pollutants and emission sources

A variety of substances are present in water discharges from paper and wood production and processing activities (with some residual organic constituents of the effluent not known in detail). Table 11 illustrates the main pollutants emitted to water and their main sources. Not all of these are SPRI reportable substances, but you need to take account of any reportable substances emitted in association with them (for example, metal contents incorporated with total suspended solids). The table should be taken as a guide only, and you should verify that there are no other reportable substances emitted from the process.

Table 11: Main water pollutants generated by and wood production and processing activities

| Main water pollutants | Main sources - Paper | Main sources - Pulp |
| --- | --- | --- |
| Alcohols/Acids | Papermaking. | Pulping processes. |
| Cadmium | Papermaking. | Pulping processes, bleaching. |
| Chloroform |  | Bleaching, RCF pulping & deinking. |
| Defoamers | Papermaking. | RCF pulping & de-inking. |
| Formaldehyde | Papermaking. |  |
| Heavy Metals | Papermaking. | Pulping processes, bleaching. |
| Mercury | Papermaking. | Pulping processes, bleaching. |
| Ammonia | Papermaking, effluent treatment. |  |
| PM/TSS | Papermaking. | Pulping processes, bleaching. |
| Pentachlorophenol | Papermaking. | RCF pulping & de-inking. |
| Phosphates/Nitrates | Papermaking, effluent treatment. | Pulping processes. |
| SOx |  | Mechanical pulping with no recovery. |
| Sulphides | Papermaking, effluent treatment. | RCF pulping & de-inking, mechanical pulping. |
| Sulphites & Sulphates | Papermaking. | RCF pulping & de-inking, mechanical pulping. |
| Surfactants | Papermaking. | RCF pulping & de-inking. |
| Wood organics | Papermaking. | Pulping processes, bleaching. |

The resulting discharge of the above substances depends on the ‘in-process’ preventative measures (e.g. good housekeeping, re-use) and the presence and technical standards of wastewater treatment facilities.

Notwithstanding the above, you should consider all emission sources to water and characterise the flows and emission concentrations from each source.

## 4.3 Off-site waste transfers

### 4.3.1 Relevant wastes

In general, the waste streams from paper and pulp facilities comprise the following (EWC codes for specific waste streams are shown in brackets for convenience):

- Green liquor sludge – from recovery of cooking liquor (EWC Code 03 03 02).

- Mechanically-separated rejects from pulping of waste paper and cardboard (EWC 03 03 07).

- De-inking sludges from paper recycling (EWC 03 03 05).

- Wastes from sorting of paper and cardboard destined for recycling (EWC 03 03 08).

- Fibre rejects, fibre-, filler- and coating-sludges from mechanical separation (EWC 03 03 10).

- Sludges from on-site effluent treatment (EWC 03 03 11).

- General inert industrial waste.

- Waste bark and wood (EWC Code 03 03 01).

- Road tanker waste removed from the site during the reporting year.

Transboundary shipments of hazardous waste

For transboundary movements of hazardous waste (out with the United Kingdom), the name and address of the recoverer or the disposer of the waste and the actual recovery or disposal site have to be reported.

# 5 	Cement and lime sector guidance

This Section covers activities that are regulated as “Production of cement, lime and magnesium oxide activities” under the Environmental Authorisation (Scotland) Regulations (EASR). The Environmental Authorisation (Scotland) Regulations (EASR) revoke the Pollution Prevention and Control (Scotland) Regulations 2012 (PPC) and  provides an integrated environmental approach to the regulation of certain industrial activities, which is now applied to schedules 20 and 26 emissions activities under EASR. This means emissions to air, water (including discharges to sewer) and land, plus a range of other environmental effects, must be considered together.

“Production of cement, lime and magnesium oxide activities” were formerly included within the scope as Part A prescribed/listed activities regulated under chapter 3, section 3.1 of the PPC Regulations. They are now found in EASR schedule 20 part 4 chapter 3 paragraph 11.

## 5.1 	Air emissions

### 5.1.1 Relevant pollutants

The main air emissions from cement and lime production activities are shown in Lists 6,7 and 8 respectively. The Lists should be taken as a guide only, and you should verify that there are no other pollutants emitted from your process. See Annexes 1 and 2 for a summary of currently used RETs and guidance for releases of substances to air from various sources.

List 6 Main air pollutants emitted by cement and related activities and their release points

Release point: Kiln exhaust gas:

- Carbon monoxide.

- Carbon dioxide.

- Dioxins and furans.

- Hydrogen chloride.

- Flourine and inorganic compounds – as HF

- Nitrous oxide.

- Sulphur dioxide.

- Metals (semi-volatile/volatile metals only).

Release points: Kiln exhaust gas, other point sources and fugitive sources (smaller bag filters) such as raw material preparation, handling and storage, fuel preparation, handling and storage, kiln, cooler, cement milling, product handling and storage:

- Particulate matter (including PM10).

Release points: Kiln exhaust gas, fugitive sources from fuel preparation, handling and storage.

- NMVOCs

List 7 - Main air pollutants emitted by lime and related activities and their release points (gas-fired kilns)

Release point: Kiln exhaust gas:

- Carbon monoxide.

- Carbon dioxide.

- Nitrous oxide.

- Sulphur oxides

Release points: Kiln exhaust gas, other point sources and fugitive sources such as raw material preparation, handling and storage, fuel preparation, handling and storage, kiln, hydrator, product handling and storage:

- Total Particulate matter (including PM10)

List 8 - Main air pollutants emitted by lime and related activities and their release points (where fuels other than gas are used)

Release point: Kiln exhaust gas:

- Dioxins and furans.

- Hydrogen chloride.

- Flourine and inorganic compounds – as HF.

- Metals.

Release points: Kiln exhaust gas, fuel preparation, handling and storage:

- NMVOCs.

### 5.1.2 Emissions sources

These emissions are exhausted via a stack or vent; i.e. a single point source into the atmosphere. Abatement equipment; e.g. scrubbing units, fabric filters (bag house) can be incorporated into the exhaust system prior to discharge to atmosphere. Point source (main stack) emissions will normally be the most significant emission source (in terms of annual mass releases) for combustion activities taking place within kilns and clinker coolers at cement and lime production sites.

### 5.1.3 Fugitive emissions

Fugitive emissions are those that are not released from a point source such as a stack.

Some examples of the potential sources of fugitive emissions to air are:

External stockpiled materials;

- Transferring material between vessels.

- Uncovered conveyor systems.

- Pipework and ductwork systems (e.g. pumps, valves, flanges).

- Accidental loss of containment from failed plant and equipment.

- Packing plant.

- Spillages and

- Ancillary bag filters.

### 5.1.4 Emission factors

###### Cement process emission factors

Refer to Reference Tables – Cement source factors cement production source emission factors provided by the British Cement Association (BCA).

Lime process emission factors

Refer to Reference Tables – Lime source factors for lime process source factors.

## 5.2 	Emissions to water and wastewater

### 5.2.1 Relevant pollutants and emission sources

Water discharges from cement and lime processes arise from storm water, from cooling water, from accidental emissions of raw materials, products or waste materials and from firefighting resulting from industrial accidents. List 9 illustrates the main pollutant emitted to water and its main sources. Suspended solids alone are not an SPRI-reportable substance. However, the table should be taken as a guide only, and you should verify that there are no other substances discharged from the process, including in association with suspended particulate matter.

List 9 - The main water pollutants generated by cement and lime and related activities

Main sources: Raw material preparation, storage & handling

- Suspended solids

Discharge of substances depends on the ‘in-process’ preventative measures (good housekeeping, reuse) and the presence and technical standards of wastewater treatment facilities.

Notwithstanding the above, you should consider all emission sources to water and characterise the flows and emission concentrations from each source.

## 5.3 	Off-site waste transfers

### 5.3.1 Relevant wastes

In general the waste streams from cement and lime production comprises:

- Cement and lime kiln dust collected in particulate abatement devices.

- Materials arising from process clean-outs.

- Spent kiln-liner and refractory bricks.

- Waste oils and lubricants.

- Chemical containers and general inert industrial waste and

- By-pass dust (cement works).

Transboundary shipments of hazardous waste

For transboundary movements of hazardous waste (outwith the United Kingdom), the name and address of the recoverer or the disposer of the waste and the actual recovery or disposal site have to be reported.

# Reference Tables – Cement source factors

The Reference Tables are designed to be an aid for compiling SPRI submissions from cement production processes, and covers the major categories of emission sources; process emissions, combustion emissions, fugitive emissions, wastewater, etc. It also includes emission factors where applicable.

The table below addresses all the pollutants which we consider likely to be reported from typical cement production processes.

### Emissions to air

Table 12: Major categories of emission sources

| Determinand | Method | Emission Source | Reference | Factors and guidance | Notes |
| --- | --- | --- | --- | --- | --- |
| CO | Measurement/Calculation | Cement kiln | BCA 2004 | 1.94 kg/te of clinker produced. |  |
| CO2 | Calculation | Site combustion and process sources | ETS | UK ETS | The threshold for CO2 applies to the overall CO2 emission (thermal and chemical combined). The percentage biomass figure; i.e. the percentage of your CO2 figure attributed to non-fossil fuel combustion sources can be recorded within your return form. |
| CH4 | Calculation | Cement kiln | BCA 2004 | 0.036 kg/te of clinker produced. |  |
| Dioxins and furans | Measurement/Calculation | Cement kiln | BCA 2004 | 2 x 10-9 kg TEQ/te of clinker produced |  |
| HCl | Measurement/Calculation | Cement kiln | BCA 2004 | 2.3 x 10-2 kg/te of clinker produced |  |
| Fluorine and inorganic compounds – as HF | Measurement/Calculation | Cement kiln | None | None |  |
| NH3 | Measurement/Calculation | Cement kiln | BCA 2004 | 1.4 x 10-2 kg/te of clinker produced |  |
| NMVOCs | Measurement/Calculation | Cement kiln | BCA 2004 | 0.1 kg/te of clinker produced |  |
| NOx | Measurement/Calculation | Cement kiln | BCA 2004 | 3.19 kg/te of clinker produced |  |
| PAHs | Measurement/Calculation | Cement kiln | BCA 2004 | 2.43 x 10-4 kg/te of clinker produced | From 2006 the SPRI requires reporting of the annual emissions of listed individual PAH compounds (with individual reporting thresholds) rather than a PAH total. |
| Total particulate matter | Measurement/Calculation | Cement kiln | BCA 2004 | 0.16 kg/te of clinker produced. |  |
| PM10 | Measurement/Calculation | Cement kiln | BCA 2004 | 0.09 kg/te of clinker produced. |  |
| SOx | Measurement/Calculation | Cement kiln | BCA 2004 | 1.69 kg/te of clinker produced. |  |

Emissions to air – Metals

Table 13: Major categories of emission sources

| Determinand | Method | Emission Source | Reference | Factors and guidance |
| --- | --- | --- | --- | --- |
| As | Measurement/Calculation | Cement kiln | BCA 2004 | 2.0 x 10-6 kg/te of clinker produced. |
| Cd | Measurement/Calculation | Cement kiln | BCA 2004 | 6.0 x 10-6 kg/te of clinker produced. |
| Cu | Measurement/Calculation | Cement kiln | BCA 2004 | 1.9 x 10-5 kg/ te of clinker produced. |
| Cr | Measurement/Calculation | Cement kiln | BCA 2004 | 4.8 x 10-5 kg/ te of clinker produced. |
| Hg | Measurement/Calculation | Cement kiln | BCA 2004 | 2.2 x 10-5 kg/ te of clinker produced. |
| Mn | Measurement/Calculation | Cement kiln | BCA 2004 | 6.0 x 10-5 kg te of clinker produced. |
| Ni | Measurement/Calculation | Cement kiln | BCA 2004 | 2.3 x 10-5 kg/te of clinker produced. |
| Pb | Measurement/Calculation | Cement kiln | BCA 2004 | 6.95 x 10-4 kg/te of clinker produced. |
| V | Measurement/Calculation | Cement kiln | BCA 2004 | 1.1 x 10-5 kg/te of clinker produced. |

# Reference Tables – Lime source factors

The Reference tables below shows the pollutants likely to be reported from typical cement production processes.

### Emissions to air

Table 14: Pollutants likely to be reported from typical cement processes

| Determinand | Method | Emission Source | Reference | Factors and guidance |
| --- | --- | --- | --- | --- |
| CO | Measurement | Lime kiln | None | None |
| CO2 | Calculation | Site combustion and process sources | ETS | ETS |
| CH4 | Calculation | Lime kiln | None | None |
| Dioxins and furans | Measurement | Lime kiln | None | None |
| HCl | Measurement | Lime kiln | None | None |
| Fluorine and inorganic compounds – as HF | Measurement | Lime kiln | None | None |
| NMVOCs | Measurement | Lime kiln | None | None |
| NOx | Measurement | Lime kiln | None | None |
| PAHs | Measurement | Lime kiln | None | None |
| Total particulate matter | Measurement | Lime kiln | None | None |
| PM10 | Measurement | Lime kiln | None | None |
| SOx | Measurement | Lime kiln | None | None |

### Emissions to air – metals

Table 15: Pollutants likely to be reported from typical cement processes

| Determinand | Method | Emission Source | Reference | Factors and guidance |
| --- | --- | --- | --- | --- |
| As | Measurement | Lime kiln | None | None |
| Cd | Measurement | Lime kiln | None | None |
| Cu | Measurement | Lime kiln | None | None |
| Cr | Measurement | Lime kiln | None | None |
| Hg | Measurement | Lime kiln | None | None |
| Mn | Measurement | Lime kiln | None | None |
| Pb | Measurement | Lime kiln | None | None |

CO2 : The threshold for CO2 applies to the overall CO2 emission (thermal and chemical combined). The percentage biomass figure, i.e. the percentage of your CO2 figure attributed to non-fossil fuel combustion sources, can be recorded within your return form.

# 6 	Production and processing of metals sector guidance

This Section covers activities that are regulated as “Production and processing of metals activities” under the Environmental Authorisation (Scotland) Regulations (EASR). The Environmental Authorisation (Scotland) Regulations (EASR) revoke the Pollution Prevention and Control (Scotland) Regulations 2012 (PPC) and provides an integrated environmental approach to the regulation of certain industrial activities, which is now applied to schedules 20 and 26 emissions activities under EASR. This means emissions to air, water (including discharges to sewer) and land, plus a range of other environmental effects, must be considered together.

“Production and processing of metals activities” were formerly included within the scope as Part A prescribed/listed activities regulated under Chapter 2, Sections 2.1 (Ferrous metals) and 2.2 (Non-ferrous metals) of the PPC Regulations. They are now found in EASR schedule 20 part 4 chapter 2.

## 6.1 	Air Emissions

### 6.1.1 Relevant pollutants

The main air emissions from ferrous and non-ferrous metals production and processing activities are shown in Tables 16 and 17 respectively. The tables should be taken as a guide only, and you should verify that there are no other pollutants emitted from your process.

###### Table 16: Main air pollutants emitted by ferrous metal and related activities and their main sources

| Main air pollutants | Release point |
| --- | --- |
| CO2 | Sinter plant, hot blast stoves, blast furnaces, basic oxygen steelmaking (BOS) blowing & charging operations, arc furnace melting and refining, arc furnace and ladle treatment scarfing, coke ovens re-heating. |
| CO | Sinter plant, hot blast stoves, blast furnaces, BOS blowing and charging operations, arc furnace melting and refining, scarfing, emergency venting, re-heat furnaces. |
| NOx | Sinter plant, hot blast stoves, arc furnace charging, melting and refining, nitric acid pickling, coke oven under-firing, emergency flaring, ammonia incinerator tail gas. |
| Particulate matter (including PM10) | Raw material handling, sinter plant, coal pulverisation, stockpiles, blast furnaces, desulphurisation plant, BOS/electric arc primary and secondary extraction charging and tapping operations, ingot casting, scarfing, skull burning, coke oven door leakage/underfiring/charging/pushing, emergency venting, emergency flaring, quenching ammonia incinerator tail gas. |
| SO2 | Sinter plant, hot blast stoves, arc furnace charging, melting and refining, slag processing, coke oven underfiring, ammonia incinerator tail gas, blast furnace casthouse extraction, re-heating. |
| VOCs | Sinter plant, coke oven charging emissions, door leakage and by-products plant, decanters and storage tank vents. |
| NH3 | Coke oven charging emissions/door leakage. |
| HCl | Sinter plant, hydrochloric acid pickling. |
| HCN | Coke oven charging/door and top leakage. |
| Fluorine and inorganic compounds – as HF | Sinter plant, arc furnace melting and refining, ladle treatments, electro slag re-melting. |
| H2S | Slag processing, coke oven door leakage, emergency venting, decanter and storage tank vents. |
| Metals | Sinter plant, BOS charging & tapping operations, arc furnace charging, melting and refining. |
| Dioxins & furans | Sinter plant, arc furnace steel making. |
| CH4 | Sinter plant, coke oven door leakage. |
| PCBs | Sinter plant, electric arc furnace steel making. |
| BTX | Coke oven door leakage, emergency venting, decanters and storage tank vents. |
| PAHs (Reference to PAHs is for collective convenience. See the SPRI Schedule for which individual PAHs you should report) | Sinter plant, electric arc furnace steel making, coke oven door and top leakage. |
| Phenols | Coke oven door and top leakage, emergency venting, decanters and storage tank vents, coke oven by-product plant. |

Table 17: Main air pollutants emitted by non-ferrous metal and related activities and their main sources

| Main air pollutants | Release point |
| --- | --- |
| CO2 | Melting furnaces. |
| CO | Shaft furnaces in copper processes, nickel and cobalt processes. |
| NOx | Pre-treatment; copper, aluminium, lead, zinc, cadmium, precious metal, refractory metal, ferro-alloy, alkali and alkaline metal, nickel and cobalt processes; carbon and graphite production. |
| Particulate matter (including PM10 as a % of total) | Raw material storage and handling; pre-treatment; copper, aluminium, lead, zinc, cadmium, precious metal, refractory metal, ferro-alloy, alkali and alkaline metal, nickel and cobalt processes; carbon and graphite production; residue handling storage and re-use; air abatement plant. |
| SO2 | Pre-treatment; copper, aluminium, lead, zinc, cadmium, precious metal, refractory metal, ferro-alloy, alkali and alkaline metal, nickel and cobalt processes; carbon and graphite production. |
| VOCs | Pre-treatment; copper, aluminium, lead, zinc, cadmium, precious metal, refractory metal, ferro-alloy, alkali and alkaline metal, nickel and cobalt processes; carbon and graphite production; air abatement plant. |
| NH3 | Aluminium, precious metal, refractory metal processes. |
| HCl | Aluminium and precious metal processes, pickling |
| Fluorine and inorganic compounds – as HF | Aluminium and precious metal processes, pickling |
| Chlorine and inorganic compounds – as HCl | Aluminium, lead, zinc, cadmium, precious metal, refractory metal, alkali and alkaline metal processes. |
| Dioxins & furans | Pre-treatment; copper, aluminium, lead, zinc, cadmium, precious metal, refractory metal, ferro-alloy, alkali and alkaline metal, nickel and cobalt processes; carbon and graphite production; residue handling storage and re-use; air abatement plant |
| Sulphides and mercaptans | Precious metal processes. |
| PAHs (Reference to PAHs is for collective convenience. See the SPRI Schedule for which individual PAHs you should report) | Aluminium processes, carbon and graphite production. |
| PFCs | Aluminium processes. |
| Non-ferrous metals | Raw material storage and handling; pre-treatment; copper, aluminium, lead, zinc, cadmium, precious metal, refractory metal, ferro-alloy, alkali and alkaline metal, nickel and cobalt processes; carbon and graphite production; residue handling storage and reuse; air abatement plant. |

### 6.2.2 Point source emissions

These emissions are exhausted via a stack or vent; i.e. a single point source into the atmosphere. Abatement equipment; e.g. scrubbing units, fabric filters (bag house) can be incorporated into the exhaust system prior to discharge to atmosphere. Point source emissions will be the most significant emission source for combustion activities taking place within metals processing and production sites. Other non-combustion point source emissions (sintering, coke-making, smelting, refining, alloying, etc.) arise from a IEDe range of ferrous and non-ferrous processes and activities.

### 6.2.3 Fugitive emissions

Fugitive emissions are those that are not released from a point source such as a stack. Only fugitive emissions that leave the site need to be reported to the SPRI. Whilst contained spills would therefore not need to be reported, you should report vapour emissions that may have dispersed.

Ferrous metals

Fugitive emissions from coke and iron and steel processes can be as significant as point source

emissions. Some examples of the potential sources of fugitive emissions to air are:

- Leakage of volatile materials through ascension pipe, door and lid seals on coke ovens.

- Open vessels (e.g. coke oven by-product plant).

- Cast house fume abatement systems overloaded; BOS primary and secondary fume collection.

- Coal and ore stockpiles.

- Vehicle loading and unloading areas.

- Transferring material between vessels (e.g. furnace, ladle, silos); Conveyor systems.

- Pipe work and ductwork systems (e.g. pumps, valves, flanges).

- Abatement equipment by-pass (e.g. coke side particulate collection hoods).

- Accidental loss of containment from failed plant and equipment.

- Solvents and

- Miscellaneous un-extracted combustion activities.

Non-ferrous metals

At most sites where non-ferrous metals processes are carried out, fugitive emissions to air give rise to a significant proportion of the overall environmental impact. Some examples of the sources of fugitive emissions to air are:

- Smelting, melting and refining furnaces and their associated extraction systems.

- Transfer operations involving molten metal.

- Casting and associated activities.

- Handling and storage of dusty raw materials such as concentrates, foundry sand,

- Drosses.

- Handling, storage and disposal of wastes such as drosses, slags and skimmings.

- Fume treatment plant.

- Ladle heating stations and

- Vehicle loading and unloading areas.

## 6.3 Emissions to water and wastewater

### 6.3.1 Relevant pollutants and emissions sources

Water discharges from ferrous and non-ferrous metals processes can be contaminated by a variety of polluting substances. Tables 18 and 19 illustrate the main substances emitted to water and their main sources. The tables should be taken as a guide only, and you should verify that there are no other pollutants emitted from the process. Suspended solids, and oils and greases are not SPRI-reportable substances; however, you may need to take account of other substances that are being emitted in association with such materials.

Table 18: Main water pollutants generated by ferrous metal and related activities

| Main water pollutants | Release point |
| --- | --- |
| Oils and greases | Steel scrap storage and handling, residue handling, storage and re-use, continuous casting, rolling mills. |
| NH3 | Blast furnace primary gas cleaning, effluent plant, coke oven by-products |
| Cyanides – as CN | Blast furnace primary gas cleaning, effluent plant, coke oven by-products. |
| PAHs | Coke oven by-products. |
| Suspended solids | Raw material storage and handling, residue handling, storage and re-use, effluent plant, blast furnace and BOS gas cleaning, rolling mills. |
| Metals | Effluent treatment discharge storage and reuse, continuous casting, rolling mills, by-products plant |
| TOC | Steel scrap storage and handling, residue handling, storage and reuse, continuous casting, rolling mills, by-products plant |

Table 19: Main water pollutants generated by non-ferrous metal and related activities

| Main water pollutants | Release point |
| --- | --- |
| Oils and greases | Raw material storage and handling, residue handling, storage and re-use, effluent plant. |
| NH3 | Aluminium, precious metal, refractory metal processes, effluent plant. |
| Cyanides – as CN | Aluminium, precious metal processes, effluent plant |
| PAHs | Aluminium processes, carbon and graphite production. |
| Suspended solids | Raw material storage and handling, residue handling, storage and re-use, effluent plant. |
| PFCs | Aluminium processes |
| Cd | Raw material storage and handling, effluent plant. |
| Hg | Raw material storage and handling, effluent plant. |
| Other metals | Effluent treatment discharge. |

The resulting discharge of the above substances depends on the ‘in process’ preventative measures (good housekeeping, re-use) and the presence and technical standards of wastewater treatment facilities.

## 6.4 Off-site waste transfers

### 6.4.1 Relevant wastes

#### Integrated steelworks

The management of residues in an integrated steelworks is characterised by advanced techniques for extracting value from recycling most of the residues in the sinter plant, BOS plant and coke ovens. The iron and steel sector is an important recycler of materials. All the ferrous scrap arisings in the UK is recycled, the bulk of it by the UK steel industry. Only small parts of the overall quantity of wastes are landfilled, including the following:

- Fine dust from blast furnace gas cleaning.

- Rubble.

- Fine dust from basic oxygen furnace gas scrubbing (where wet cleaning is used).

- Alkali metal chlorides and heavy metal chlorides from electrostatic precipitators.

- Refractory waste and

- General wastes (e.g. packaging).

#### Hot and cold rolling mills

Hot rolling operations generate the following waste products which are sent off-site:

- Metal-bearing wastes (e.g. oily mill scale).

- Dusts from scarfing and rolling.

- Oily and non-oily mill scale.

- Water treatment and mill scale sludge and

- Oil and greases (can be used as secondary fuel in blast furnace or coke ovens).

Cold rolling operations generate the following waste products which are sent off-site:

- Acid neutralisation sludges and

- Oily sludges.

#### Non-ferrous metal activities

The production of non-ferrous metals from primary and secondary raw materials gives rise to the generation of a IEDe variety of by-products, intermediate products and residues some of which are given below:

- Skimmings.

- Tars/carbon-containing wastes from anode manufacture.

- Flue gas dust.

- Gas treatment sludges.

- Lining and refractory waste.

- Wastewater treatment sludge.

- Waste oils and

- Absorbents and filter materials.

#### Transboundary shipments of hazardous waste

For transboundary movements of hazardous waste (outwith the United Kingdom), the name and address of the recoverer or the disposer of the waste and the actual recovery or disposal site have to be reported.

# Reference Tables - Ferrous metals source factors

The Reference Tables are designed to be an aid for compiling SPRI submissions from ferrous metal processes, and covers the major categories of emission sources; process emissions (integrated iron and steel processes, coke-making and rolling mills), combustion emissions (heaters, boilers, etc.), fugitive emissions, wastewater, etc. It also includes emission factors where available.

The table below addresses all the pollutants which we consider potentially emitted from typical ferrous metal processes.

### Emissions to air

Table 20: Main water pollutants generated by non-ferrous metal and related activities

| Determinand | Method | Emission Source |
| --- | --- | --- |
| Benzene | Calculation | Coke-making |
| Cd | Measurement/Calculation | Coke-making |
| Cd | Measurement | Sinter plant |
| Cd | Measurement/Calculation | Blast furnace |
| Cd | Measurement | Steel making |
| Cd | Calculation | 1st and 2nd rolling mills |
| Cd | Measurement/Calculation | Combustion |
| CO | Measurement/Calculation | Combustion |
| CO | Measurement/Calculation | Coke oven charging and door leakage |
| CO | Measurement/Calculation | Coke oven pushing |
| CO | Measurement/Calculation | Sinter plant wind box |
| CO | Calculation | BOS refining |
| CO | Calculation | EAF steel making |
| CO | Calculation | 1st and 2nd rolling mills |
| CO2 | Calculation | Combustion and process sources (Reference: ETS). |
| CH4 | Calculation | Combustion |
| CH4 | Calculation | Coke-making |
| Dioxins and furans | Measurement | Sinter plant |
| Dioxins and furans | Measurement | EAF steel making |
| HCl | Measurement/Calculation | Combustion |
| HCl | Measurement/Calculation | Sinter plant |
| HCl | Measurement/Calculation | Acid pickling |
| HCN | Measurement/Calculation | Coke-making |
| F and inorganic compounds – as HF | Measurement/Calculation | Sinter plant |
| F and inorganic compounds – as HF | Measurement/Calculation | EAF steel making |
| H2S | Measurement/Calculation | Coke-making |
| NH3 | Measurement/Calculation | Coke oven charging |
| NH3 | Measurement/Calculation | Coke oven door leakage |
| NH3 | Measurement/Calculation | Coke oven pushing |
| NH3 | Calculation | 1st and 2nd rolling mills |
| NMVOCs | Measurement/Calculation | Coke oven charging |
| NMVOCs | Measurement/Calculation | Coke oven door leakage |
| NMVOCs | Measurement/Calculation | Coke oven pushing |
| NMVOCs | Measurement/Calculation | Sinter plant |
| NOx | Calculation | Combustion |
| NOx | Measurement/Calculation | Coke oven charging |
| NOx | Measurement/Calculation | Coke oven door leakage |
| NOx | Calculation | EAF steel making |
| NOx | Measurement/Calculation | Sinter plant |
| NOx | Measurement/Calculation | Blast furnace |
| NOx | Calculation | 1st and 2nd rolling mills |
| N2O | Calculation | Combustion |
| PAHs | Calculation | Combustion |
| PAHs | Measurement/Calculation | Coke-making |
| PAHs | Measurement/Calculation | Sinter plant |
| PAHs | Measurement/Calculation | Blast furnace |
| PCBs as WHO-TEQ | Measurement | EAF steel making |
| PCBs as WHO-TEQ | Measurement | Sinter plant |
| Phenol | Measurement/Calculation | Coke-making |
| Phenol | Measurement/Calculation | Sinter plant |
| Total particulate matter | Calculation | Combustion |
| PM10 (also PM2.5) | Calculation | Combustion |
| PM10 (also PM2.5) | Measurement/Calculation | Coke oven charging |
| PM10 (also PM2.5) | Measurement/Calculation | Coke oven door leakage |
| PM10 (also PM2.5) | Measurement/Calculation | Coke oven pushing |
| PM10 (also PM2.5) | Measurement/Calculation | Coke oven flue gas |
| PM10 (also PM2.5) | Measurement/Calculation | Sinter plant wind box |
| PM10 (also PM2.5) | Measurement/Calculation | Sinter plant waste gas discharge |
| PM10 (also PM2.5) | Calculation | Blast furnace slips |
| PM10 (also PM2.5) | Calculation | Blast furnace cast house, furnace and tap hole/through |
| PM10 (also PM2.5) | Calculation | Hot metal desulphurisation |
| PM10 (also PM2.5) | Calculation | BOS refining |
| PM10 (also PM2.5) | Calculation | BOS charging |
| PM10 (also PM2.5) | Calculation | BOS tapping |
| PM10 (also PM2.5) | Calculation | BOS hot metal transfer |
| PM10 (also PM2.5) | Calculation | EAF steel making |
| PM10 (also PM2.5) | Calculation | Steel teeming |
| PM10 (also PM2.5) | Calculation | Steel casting |
| PM10 (also PM2.5) | Calculation | 1st and 2nd rolling mills |
| SOx | Calculation | Combustion (Reference: Industry experience. Guidance from ‘fuel sulphur content x flow rate’.). |
| SOx | Calculation | Sinter plant |
| SOx | Measurement/Calculation | Coke oven charging |
| SOx | Measurement/Calculation | Coke oven flue gas |
| SOx | Calculation | Blast furnace |
| Toluene | Measurement/Calculation | Coke-making |
| Toluene | Measurement/Calculation | Sinter plant |
| Toluene | Measurement/Calculation | Blast furnace |
| Xylene | Measurement/Calculation | Coke-making |
| Xylene | Measurement/Calculation | Sinter plant |
| Xylene | Measurement/Calculation | Blast furnace |
| Zinc | Measurement/Calculation | Sinter plant |
| As | Measurement | Not provided |
| Cr | Measurement | Not provided |
| Cu | Measurement | Not provided |
| Mn | Measurement | Not provided |
| Ni | Measurement | Not provided |
| Pb | Measurement | Not provided |
| Sb | Measurement | Not provided |

H2S: Not stated on SPRI list of pollutants, but reportable as “Other individual acid-forming gases”.

PAH: From 2006 the SPRI required reporting of the annual emissions of listed individual PAH compounds (with individual reporting thresholds) rather than a PAH total.

### Releases to the water environment and transfers in wastewater

Wherever possible data should be net of incoming cooling water quality, however where this would result in a negative return (i.e. removal of pollutants from incoming waters) a BRT return should be made. Where a site is reporting for the first year the return is pre-populated with an indicative list.

### Releases to water environment and transfer in waste water – Effluent Treatment Facilities

Table 21: Main water pollutants generated by non-ferrous metal and related activities

| Determinand | Method | Factors and Guidance |
| --- | --- | --- |
| As | Measurement | Effluent quality data |
| Cd | Measurement | Effluent quality data |
| Chlorides (as total Cl) | Measurement | Effluent quality data |
| Cr | Measurement | Effluent quality data |
| Cu | Measurement | Effluent quality data |
| Cyanides (as total CN) | Measurement | Effluent quality data |
| Fluorides (as total F) | Measurement | Effluent quality data |
| Hg | Measurement | Effluent quality data |
| NH3 | Measurement | Effluent quality data |
| Ni | Measurement | Effluent quality data |
| Pb | Measurement | Effluent quality data |
| PAHs | Measurement | Effluent quality data |
| Phenols (as C) | Measurement | Effluent quality data |
| TOC | Measurement | Effluent quality data |
| ZN | Measurement | Effluent quality data |

Benzo(a)pyrene, benzo(b)fluoranthene, benzo(k)fluoranthene, indeno(1, 2, 3-cd)pyrene.

# Reference Tables - Non-ferrous metals source factors

This Annex is designed to be an aid for compiling SPRI submissions from non-ferrous metal processes, and covers the major categories of emission sources; process emissions, combustion emissions (heaters, boilers, etc.), fugitive emissions, wastewater, etc. The Reference Tables below address all the pollutants which we consider potentially emitted from typical non-ferrous metal processes.

### Emissions to air

Table 22: Main water pollutants generated by non-ferrous metal and related activities

| Determinand | Method | Emission Source |
| --- | --- | --- |
| Benzene | Not provided | Not provided |
| Cd | Measurement | Not provided |
| Cl and inorganic compounds as Cl | Not provided | Not provided |
| CO | Measurement | Combustion |
| CO2 | Calculation | Site combustion and process (Reference: ETS) |
| CH4 | Calculation | Combustion |
| Dioxins and furans | Measurement | Process |
| HCl | Measurement | Not provided |
| F and inorganic compounds as HF | Calculation | Primary aluminium alumina reduction |
| HFCs | Not provided | Not provided |
| Hg | Measurement | Not provided |
| NH3 | Measurement | Not provided |
| NMVOCs | Measurement | Not provided |
| NOx | Calculation | Combustion (Reference: EA. Guidance from site-specific factors agreed for combustion plant.). |
| N2O | Calculation | Combustion |
| PAHs | Calculation | Combustion |
| PAHs |  | Primary aluminium alumina reduction |
| PFCs | Not provided | Not provided |
| Total particulate matter | Calculation | Combustion |
| Total particulate matter | Calculation | Primary aluminium anode production |
| Total particulate matter | Calculation | Primary aluminium alumina reduction pre-bake cell |
| Total particulate matter | Calculation | Secondary aluminium sweating furnace |
| Total particulate matter | Calculation | Secondary aluminium smelting (crucible furnace) |
| Total particulate matter | Calculation | Secondary aluminium smelting (reverberatory furnace) |
| Total particulate matter | Calculation | Secondary aluminium chlorine demagging |
| Total particulate matter | Calculation | Secondary aluminium refining |
| PM10 | Calculation | Combustion |
| PM10 | Calculation | Secondary magnesium pot furnace |
| PM10 | Calculation | Primary zinc roasting |
| PM10 | Calculation | Primary zinc sinter plant |
| PM10 | Calculation | Primary zinc vertical retort |
| PM10 | Calculation | Primary zinc electric retort |
| PM10 | Calculation | Primary zinc electrolytic process |
| PM10 | Calculation | Secondary zinc reverberatory sweating |
| PM10 | Calculation | Secondary zinc kettle pot |
| PM10 | Calculation | Secondary zinc calcining |
| PM10 | Calculation | Secondary zinc electric resistance sweating |
| PM10 | Calculation | Secondary zinc muffle sweating |
| PM10 | Calculation | Secondary zinc kettle sweating |
| PM10 | Calculation | Secondary zinc rotary sweating |
| PM10 | Calculation | Secondary zinc retort and muffle distillation |
| PM10 | Calculation | Secondary zinc retort distillation/oxidation |
| PM10 | Calculation | Secondary zinc muffle distillation/oxidation |
| PM10 | Calculation | Secondary zinc retort reduction |
| PM10 | Calculation | Secondary zinc fugitive emissions |
| PM10 | Calculation | Primary lead ore crushing |
| PM10 | Calculation | Primary lead ore screening |
| PM10 | Calculation | Primary lead tetrahedrite dryer |
| PM10 | Calculation | Primary lead sinter machine |
| PM10 | Calculation | Primary lead sinter building fugitives |
| PM10 | Calculation | Primary lead blast furnace |
| PM10 | Calculation | Secondary lead fugitive emissions |
| PM10 | Calculation | Secondary lead sweating |
| PM10 | Calculation | Secondary lead reverberatory smelting |
| PM10 | Calculation | Secondary lead blast smelting |
| PM10 | Calculation | Secondary lead kettle oxidation |
| PM10 | Calculation | Secondary lead casting |
| PM10 | Calculation | Secondary lead kettle refining |
| PM10 | Calculation | Primary copper reverberatory furnace followed by converter |
| PM10 | Calculation | Primary lead ore crushing |
| PM10 | Calculation | Primary lead ore screening |
| PM10 | Calculation | Primary lead tetrahedrite dryer |
| PM10 | Calculation | Primary lead sinter machine |
| PM10 | Calculation | Primary lead sinter building fugitives |
| PM10 | Calculation | Primary lead blast furnace |
| PM10 | Calculation | Secondary lead fugitive emissions |
| PM10 | Calculation | Secondary lead sweating |
| PM10 | Calculation | Secondary lead reverberatory smelting |
| PM10 | Calculation | Secondary lead blast smelting cupola |
| PM10 | Calculation | Secondary lead kettle oxidation |
| PM10 | Calculation | Secondary lead casting |
| PM10 | Calculation | Secondary lead kettle refining |
| PM10 | Calculation | Primary copper reverberatory furnace followed by converter |
| PM10 | Calculation | Primary copper multiple hearth roaster followed reverberatory furnace and converter |
| PM10 | Calculation | Primary copper fluid bed roaster followed by reverberatory furnace and converter |
| PM10 | Calculation | Primary copper concentrate dryer followed by electric furnace and converter |
| PM10 | Calculation | Primary copper fluid bed roaster followed by electric furnace and converter |
| PM10 | Calculation | Primary copper concentrate dryer followed by flash furnace, cleaning furnace and converter |
| PM10 | Calculation | Primary copper concentrate dryer followed by Noranda reactors and converter |
| PM10 | Calculation | Primary copper fugitive emissions |
| PM10 | Calculation | Secondary copper cupola |
| PM10 | Calculation | Secondary copper reverberatory furnace |
| PM10 | Calculation | Secondary copper crucible and pot furnace |
| PM10 | Calculation | Secondary copper electric arc furnace |
| PM10 | Calculation | Secondary copper electric induction |
| PM10 | Calculation | Secondary copper rotary furnace |
| Phenol | Calculation | Primary aluminium alumina reduction |
| SOx | Calculation | Combustion |
| SOx | Calculation | Flare |
| SOx | Calculation | Primary copper reverberatory furnace followed by Converter |
| SOx | Calculation | Primary copper multiple hearth roaster followed reverberatory furnace and converter |
| SOx | Calculation | Primary copper fluid bed roaster followed by reverberatory furnace and converter |
| SOx | Calculation | Primary copper concentrate dryer followed by electric furnace and converter |
| SOx | Calculation | Primary copper fluid bed roaster followed by electric furnace and converter |
| SOx | Calculation | Primary copper concentrate dryer followed by flash furnace, cleaning furnace and converter |
| SOx | Calculation | Primary copper concentrate dryer followed by Noranda reactors and converter |
| SOx | Calculation | Primary copper fugitive emissions |
| SOx | Calculation | Primary lead sinter machine |
| SOx | Calculation | Primary lead blast furnace |
| SOx | Calculation | Secondary lead blast smelting cupola |
| SOx | Calculation | Secondary lead reverberatory smelting |
| Toluene | Not provided | Not provided |
| Trichloroethylene | Not provided | Not provided |
| Xylene | Not provided | Not provided |
| Zinc | Measurement | Not provided |

PAH - From 2006 the SPRI required reporting of the annual emissions of listed individual PAH compounds (with individual reporting thresholds) rather than a PAH total.

### Emissions to air – Other metals

Table 23: Main water pollutants generated by non-ferrous metal and related activities

| Determinand | Method | Emission Source |
| --- | --- | --- |
| As | Measurement | Not provided |
| Cu | Measurement | Not provided |
| Cr | Measurement | Not provided |
| Mn | Measurement | Not provided |
| Nickel | Measurement | Not provided |
| Pb | Measurement/Calculation | Secondary copper reverberatory furnace |
| Pb | Calculation | Primary lead ore crushing |
| Pb | Calculation | Primary lead ore |
| Pb | Calculation | Primary lead tetrahedrite dryer |
| Pb | Calculation | Primary lead sinter machine |
| Pb | Calculation | Primary lead sinter building fugitives |
| Pb | Calculation | Primary lead blast furnace |
| Pb | Calculation | Secondary lead sweating |
| Pb | Calculation | Secondary lead reverberatory smelting |
| Pb | Calculation | Secondary lead blast smelting cupola |
| Pb | Calculation | Secondary lead kettle refining |
| Pb | Calculation | Secondary lead casting |
| Pb | Calculation | Secondary lead smelting |
| Pb | Calculation | Secondary lead kettle refining |
| Pb | Calculation | Secondary lead casting |
| Pb | Calculation | Secondary lead sweating |
| Sb | Measurement | Not provided |

### Releases to the water environment and transfers in wastewater

Table 24: Main water pollutants generated by non-ferrous metal and related activities

| Determinand | Method | Emissions Source | Factors and guidance |
| --- | --- | --- | --- |
| Cd | M | Effluent Treatment Facilities | Effluent quality data |
| Chlorides (as total Cl) | M | Effluent Treatment Facilities | Effluent quality data |
| Cyanides (as total CN) | M | Effluent Treatment Facilities | Effluent quality data |
| Fluorides (as total F) | M | Effluent Treatment Facilities | Effluent quality data |
| Hg | M | Effluent Treatment Facilities | Effluent quality data |
| NH3 | M | Effluent Treatment Facilities | Effluent quality data |
| PAHs (Benzo(a)pyrene, benzo(b)fluoranthene, benzo(k)fluoranthene, indeno(1, 2, 3-cd)pyrene) | M | Effluent Treatment Facilities | Effluent quality data |
| TOC | M | Effluent Treatment Facilities | Effluent quality data |
| Zinc | M | Effluent Treatment Facilities | Effluent quality data |

# 7 Combustion sector guidance

This Section covers activities that are regulated as “combustion activities” under the Environmental Authorisation (Scotland) Regulations (EASR). The Environmental Authorisation (Scotland) Regulations (EASR) revoke the Pollution Prevention and Control (Scotland) Regulations 2012 (PPC) and provides an integrated environmental approach to the regulation of certain industrial activities, which is now applied to schedules 20 and 26 emissions activities under EASR. This means emissions to air, water (including discharges to sewer) and land, plus a range of other environmental effects, must be considered together.

“Combustion activities” were formerly included within the scope as Part A prescribed/listed activities regulated under Chapter 1, Section 1.1 of the PPC Regulations. They are now found in “energy industries” in EASR schedule 20, part 4, chapter 1, paragraph 1.

Schedule 21 of EASR “operating large combustion plant” may also be applicable to your activity. Please see the SEPA webpage for “combustion of fuels”.

## 7.1 Air pollutants

The main air emissions from the combustion of fossil fuels will almost always lead to releases to air of CO2, CO, NOx and particulate matter. Other pollutants will be released dependent on the fuel and combustion technology used, particularly SO2. Smaller quantities of pollutants such as trace metals, hydrogen halides, un-burnt hydrocarbons, non-methane volatile organic compounds (NMVOCs) and dioxins may also be emitted, but they may have a significant influence on the environment due to their toxicity or their persistence.

Pyrolysis and gasification can be used to pre-treat fuels in order to remove impurities or to produce fuel that can be combusted more readily. The fuels can be either pre-cleaned before combustion or directly combusted. The pollutants emitted from the process are therefore critically dependent on the fuel used and the nature of the process.

The table on the next page illustrates the likely pollutants emitted from different types of fuel. The table should be taken as a guide only, and you should verify that there are no other pollutants emitted from your process.

Table 25: Guide of main pollutants emitted by combustion activities to air

| Fuel type | Inputs | Potential air emissions | Further information |
| --- | --- | --- | --- |
| Solid | Coal | NOx, CO, CO2, SOx, particulate matter (including PM10 and 2.5), fugitive dust, trace metals, PCBs and PAHs, hydrogen halides, methane, NMVOCs, dioxins, N2O | None |
| Solid | Biomass | NOx, CO, CO2, SOx (low), particulate matter (including PM10 and 2.5), CH4, NMVOCs, trace metals (from sewage sludge). | The percentage proportion of the total CO2 attributable to biomass (non-fossil fuel) can be recorded within Section C of the SPRI reporting form, within the carbon dioxide substance. This is the percentage of the total CO2 emissions attributable to biomass fuels. Where you record “BRT” you can still enter a percentage figure within the “Biomass Percentage” box. If you calculate you are above the reporting threshold enter the total CO2 figure which should include both fossil fuel emissions as well as biomass-derived emissions. |
| Liquid | Fuel oil | NOx, CO, CO2, SOx, particulate matter (including PM10 and 2.5), PCBs and PAHs, hydrogen chloride, trace metals, and dioxins. | None |
| Gaseous | Natural gas | NOx, CO, CO2, CH4. | None |
| Secondary fuels | Solid, liquid or gaseous | NOx, CO, CO2, SOx, particulate matter (including PM10 and 2.5), PCBs and PAHs, hydrogen halides, trace metals, NMVOCs, H2S, ammonia and dioxins. | None |

Note: Subsidiary or substitute fuels may contain other pollutants that will require reporting.

### 7.1.1 Point source emissions

These emissions are exhausted via a stack or vent; i.e. a single point source into the atmosphere. Abatement equipment; e.g. an electrostatic precipitator (ESP) or fabric filter (bag house) can be incorporated into the exhaust system prior to discharge to atmosphere. Point source emissions will be the most significant emission source for combustion activities.

### 7.1.2 Fugitive emissions

Fugitive emissions are those that are not released from a point source such as a stack. Examples of fugitive emissions include dust from coal and ash stockpiles and entrainment of pollutants during material handling. Leaks from valves and flanges are also examples of fugitive emissions. With appropriate management and control, these emission sources are generally minor for the combustion sector. Only fugitive emissions that leave the site need to be reported to the SPRI. Whilst contained spills would therefore not need to be reported, you should report vapour emissions that may have dispersed.

### 7.1.3 Quantification of emissions

You should refer to the SEPA SPRI operator guidance: Release Estimation Techniques (RET) (SPRI-G-003) for information on quantification of emissions.

Using continuous emission monitoring systems (CEMS)

For the Electricity Supply Industry (ESI), it may not be possible to obtain valid CEMS data during start-up and shutdown. In these circumstances, for the purposes of reporting to the SPRI, an alternative approach, for example using emission factors, can be used by agreement.

### 7.1.4 General emission factors

You should refer to the SEPA SPRI operator guidance: Release Estimation Techniques (RET) (SPRI-G-003)

### 7.1.5 Carbon dioxide emission factors

ETS guidance should be used for CO2 reporting. Estimation of CO2 emissions from combustion sources is based on the following formula.

Equation 4

ECO2 = Ae x EFCO2 x OFCO2

Where:

ECO2 = Emission rate of CO2 (tonnes/year)

Ae = Activity rate (annual energy consumption) (TJ/yr)

EFCO2 = Emission factor of CO2 (teCO2/TJ)

OFCO2 = Oxidation factor

The oxidation factor takes into account the fact that when energy is consumed not all of the carbon in the fuel oxidises to CO2. The oxidation factor expresses the proportion of carbon in the fuel that is oxidised to CO2.

The ETS provides specific guidance regarding the determination of activity-specific emission factors for defined fuels. In the absence of activity-specific factors, general emission factors for the combustion of various fossil fuels are provided in the ETS guidance.

By contrast to combustion sources of CO2 emissions, the ETS allows process emissions (emissions of CO2 from non-combustion activities) to be derived from activity data based on raw material inputs, throughputs and outputs. The basic calculation equation remains multiplication of activity data by appropriate emission factors, but use of conversion factors (CFs) in place of oxidation factors.

Biomass is considered to be CO2 neutral in terms of the ETS, but the mass of CO2 emitted from its combustion or use is an SPRI reporting requirement and should at least match or be greater than your reported ETS figures. The proportion of the total CO2 attributable to biomass should be recorded within the Method Description or Qualification box and the Biomass Percentage recorded within the SPRI reporting form.

Under the ETS Guidelines, operators can choose the most appropriate oxidation factor tier, shown below:

#### List 10 - CO2 oxidation factor tiers

Tier 1 = An oxidation of 1 is used.

Tier 2 = The operator applies oxidation factors for the respective fuel as reported in the UK Greenhouse Gas Inventory (UKGHGI) (country-specific factors).

Tier 3 = For fuels activity-specific factors are derived by the operator using approved methodologies.

In some cases the emission factor may already take into account the proportion of carbon in the fuel that is oxidised and you should verify if this is the case. If so, the oxidation factor in Equation 4 is equal to 1.

It is also possible to use an emission factor based on the amount of fuel used, in essence Equation 4 from the RET Guidance (SPRI-G-003). This is due to the fact that the annual energy consumption is related to annual fuel consumption by the net thermal capacity of the fuel.

The ETS allows CO2 emissions to be determined by two methodologies; a calculation methodology or a measurement methodology. However, where a measurement methodology is used, a requirement exists to verify the measured emissions by calculation. In both cases however, it is likely that measurements of fuel burned or fuel flow will be available and these can be used for the determination of other emission quantities when combined with appropriate emission factors.

### 7.1.6 Solid fuel combustion factors

(a) Electricity supply industry (ESI)

The emission factors contained in this Section (and in 7.1.7(a) and 7.1.8(a)) represent currently agreed factors for electricity generating stations. They have been developed from measurements on large electricity generating stations (greater than 300 MWth) and are regularly updated. Further detail on these emission factors is contained in the ESI methodology.

Also included in Table 26 are estimates of the amount of ‘as received’ coal that would need to be burnt in a year in order to exceed the reporting thresholds. However, a value for the tonnes of coal that would need to be burnt for SO3 has not been provided, as it will be reported with SO2 (which is either normally estimated from CEMS or fuel analyses in accordance with Section 2 of the RET Guidance (SPRI-G-003)). ESI plant burning less than the indicated tonnage of coal should therefore report emissions as ‘BRT’ for that particular pollutant, provided that there are no other sources of the pollutant on site.

Emissions of NOx from ESI coal-fired power plants that do not use CEMS for annual mass emissions can be calculated from the fuel burn and NOx factors which have been agreed with SPRI on a station-specific basis.

PM concentrations are normally measured by CEMS for ESI plant. The annual mass emission should be calculated from the instantaneous measured concentration multiplied by the instantaneous volumetric emission, as in Equation 1 of the RET Guidance (SPRI-G-003). Where load and abatement performance is constant, the average concentration and flow rate can be used (Equation 2 of the RET Guidance) (SPRI-G-003). For ESI plant it is generally assumed that each tonne of coal generates 9,000 m3 of flue gas normalised at 6% oxygen concentration.

The SPRI return requires the Total, PM10 and PM2.5 fractions of the particulate matter to be reported. For ESI coal-fired plant, the PM10 and PM2.5 fractions are assumed to be 80% and 40% respectively of the total particulate matter emitted where ESP is used. For plant with FGD the PM10 and PM2.5 fractions are assumed to be 90% and 80% respectively of the total particulate matter emitted. You can also use these factors when burning peat or biomass.

Emissions of hydrogen chloride, “chlorine and inorganic compounds – as HCl” and “fluorine and inorganic compounds – as HF” are normally estimated from fuel analysis, as shown in Section 2 of the RET Guidance (SPRI-G-003).

Table 26: UK ESI PI emission factors for large combustion plant 2013

| Pollutant species | Emission factor (kg/tonne ‘as received’ coal burnt) for pulverised coal plant with or without FGD | [te coal burnt for above reporting threshold] | Notes |
| --- | --- | --- | --- |
| Dioxins and furans (expressed as I- TEQ) | 1.5 x 10-11 | [666,667] | None |
| Dioxins and furans (expressed as WHO– TEQ) | 1.6 x 10-11 | [625,000] | None |
| PCB (expressed as WHO–TEQ) | 1.8 x 10-10 | [55,556] | None |
| PCB (expressed as total mass) | 9.9 x 10-8 | [1,010,101] | None |
| Benzo(a)pyrene (BaP) | 9.0 x 10-7 | [1,111,111] | None |
| Anthracene | 2.7 x 10-7 | [BRT] | These emissions should always be reported as below the reporting threshold given the required fuel burn. |
| Benzo(b)fluoranthene | 5.4 x 10-7 | [1,851,852] | None |
| Benzo(g,h,i)perylene | 5.4 x 10-7 | [1,851,852] | None |
| Benzo(k)fluoranthene | 5.4 x 10-7 | [1,851,852] | None |
| Chrysene | 3.6 x 10-7 | [BRT] | These emissions should always be reported as below the reporting threshold given the required fuel burn. |
| Fluoranthene | 4.5 x 10-7 | [2,222,222] | None |
| Indeno(1,2,3-cd)pyrene | 5.4 x 10-7 | [1,851,852] | None |
| Naphthalene | 4.9 x 10-5 | [2,040,816] | None |
| Methane | 1.4 x 10-2 | [714,286] | None |
| NMVOCs | 2.7 x 10-2 | [370,370] | None |
| Carbon monoxide | 1.1 | [90,909] | None |
| SO3 (reported as part of sulphur oxides – SO2 and SO3 as SO2) | 6.3 x 10-2 | - | None |
| Nitrous oxide | 2.6 x 10-2 | [384,615] | None |

(b) Generic factors

In the absence of other information, generic emission factors based on currently achievable emission rates from various combustion plants can be used. For this Section, 7.1.7(b) and 7.1.8(b) the generic emission factors have been taken from existing UK regulatory guidance for combustion appliances and supplemented with Local Authority Guidance (listed in the useful references Section).

The existing UK regulatory guidance contains achievable emission data post abatement in terms of mg/MJ of net thermal input together with the equivalent normalised emission concentrations (mg/m3).

If you know the normalised emission concentration of your combustion appliance, you can calculate the equivalent emission factor in terms of mg/MJ and use Equation 5 below. The relevant mg/MJ and mg/m3 values from the guidance are listed in Table 27, Table 29, and   Table 31 for each fuel category. The generic emission factors (EFi) for coal-firing are presented in Table 27 below and are expressed in terms of the net heat input into the combustion appliance.

Equation 5

E = Ae x EF x 10-6

Where:

E = Emission of pollutant (kg/year)

Ae = Annual energy consumption (MJ/yr)

EF= Emission factor of pollutant (teCO2/TJ)

The emission factors also enable an estimate to be made of the size of combustion plant that would need to be operated for the emission to be above the reporting threshold values. These estimates are also included in Table 27 in terms of the average MWth net thermal input of the plant, assuming that the combustion appliance operates for 100% of the year. If the combustion plant operates for less than 100% of the year, the calculated MWth threshold can simply be divided by the percentage operating time to provide the appropriate MWth threshold. For single combustion appliances that are rated less than the indicated MWth threshold, you can make a BRT return. If you have more than one combustion appliance on site, your aggregate emission may be above the threshold value.

###### Table 27: Solid fuel-firing generic emission factors (net basis)

| Technology | Emission factor | Particulate matter (mg/MJ) | Particulate matter (mg/m3) | NOx (mg/MJ) | NOx (mg/m3) | CO (mg/MJ) | CO (mg/m3) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stoker boiler, in-furnace desulphurisation | EF | 9 | 25 | 105 | 300 | 50 | 150 | Use HMSO. Process Guidance Note S2 1.01 Combustion processes: large boilers and furnaces 50MW(th) and over, November 1995 ISBN 0-11-753206-1. |
| Stoker boiler, in-furnace desulphurisation | Threshold BRT (Wth) | 35 | 35 | 30 | 30 | 63 | 63 | Use HMSO. Process Guidance Note S2 1.01 Combustion processes: large boilers and furnaces 50MW(th) and over, November 1995 ISBN 0-11-753206-1. |
| CFBC, in-bed desulphurisation | EF | 9 | 25 | 70 | 200 | 50 | 150 | Use HMSO. Process Guidance Note S2 1.01 Combustion processes: large boilers and furnaces 50MW(th) and over, November 1995 ISBN 0-11-753206-1. |
| CFBC, in-bed desulphurisation | Threshold for BRT (MWth) | 35 | 35 | 45 | 45 | 63 | 63 | Use HMSO. Process Guidance Note S2 1.01 Combustion processes: large boilers and furnaces 50MW(th) and over, November 1995 ISBN 0-11-753206-1. |
| PFBC, in-bed desulphurisation and SNCR | EF | 9 | 25 | 21 | 60 | 10 | 30 | Use HMSO. Process Guidance Note S2 1.01 Combustion processes: large boilers and furnaces 50MW(th) and over, November 1995 ISBN 0-11-753206-1. |
| PFBC, in-bed desulphurisation and SNCR | Threshold for BRT (MWth) | 35 | 35 | 151 | 151 | 315 | 315 | Use HMSO. Process Guidance Note S2 1.01 Combustion processes: large boilers and furnaces 50MW(th) and over, November 1995 ISBN 0-11-753206-1. |
| PF boiler, dry lime injection, low-NOx burners | EF | 9 | 25 | 225 | 650 | 35 | 100 | Use HMSO. Process Guidance Note S2 1.03 Combustion processes: gas turbines, September 1995 ISBN 0-11-753166-9. |
| PF boiler, dry lime injection, low-NOx burners | Threshold for BRT (MWth) | 35 | 35 | 14 | 14 | 91 | 91 | Use HMSO. Process Guidance Note S2 1.03 Combustion processes: gas turbines, September 1995 ISBN 0-11-753166-9. |
| PF boiler, wet limestone scrubbing, low-NOx burners and re-burn | EF | 5 | 15 | 87 | 250 | 35 | 100 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11-310183. |
| PF boiler, wet limestone scrubbing, low-NOx burners and re-burn | Threshold for BRT (MWth) | 63 | 63 | 36 | 36 | 91 | 91 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11-310183. |
| PF boiler, wet limestone scrubbing, low-NOx burners and SCR | EF | 5 | 15 | 70 | 200 | 35 | 100 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11-310183. |
| PF boiler, wet limestone scrubbing, low-NOx burners and SCR | Threshold for BRT (MWth) | 63 | 63 | 45 | 45 | 91 | 91 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11-310183. |
| Boiler, 20-50MWth, stoker firing | EF | 108 | 300 | 160 | 450 | 50 | 150 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Boiler, 20-50MWth, stoker firing | Threshold for BRT (MWth) | 2 | 2 | 20 | 20 | 63 | 63 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Boiler, 20-50MWth, other firing | EF | 108 | 300 | 225 | 650 | 50 | 150 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Boiler, 20-50MWth, other firing | Threshold for BRT (MWth) | 2 | 2 | 14 | 14 | 63 | 63 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |

For coal-fired plants incorporating Selective Catalytic Reduction (SCR), an emission factor of 4 mg/MJ (10 mg/m3) for ammonia can be used.

Plant employing Selective Non-catalytic Reduction (SNCR) could also release ammonia and nitrous oxide. In the absence of other information, emission factors for SNCR for ammonia and nitrous oxide releases can be assumed to be 2 mg/MJ (5 mg/m3) and 21 mg/MJ (60 mg/m3) respectively.

For unabated emissions of coal, peat or biomass-powered plant the PM10 fraction can be assumed to be 40% of the total particulate matter emitted, rising to 80% for plant with ESPs or bag filters and dry FGD and 95% for plant with ESPs or bag filters and wet FGD.

### 7.1.7 Emission factors for liquid fuel combustion

(a) Electricity supply industry (ESI)

In a similar manner to Section 7.1.6(a), the UK ESI emission factors are listed in Table 28 for heavy fuel oil-fired (HFO) plant. Further detail on these emission factors is contained in the ESI methodology.

Also included in Table 28 are estimates of the amount of oil that would need to be burnt in order to exceed the reporting thresholds given the listed emission factors. However, a value for the tonnes of oil that would need to be burnt for SO3 has not been provided, as it will be reported with SO2. ESI plant burning less than the indicated tonnage of oil should therefore report emissions as BRT for that particular pollutant provided that there are no other sources of the pollutant on site.

###### Table 28: UK ESI PI emission factors for large combustion plant 2013

| Pollutant species | Emission factor (kg/tonne oil burnt) for heavy oil-fired plant | [te oil burnt for above the reporting threshold] | Notes |
| --- | --- | --- | --- |
| Dioxins and furans (expressed as I-TEQ) | 2.1 x 10-11 | [497,512] | None |
| Dioxins and furans (expressed as WHO– TEQ) | 2.1 x 10-11 | [497,512] | None |
| PCB (expressed as WHO–TEQ) | 2.4 x 10-10 | [41,667] | None |
| PCB (expressed as total mass) | 1.3 x 10-7 | [769,231] | None |
| Benzo(a)pyrene (BaP) | 1.2 x 10-6 | [833,333] | None |
| Anthracene | 3.6 x 10-7 | [BRT] | These emissions should always be reported as below the reporting threshold given the required fuel burn. |
| Benzo(b)fluoranthene | 7.2 x 10-7 | [1,388,889] | None |
| Benzo(g,h,i)perylene | 7.2 x 10-7 | [1,388,889] | None |
| Benzo(k)fluoranthene | 7.2 x 10-7 | [1,388,889] | None |
| Chrysene | 4.8 x 10-7 | [BRT] | These emissions should always be reported as below the reporting threshold given the required fuel burn. |
| Fluoranthene | 6.0 x 10-7 | [1,666,667] | None |
| Indeno(1,2,3-cd)pyrene | 7.2 x 10-7 | [1,388,889] | None |
| Naphthalene | 6.5 x 10-5 | [1,538,462] | None |
| Methane | 1.8 x 10-2 | [555,556] | None |
| Non-methane volatile organic compounds | 3.6 x 10-2 | [277,778] | None |
| Carbon monoxide | 1.5 | [6,667] | None |
| SO3 (reported as part of sulphur oxides - SO2 and SO3 as SO2) | 4.2 x 10-2 1.08 | [-] [-] | For plant with Mg (OH) 2 flue gas conditioning. For plant without Mg (OH) 2 flue gas conditioning. |
| Nitrous oxide | 3.5 x 10-2 | [285,714] | None |

Emissions of NOx from ESI oil-fired power plants that do not use CEMS for determining annual mass emission can be calculated from the fuel burn and NOx factors that have been agreed with SPRI on a station-specific basis.

PM concentrations are normally measured by CEMS for ESI plant. The annual mass emission should be calculated from the instantaneous measured concentration multiplied by the instantaneous volumetric emission, as in Equation 1 of the RET Guidance (SPRI-G-003). Where load is constant, the average concentration and flow rate can be used (Equation 2 in the RET Guidance (SPRI-G-003)). For ESI plant it is generally assumed that each tonne of oil generates 12,000m3 of flue gas normalised at 3% oxygen concentration.

The SPRI return requires the Total, PM10 and PM2.5 fractions of the particulate matter to be reported. For ESI oil-fired plant, the PM10 and PM2.5.5 fractions are assumed to be 71% and 52% respectively of the Total particulate matter emitted.

(b) Generic factors

In a similar manner to Section 7.1.6(b), and based on the same referenced information sources, emission factors based on achievable emission rates from various liquid-fired combustion plant can be used in Equation 5. Relevant emission factors for liquid-fired plant are given in Table 29 (in terms of net heat input), and the same methodology should be used as that outlined in Section 7.1.6(b).

###### Table 29: Liquid fuel-firing generic emission factors (net basis)

| Technology | Emission factor | Particulate matter (mg/MJ) | Particulate matter (mg/m3) | NOx (mg/MJ) | NOx (mg/m3) | CO (mg/MJ) | CO (mg/m3) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gas turbine (after 1994) | EF | 0 | 0 | 105 | 125 | 50 | 60 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Gas turbine (after 1994) | Threshold for BRT (MWth) | - | - | 30 | 30 | 63 | 63 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Gas turbine (pre-1994) | EF | 0 | 0 | 140 | 165 | 84 | 100 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Gas turbine (pre-1994) | Threshold for BRT (MWth) | - | - | 23 | 23 | 38 | 38 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Boilers, 20-50MW, L/M/H fuel oil–firing | EF | 42 | 150 | 125 | 450 | 42 | 150 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Boilers, 20-50MW, L/M/H fuel oil–firing | Threshold for BRT (MWth) | 8 | 8 | 25 | 25 | 75 | 75 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Boilers, 20-50MW, distillate firing | EF | 28 | 100 | 55 | 200 | 42 | 150 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Boilers, 20-50MW, distillate firing | Threshold for BRT (MWth) | 12 | 12 | 58 | 58 | 75 | 75 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Compression ignition engine, SCR | EF | 40 | 50 | 125 | 150 | 125 | 150 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |
| Compression ignition engine, SCR | Threshold for BRT (MWth) | 8 | 8 | 25 | 25 | 25 | 25 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |

For liquid fuel-fired plants incorporating SCR, an emission factor of 3 mg/MJ (10 mg/m3) for ammonia can be used.

For unabated emissions, the PM10 fraction can be assumed to be 45% of the Total particulate matter emitted, rising to 80% for plant with ESPs or bag filters and dry FGD and 90% for plant with ESPs or bag filters and wet FGD.

### 7.1.8 Emission factors for natural gas combustion

(a) Electricity supply industry (ESI)

In a similar manner to Section 7.1.6(a), the UK ESI emission factors are listed in Table 30 for natural gas-fired plant. Further detail on these emission factors is contained in the ESI methodology.

Also included in Table 30 are the approximate GJ of energy that would be required to exceed the reporting thresholds given the listed emission factors and burning only natural gas.

###### Table 30: 	2013 UK ESI PI emission factors for large combustion plant

| Pollutant species | Emission factor (g/GJ gas burnt) for gas firing (based on GCV) | [GJ of energy giving rise to numerical reported value] |
| --- | --- | --- |
| Dioxins and furans (expressed as I-TEQ) | 0 | [-] |
| Dioxins and furans (expressed as WHO-TEQ) | 0 | [-] |
| PCBs (expressed as WHO-TEQ) | 0 | [-] |
| Benzo(a)pyrene (BaP) | 0 | [-] |
| Anthracene | 0 | [-] |
| Benzo(b)fluoranthene | 0 | [-] |
| Benzo(g,h,i)perylene | 0 | [-] |
| Benzo(k)fluoranthene | 0 | [-] |
| Chrysene | 0 | [-] |
| Fluoranthene | 0 | [-] |
| Indeno(1,2,3-cd)pyrene | 0 | [-] |
| Naphthalene | 0 | [-] |
| Methane | 3.7 | [2,702,700] |
| Non-methane volatile organic compounds | 0.9 | [11,111,000] |
| Carbon monoxide | 13 | [7,692,300] |
| SO3 (reported as part of sulphur oxides – SO2 and SO3 as SO2) | 0 | [-] |
| Nitrous oxide | 0.5 | [20,000,000] |

NOx emissions from CCGT and gas-fired plant are generally calculated from continuous monitors.

For the combustion of natural gas, the ESI PI methodology advises a revised emission factor of 0 for particulate matter from gas turbine plant.

For gas turbines running on distillate fuel, for example during start-up, the emission factors given in Table 30 can be used, and 100% of the Total particulate matter emission are assumed to be PM10 and 50% PM2.5. Note that from 2009, the ESI proposes that they will assume that 100% of Total particulate matter is PM2.5, as a conservative estimate.

(b) Generic factors

In a similar manner to Section 7.1.6(b), and based on the same referenced information sources, emission factors based on achievable emission rates from various natural gas-fired combustion plant can be used in Equation 5. Relevant emission factors for natural gas-fired plant are given in Table 31 (in terms of net heat input), and the same methodology should be used as that outlined in Section 7.1.6 (b). Note that the emission factors are based on net calorific values (NCV).

###### Table 31: Natural gas fuel-firing generic emission factors (net basis)

| Technology | Emission factor | Particulate matter (mg/MJ) | Particulate matter (mg/m3) | NOx (mg/MJ) | NOx (mg/m3) | CO (mg/MJ) | CO (mg/m3) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gas turbine, after 1994 | EFi (mg/MJ) | 0 | 0 | 50 | 60 | 50 | 60 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Gas turbine, after 1994 | Threshold for BRT (MWth) | - | - | 63 | 63 | 63 | 63 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Gas turbine, pre-1994 | EFi (mg/MJ) | 0 | 0 | 105 | 125 | 84 | 100 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Gas turbine, pre-1994 | Threshold for BRT (MWth) | - | - | 30 | 30 | 38 | 38 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Boilers, 20-50MW, Flue gas recirculation (FGR) | EFi (mg/MJ) | 1 | 5 | 39 | 140 | 30 | 100 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Boilers, 20-50MW, Flue gas recirculation (FGR) | Threshold for BRT (MWth) | 317 | 317 | 81 | 81 | 106 | 106 | Use Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth. |
| Dual fuel compression ignition engine, SCR | EFi (mg/MJ) | 15 | 20 | 85 | 100 | 375 | 450 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |
| Dual fuel compression ignition engine, SCR | Threshold for BRT (MWth) | 21 | 21 | 37 | 37 | 9 | 9 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |
| Dual fuel compression ignition engine, lean burn | EFi (mg/MJ) | 15 | 20 | 125 | 150 | 125 | 150 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |
| Dual fuel compression ignition engine, lean burn | Threshold for BRT (MWth) | 21 | 21 | 25 | 25 | 25 | 25 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |
| Spark ignition engine, SCR | EFi (mg/MJ) | 0 | 0 | 85 | 100 | 125 | 150 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |
| Spark ignition engine, SCR | Threshold for BRT (MWth) | - | - | 37 | 37 | 25 | 25 | Use HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0. |
| Spark ignition engine, lean burn, exhaust gas recirculation (EGR) | EFi (mg/MJ) | 0 | 0 | 125 | 150 | 125 | 150 | None |
| Spark ignition engine, lean burn, exhaust gas recirculation (EGR) | Threshold for BRT (MWth) | - | - | 25 | 25 | 25 | 25 | Use HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11310183. |

For both compression ignition engines and spark ignition engines, a factor of 170 mg/MJ (200 mg/m3) can be used for NMVOCs. For natural gas-fired plants incorporating SCR, an emission factor of 8 mg/MJ (10 mg/m3) for ammonia can be used.

Apart from mercury, trace metallic elements in natural gas are assumed to be zero. For mercury, an emission factor of 1 x 10-4 mg/m3 of gas burnt can be used to calculate mercury vapour emissions. In this case, greater than 1 x 1010 m3 of gas would need to be burnt in a year to exceed the reporting threshold.

The PM10 and PM2.5 fractions can be assumed to be 100% of the total particulate matter emitted when burning natural gas.

### 7.1.9 Fuel analysis data

(a) General

The use of fuel analysis data to determine emissions is similar to the use of emission factors.

The basic equation used in fuel analysis emission calculations is:

Equation 6

E = Qf x [Op hours] x [PCf/100] x (MWp / EWf)

Where:

E 	=	Emission of pollutant (kg/yr)

Qf 	=	Fuel use (kg/hr)

PCf 	=	Pollutant concentration in the fuel (%)

Op hours =	Operating hours per year (hr/yr)

MWp 	=	Molecular weight of pollutant as emitted after combustion

EWf 	=	Elemental weight of pollutant as present in fuel

Equation 6 is the method usually used for calculating SO2 emissions where it is normally assumed that all of the sulphur in the fuel is converted to SO2. However, when using the equation for coal-fired plant, it is assumed that 5% of the sulphur is retained in the ash.

Where the pollutant concentration in the fuel is consistent over the averaging period (i.e. one year), Equation 6 can be written as:

E = M x [PCf/100] x (MWp / EWf)

Where:

E 	=	Emission of pollutant (kg/yr)

M 	=	Mass of fuel burnt in one year (kg/yr)

PCf 	=	Pollutant concentration in the fuel (%)

MWp 	=	Molecular weight of pollutant as emitted after combustion

EWf 	=	Elemental weight of pollutant as present in fuel

###### Example 6

This example below shows how SO2 emissions can be calculated from oil combustion, based on fuel analysis results and fuel flow information. It is assumed that the facility operates using oil for 150 hours per year and that abatement of SO2 does not occur.

| Qf = | 2000 kg/yr |
| --- | --- |
| PCf = | 1.17% |
| MWp = | 64 |
| EWf = | 32 |
| Op Hours = | 1500 hr/yr |
| E = Qf x PCf x (MWp/EWf) x [Op Hours] |  |
| E = [(2000) x (1.17/100) x (64/32) x 150] kg/yr |  |
| E = 7.02 x 103 kg/yr |  |

Equation 6 can also be used for volatile elements such as fluorine and chlorine as well as trace metallic pollutants, although some of these species are retained in the plant, either in the ash or in abatement equipment (see below). When using Equation 6, you should be aware that the amount of pollutants present in the fuel can vary significantly.

For UK ESI plant, the trace element concentration in the coal is calculated as a yearly weighted mean average for each plant, based on the delivered tonnage of coal. Where coals have not been analysed for trace element content, then you should use an average value for coals from a similar geographic region.

###### (b) 	Solid fuel analysis

For elements that are effectively captured in either bottom ash or fly ash, Equation 6 can result in the overestimation of emissions. In addition, emission quantities of volatile and semi-volatile components will greatly depend on the emission temperature and abatement collection efficiency as volatile and semi-volatile substances can condense on fine particulate matter. Increasing the emission temperature may significantly increase the pollutant release rate of volatile components. Any changes to the process conditions that may affect pollutant partitioning or capture should be taken into account in emission calculations by the determination of plant-specific retention and enrichment factors (see below).

Mass emissions of trace metallic elements from coal combustion can be calculated indirectly from the amount of total particulate matter emitted, corrected by factors representing the concentration of the trace element in the coal and how much of the element is chemically present in the ash, using the following equations.

The total emission is the sum of the emission from the non-volatile and volatile phases:

Equation 7

Non-volatile phase:

Env = PCcoal x (100 / AA) x F x R x PM

Equation 8

Volatile phase:

Ev = PCcoal x (1 – F) x CB

Where:

Env 	=	Non-volatile pollutant emission (kg/yr)

Ev 	=	Volatile pollutant emission (kg/yr)

PCcoal 	=	Trace element weight fraction in the coal (-)

AA 	=	Average ash mass percentage in coal (%)

F	=	Retention factor in ash (-)

R	=	Enrichment factor (-)

PM 	=	Particulate matter emission (kg/yr)

CB	=	Coal burned (kg/yr)

Common enrichment and retention factors that are used across UK coal-fired ESI plant are given in Table 32. These factors are appropriate for exhaust gas temperatures of less than approximately 130oC. For higher exhaust gas temperatures the factors may differ, especially for the more volatile elements, and you should therefore use site-specific factors.

The use of flue gas desulphurisation (FGD) will lead to the retention of sulphur from the emission, but it will also lead to the retention of soluble acid halides such as HCl and HF. In the absence of site specific data for FGD, retention factors for HCl and HF should be taken as 98% and 72% respectively. The additional factors given in Table 32 for FGD are for wet limestone/gypsum systems.

###### Table 32: 2013 UK ESI PI coal combustion factors

| Element | Retention factor (F) | Enrichment factor (R) | FGD retention factor (vapour) |
| --- | --- | --- | --- |
| Arsenic | 1 | 3.4 | - |
| Selenium | 0.8 | 9.0 | 0.65 |
| Lead | 1 | 2.9 | - |
| Antimony | 1 | 3.6 | - |
| Nickel | 1 | 1.9 | - |
| Chromium (total) | 1 | 1.7 | - |
| Copper | 1 | 2.0 | - |
| Manganese | 1 | 1.7 | - |
| Vanadium | 1 | 1.4 | - |
| Zinc | 1 | 4 | - |
| Cadmium | 1 | 4.5 | - |
| Mercury | 0.5 | 4.0 | 0.5 |

In order to use Equations 7 and 8, it is necessary to know the composition of the coal in terms of the percentage of trace metallic elements present. As coal composition can vary significantly depending on the source of the fuel, you should obtain information on the trace elements from the supplier, or have specific coal analysis carried out.

Fuel composition should also be taken into account in determining pollutant emissions from subsidiary or substitute fuels. In the absence of other data, for fuels such as biomass that have a low ash content, the enrichment and retention factors given above can be used.

###### (c) 	Liquid fuel analysis

Equation 8 can be used to calculate trace metallic emissions from oil-fired plant. As a generality within the ESI methodology it is assumed that for large, heavy fuel oil plant with particulate or grit arrestors a retention factor of 0.75 can be used for trace elements. Appropriate retention factors should be determined for other abatement plant. In the absence of abatement, the retention factor would be zero.

As with coal, information on the composition of the trace elements in the fuel should be obtained from the supplier or be measured.

Emissions of halogens from oil-fired plant can be assumed to be zero.

###### (d) 	Natural gas analysis

As indicated in Section 7.3.5(b), apart from mercury, natural gas supplies in the UK are currently assumed to have zero trace metal content. Emissions of halogens from natural gas-fired plant are also assumed to be zero.

Where no specific gas analysis is available and in order to calculate emissions from gas venting, natural gas should be assumed to comprise 1% CO2, 1% nitrogen, 92% CH4 and 6% NMVOCs.

### 7.1.10 Fugitive emissions

For solid fuel plant, particulate matter emissions can occur by dust blown from coal stocks and ash storage areas. However, these can be assumed to be low in comparison to stack emissions unless specific events have occurred which are known to have released significant quantities of material offsite.

Techniques for estimating fugitive emissions from the surface of stockpiles are limited. Options include measuring ambient dust levels upwind and downwind of the source of interest and/or applying predictive mathematical models.

Methane (CH4) and other hydrocarbon emissions from coal stocks and oil tank filling can be assumed to be small in relation to emissions through the stacks. For natural gas-fired plant, where gas is vented to atmosphere for operational and maintenance purposes, the mass emissions of CO2, CH4 and NMVOCs should be calculated from the gas composition.

## 7.2 	Emissions to water and wastewater

### 7.2.1 Relevant pollutants

A variety of substances need to be considered when reporting emissions to water or transfers in wastewater to sewer. The main ones are illustrated in the list below. The list should be taken as a guide only, and you should verify that there are no other pollutants emitted from the process. The significance of each parameter depends on the specific plant configuration and the process applied, which also determines the type and amount of pollutant present in the wastewater prior to treatment.

#### List 11 - Guide on main reportable substances likely to be emitted to water

- Arsenic

- Zinc

- Cadmium

- Iron

- Chromium

- Ammonia (total)

- Copper

- Nitrogen (total as N)

- Lead

- Phosphorus (total as P)

- Mercury

- Chlorides

- Nickel

- Fluorides

### 7.2.2 Emission sources

Emissions to water generally arise from the following sources:

- Cooling water systems.

- Demineralised water treatment plant.

- Boiler blowdown.

- FGD wastewater treatment plant.

- Ash transport wastewater.

- Surface water run-off from storage areas (e.g. fuel, ash, FGD material).

- Cleaning water.

Notwithstanding the above, you should consider all emission sources to water and characterise the flows and emission concentrations from each source.

## 7.3 Off-site waste transfers

### 7.3.1 Relevant wastes

Typical wastes and by-products generated in this sector are:

- Bottom ash and/or boiler slag.

- Fly-ash.

- Fluidised bed ash.

- Flue gas desulphurisation residues and by-products.

- Special wastes (e.g. solvents).

- Metallic wastes.

- Chemical wastes.

- Waste oils.

- General waste.

#### Transboundary shipments of hazardous waste

For transboundary movements of hazardous waste (outwith the United Kingdom), the name and address of the recoverer or the disposer of the waste and the actual recovery or disposal site have to be reported.

## 7.4 	Useful references

HMSO. Process Guidance IPC S3 1.01 Combustion processes Supplementary Guidance Note, November 1995 ISBN 0-11-310183.

HMSO. Process Guidance Note S2 1.01 Combustion processes: large boilers and furnaces 50MW(th) and over, November 1995 ISBN 0-11-753206-1.

HMSO. Process Guidance Note S2 1.03 Combustion processes: gas turbines, September 1995 ISBN 0-11-753166-9.

HMSO. Process Guidance Note IPR1/2 Combustion Processes: gas turbines, September 1994, ISBN 0-11-752954-0.

Local Authority Unit. Assessment for second revision of PG1/3, Process Guidance Note for Part B. Boilers and furnaces with a net rated thermal input of 20 – 50 MWth.

Local Authority Unit. Assessment for second revision of PG1/4, Process Guidance Note for Part B. Gas turbines 20 – 50 MWth rated thermal input.

# 8 	Chemical sector guidance

This Section covers activities that are regulated as “chemical industry activities” under the Environmental Authorisation (Scotland) Regulations (EASR). The Environmental Authorisation (Scotland) Regulations (EASR) revoke the Pollution Prevention and Control (Scotland) Regulations 2012 (PPC) and provides an integrated environmental approach to the regulation of certain industrial activities, which is now applied to schedules 20 and 26 emissions activities under EASR. This means emissions to air, water (including discharges to sewer) and land, plus a range of other environmental effects, must be considered together.

“Chemical industry activities” were formerly included within the scope as Part A prescribed/listed activities regulated under under Chapter 4, Sections 4.1 – 4.7 of the PPC Regulations. The relevant sub-sections are given below:

4.1 	Organic chemicals.

4.2 	Inorganic chemicals.

4.3 	Chemical fertiliser production.

4.4 	Biocide production.

4.5 	Pharmaceutical production.

4.6 	Explosives production.

4.7 	Manufacturing activities involving ammonia.

They are now found in “chemical industry activities” in EASR schedule 20 part 4 chapter 4.

Schedule 26 chapter 4 of EASR “chemical industry activities – formerly PPC Part A activities) may also be applicable to your activity. Please see the SEPA webpage for “chemicals” activities.

## 8.1 	Emissions to air

The main emissions from chemical activities will depend on the nature of the activity being conducted. The following lists and tables describe the main emissions associated with different types of chemical activity.

#### List 12 - Air pollutants emitted by the organic chemicals sector (continued overleaf)

- Aldrin

- Ammonia

- Anthracene

- Arsenic

- Benzene

- 1,3-butadiene

- Cadmium

- Carbon dioxide

- Carbon monoxide

- Carbon tetrachloride

- Chlordane

- Chlordecone

- Chlorine and inorganic compounds as HCl

- Chlorofluorocarbons

- Chloroform

- Chromium

- Copper

- Dichloromethane 	(methylene chloride)

- Dieldrin

- Di-(2-ethyl hexyl) phthalate

- Dioxins and furans

- Endrin

- Ethylene oxide

- Ethylene 	dichloride 	(1,2-

dichloroethane)

- Fluorine 	and 	inorganic

- compounds as HF

- Formaldehyde

- Halons

- Heptachlor

- Hexabromobiphenyl

- Hexachlorobenzene

- Hexachlorocyclohexane

- Hydrochlorofluorocarbons

- Hydrofluorocarbons

- Hydrogen chloride

- Hydrogen cyanide

- Lead

- Lindane

- Mercury

- Methane

- Methyl chloride

- Methyl chloroform (trichloroethane)

- Naphthalene

- Nickel

- Nitrogen dioxide (oxides of nitrogen – 	NO & NO2 as NO2)

- Nitrous oxide

- Non-methane volatile organic compounds

- Particulate matter (including PM10 and 2.5)

- Pentachlorobenzene

- Pentachlorophenol

- Perfluorocarbons

- Polychlorinated biphenyls

- Polycyclic aromatic hydrocarbons

- Sulphur dioxide (Oxides of sulphur – SO2 and SO3 as SO2)

- Sulphur hexafluoride

- Styrene

- Tetrachloroethane

- Tetrachloroethylene

- Toluene

- Toxaphene

- Trichlorobenzene

- Trichloroethylene

- Vinyl chloride

- Xylene

- Zinc

#### List 13 - Air pollutants emitted by the inorganic chemicals sector

- Ammonia

- Arsenic

- Benzene

- Cadmium

- Carbon dioxide

- Carbon monoxide

- Carbon tetrachloride (tetrachloromethane)

- Chlorine & inorganic compounds as HCl

- Chlorofluorocarbons

- Chloroform (trichloromethane)

- Chromium

- Copper

- Dichloromethane (methylene chloride)

- Dioxins and furans

- Ethylene dichloride (1,2-dichloroethane)

- Fluorine and inorganic compounds as HF

- Halons

- Hexachlorobenzene

- Hexachlorocyclohexane

- Hydrochlorofluorocarbons

- Hydrofluorocarbons

- Hydrogen chloride

- Hydrogen cyanide

- Hydrogen fluoride

- Lead

- Mercury

- Methane

- Methyl chloroform (trichloroethane)

- Nickel

- Nitrogen dioxide (oxides of nitrogen – NO and NO2 as NO2)

- Nitrous oxide

- Non-methane volatile organic compounds

- Particulate matter (including PM10)

- Pentachlorobenzene

- Pentachlorophenol

- Perfluorocarbons

- Phenol

- Polycyclic aromatic hydrocarbons

- Sulphur dioxide (oxides of sulphur – SO2 and SO3 as SO2)

- Sulphur hexafluoride

- Tetrachloroethane

- Tetrachloroethylene

- Trichlorobenzenes

- Trichloroethylene

- Zinc

Main air pollutants emitted by the explosives sector

- Nitrogen dioxide (oxides of nitrogen – NO and NO2 as NO2) from glycerol or cellulose nitration reactions.

Table 33: Main air pollutants emitted by the chemical fertiliser sector

| Main air pollutants | Main sources |
| --- | --- |
| Ammonia | Ammonium nitrate/phosphate production |
| Carbon dioxide | Ammonium phosphate production |
| Carbon monoxide | Ammonium phosphate production |
| Hydrogen chloride | Single super phosphate manufacture |
| Hydrogen fluoride (fluorine and inorganic compounds – as HF) | Single super phosphate manufacture |
| Methane | Ammonium nitrate/phosphate production |
| Nitrogen dioxide (oxides of nitrogen – NO and NO2 as NO2) | Ammonium nitrate production |
| Particulate matter (including PM10 and 2.5) | Ammonium nitrate/phosphate and NPK production, fugitive dust emissions from conveyors, screens, etc. Single super phosphate production |
| Sulphur dioxide (oxides of sulphur – SO2 and SO3 as SO2) | Ammonium phosphate production |

##### Table 34: Main air pollutants emitted by the plant health products and biocides sector

| Main air pollutants | Main sources |
| --- | --- |
| Carbon dioxide | Reaction, distillation and blending operations |
| Chlorine and inorganic compounds as HCl | Reaction, distillation and blending operations |
| Particulate matter (including PM10 and 2.5) | Dusts from vessel charging, crushing/pulverising and handling, blending, milling, packaging |
| Volatile organic compounds | Reaction, distillation and blending operations |

Table 35: Main air pollutants emitted by the pharmaceutical sector

| Main air pollutants | Main sources |
| --- | --- |
| Particulate matter (including PM10 and 2.5) | From vessel charging, sieving, milling, weighing, mixing, blending, granulating, tablet manufacture, filling and packing |
| Volatile organic compounds | From reaction, separation, drying, tablet manufacture and aerosol preparation |

Table 36: Main air pollutants emitted by the explosive sector

| Main air pollutants | Main sources |
| --- | --- |
| ​ Nitrogen dioxide (oxides of nitrogen – NO and NO2 as NO2) Hazardous | ​Glycerol or cellulose nitration reactions |

Table 37: Main air pollutants emitted by manufacturing carbon disulphide and ammonia

| Main air pollutants | Main sources |
| --- | --- |
| Ammonia | Start-up stages (NH3 production), pressure relief vents, condensate stripper |
| Carbon dioxide | CO2 removal plant (NH3 production), start-up, primary reformer flue gas, pressure relief vents |
| Carbon monoxide | Start-up stages (NH3 production), primary reformer flue gas, pressure relief vents |
| Methane | Start-up stages (NH3 production), pressure relief vents |
| Nitrogen dioxide (oxides of nitrogen – NO and NO2 as NO2) | Furnaces (CS2 production), primary reformer flue gas (NH3 production) |
| Sulphur dioxide (oxides of sulphur – SO2 and SO3 as SO2) | Combustion of CS2 and H2S in flare system, primary reformer flue gas (NH3 production) |

### 8.1.1 Point source emissions

These emissions are exhausted via a stack or vent, i.e. a single point source into the atmosphere. Abatement equipment, e.g. scrubbing units, fabric filters (bag house) can be incorporated into the exhaust system prior to discharge to atmosphere.

### 8.1.2 Fugitive emissions

Fugitive emissions are those that are not released from a point source such as a stack. In some chemical production activities fugitive, or diffuse, emissions may be more significant than point source emissions. Fugitive VOC emissions are often a very significant factor in emissions to air from large volume organic chemicals (LVOC) installations.

Some examples of the potential sources of fugitive emissions to air are:

- The loading and unloading of materials into transport containers.

- Storage areas (e.g. bays, stockpiles, etc.).

- Transferring material between vessels.

- Conveyor systems.

- Pipework and ductwork systems (e.g. pumps, valves, flanges, equipment vents).

- Accidental loss of containment from failed plant and equipment.

- Poor building containment and extraction.

## 8.2 	Emissions to water and wastewater

### 8.2.1 Relevant pollutants & emission sources

Water discharges from chemical processes arise from the air abatement equipment (e.g. wet scrubbers), storm water, cooling water, boiler blow-down, raw material handling areas, accidental emissions of raw materials, products or waste materials, fire-fighting and from on-site effluent treatment. An indicative list of water emissions for the organic/inorganic sectors is given in Lists 14 and 15 respectively. The main water emissions, and their sources, from the remaining chemical sub-sector activities, are shown in Tables 38 and Lists 16-19. These should be taken as a guide only, and you should verify the presence of any other pollutants emitted from the process.

List 14 - Water pollutants emitted by the organic chemicals sector (continued overleaf)

- Alachlor

- Aldrin

- Anthracene

- Arsenic

- Atrazine

- Benzene

- Benzo(g,h,i)perylene

- Brominated diphenylethers

- Cadmium

- Chlordane

- Chlordecone

- Chlorfenvinphos

- Chlorides as total Cl

- Chloropyrifos

- Chromium

- Copper

- Cyanides as total CN

- Dichlorodiphenyltrichloroethane

- Ethylene 	dichloride 	(1,2-

dichloroethane))

- Dichloromethane

- Dieldrin

- Di-(2-ethyl hexyl) phthalate

- Dioxins and furans

- Diuron

- Endosulphan

- Endrin

- Ethyl benzene

- Ethylene oxide

- Fluorides as total F

- Halogenated 	organic

compounds (as AOX)

- Heptachlor

- Hexabromobiphenyl

- Hexachlorobenzene

- Hexachlorobutadiene

- Hexachlorocyclohexane

- Lead

- Lindane

- Mercury

- Mirex

- Naphthalene

- Nickel

- Nitrogen (total)

- Nonylphenols

- Pentachlorophenol

- Phenols

- Phosphorus (total)

- Polychlorinated biphenyls

- Polycyclic aromatic hydrocarbons

- Simazine

- Tetrachloroethylene

- Tetrachloromethane

- Toluene

- Toxaphene

- Tributyltin

- Trichlorobenzenes

- Trichloroethylene

- Trichloromethane (Chloroform)

- Trifluralin

- Triphenyltin

- Vinyl chloride

- Xylenes

- Zinc

#### List 15 - Water pollutants emitted by the inorganic chemicals sector (continued on the next page)

- Arsenic

- Asbestos

- Benzene

- Benzo(g,h,i)perylene

- Cadmium

- Chlorides as total Cl

- Chromium

- Copper

- Cyanides as total CN

- Dichloromethane

- Ethylene dichloride (1,2-dichloroethane)

- Ethyl benzene

- Fluorides as total F

- Halogenated organic compounds (as AOX)

- Hexachlorobenzene

- Hexachlorobutadiene

- Hexachlorocyclohexane

- Lead

- Mercury

- Nickel

- Nitrogen (total)

- Nonylphenols

- Octylphenols

- Pentachlorobenzene

- Phenols

- Phosphorus (total)

- Polycyclic aromatic hydrocarbons

- Toluene

- Total organic carbon

- Xylenes

- Zinc

###### Table 38: Main water pollutants emitted by the chemical fertiliser sector

| Main air pollutants | Main sources |
| --- | --- |
| Particulate matter (including PM10 and 2.5) | From vessel charging, sieving, milling, weighing, mixing, blending, granulating, tablet manufacture, filling and packing |
| Volatile organic compounds | From reaction, separation, drying, tablet manufacture and aerosol preparation |

#### List 16 Main water pollutants emitted by the plant health products and biocides sector

- Aqueous organics from reaction, separation and fermentation.

#### List 17 Main water pollutants emitted by the pharmaceutical sector

- Aqueous organics from reaction, separation and fermentation.

#### List 18 Main water pollutants emitted by the explosives sector

- Acid products from nitration reactions.

#### List 19 Main water pollutants emitted by manufacturing carbon disulphide and ammonia

- Mercury from ion exchange resin regeneration (ammonia production).

The resulting discharge of the above substances depends on the ‘in process’ preventative measures (good housekeeping, re-use) and the presence and technical standards of wastewater treatment facilities. Notwithstanding the above, you should consider all emission sources to water and characterise the flows and emission concentrations from each source.

## 8.3 	Off-site waste transfers

### 8.3.1 Relevant wastes

In general, the waste streams from chemical facilities can be classified into types as follows:

- By-products for which no internal use or external sale is available.

- Residues from separation processes such as distillation.

- Catalysts which have declined in performance and require replacement.

- Filter cake, activated carbon, ion exchange resins, molecular sieves and other treatment materials.

- Sludges from wastewater treatment.

- Emptied containers and packaging.

- Maintenance and construction materials.

The key substances likely to be present can be derived from knowledge of the process, materials of construction, corrosion/erosion mechanisms and materials related to maintenance. The presence of substances created by abnormal operation should be identified.

###### Transboundary shipments of hazardous waste

For transboundary movements of hazardous waste (outwith the United Kingdom), the name and address of the recoverer or the disposer of the waste and the actual recovery or disposal site have to be reported.

# 9 	Glossary

ACI 	Animal Carcass Incineration

ALT 	Alternative (measurement method)

AOX 	Adsorbable Organic Halogens

APC 	Air Pollution Control

ART 	Above Reporting Threshold

As 	Arsenic

BaP 	Benzo(a)pyrene

BAT 	Best Available Techniques

BCA 	British Cement Association

BDEs 	Brominated Diphenylethers

BOS 	Basic Oxygen Steelmaking

BRT 	Below Reporting Threshold

BS 	British Standard

BSI 	British Standards Institute

BTX 	Benzene, Toluene, Xylene

CAR 	Water 	Environment 	(Controlled 	Activities) 	(Scotland) Regulations 2011

CCGT 	Combined Cycle Gas Turbine

CCU 	Catalytic Cracking Unit

CEMS 	Continuous Emission Monitoring System

CEN 	European Committee for Standardisation

Cd 	Cadmium

CF 	Conversion Factor

CFCs 	Chlorofluorocarbons

CH4 	Methane

CHP       		 Combined Heat and Power

| CO | Carbon Monoxide |
| --- | --- |
| CO2 | Carbon Dioxide |
| COD | Chemical Oxygen Demand |
| CoT | Committee on Toxicity |
| CN | Cyanide |
| Cr | Chromium |
| CRM | Certified Reference material |
| Cu | Copper |
| CWI | Clinical Waste Incineration |
| D&R | Disposal and Recovery |
| DI | Drum Incineration |
| EAF | Electric Arc Furnace |
| EASR | Environmental Authorisations (Scotland) Regulations (2018 and as amended 2025) |
| EC | European Commission |
| EEA | European Environment Agency |
| EF | Emission Factor |
| EGR | Exhaust Gas Recirculation |
| EIA | Environmental Impact Assessment |
| ELV | Emission Limit Value |
| EPER | European Pollutant Emission Register |
| ESI | Electricity Supply Industry |
| ESP | Electrostatic Precipitator |
| ETP | Effluent Treatment Plant |
| EU | European Union |
| EU ETS | European Union Emissions Trading System |
| E-PRTR | European Pollutant Release and Transfer Register |
| ESP | Electrostatic Precipitator |
| EWC | European Waste Catalogue |
| FCCU | Fluidised Catalytic Cracking Unit |
| FGD | Flue Gas Desulphurisation |

| FGR | Flue Gas Recirculation |
| --- | --- |
| FSF | Fully Slatted Floor |
| GCV | Gross Calorific Value |
| FID | Flame Ionisation Detector |
| GHG | Greenhouse Gas |
| HCFCs | Hydrochlorofluorocarbons |
| HCl | Hydrogen Chloride |
| HCN | Hydrogen Cyanide |
| HF | Hydrogen Fluoride |
| HFCs | Hydrofluorocarbons |
| HFO | Heavy Fuel Oil |
| Hg | Mercury |
| H2S | Hydrogen Sulphide |
| HW | Hazardous Waste |
| HWI | Hazardous Waste Incineration |
| IBC | Intermediate Bulk Container |
| IED | Industrial Emissions Directive |
| INT | Internationally Approved (measurement standard) |
| IOWWTP | Independently-operated Wastewater Treatment Plant |
| IPC | Integrated Pollution Control |
| IPPC | Integrated Pollution Prevention and Control |
| I-TEF | International Toxicity Equivalency Factor |
| I-TEQ | International Toxicity Equivalents of Dioxins |
| JEP | Joint Environment Programme |
| K | Kelvin (unit of temperature) |
| LOD | Limit of Detection |
| LPG | Liquefied Petroleum Gas |
| LRTAP | Long-range transboundary air pollution (convention on) |
| LVOC | Large Volume Organic Chemicals |
| MAB | Mass Balance Method |
| MBq | Mega Becquerel |
| MCERTS | (Environment Agency’s) Monitoring Certification Scheme |
| Mg(OH)2 | Magnesium Hydroxide |
| Mn | Manganese |
| MRR | Monitoring and Reporting Requirements |
| MSW | Municipal Solid Waste |
| MWI | Municipal Waste Incineration |
| NAEI | National Atmospheric Emissions Inventory |
| NCV | Net calorific value |
| Ni | Nickel |
| NH3 | Ammonia |
| NIC | National Identity Code |
| NLA | No Longer Applicable |
| NOx | Oxides of nitrogen (mixture of NO and NO2) |
| NO2 | Nitrogen Dioxide |
| NO | Nitric Oxide |
| N2O | Nitrous Oxide |
| NRB | National or Regional Binding (measurement method) |
| OCC | Opencast Coal |
| OTH | Other (measurement method) |
| NMVOCs | Non-methane Volatile Organic Compounds |
| PAHs | Polycyclic Aromatic Hydrocarbons |
| Pas | Publically Available Standard |
| Pb | Lead |
| PCBs | Polychlorinated Biphenyls |

| PCDDs PCDF PER PF PFBC PFCs PI PM PM2.5 P PM10 M10 | Polychlorinated Dibenzodioxins Polychlorinated Dibenzofurans Measurement method already prescribed by the competent authority (SEPA) in a license or permit for that facility Pulverised Fuel Pulverised Fuel Bed Combustion Polyfluorinated Hydrocarbons Pollution Inventory Particulate Matter Particulate Matter (<2.5μm aerodynamic diameter) Particulate Matter (<10μm aerodynamic diameter) |
| --- | --- |
| PPC | Pollution Prevention and Control (Scotland) Regulations 2012 (as amended) |
| Ppm | Parts per million |
| Ppmv | Parts per Million by Volume |
| PSF | Partially Slatted Floor |
| PVC | Polyvinyl Chloride |
| RCF | Recycled Fibre |
| RET | Release Estimation Technique |
| RS | Radioactive Substances |
| Sb | Antimony |
| SCR | Selective Catalytic Reduction |
| SIC | Standard Industry Classification |
| SLF | Substitute Liquid Fuel |
| Sn | Tin |
| SNCR | Selective Non-catalytic Reduction |
| SOx | Oxides of Sulphur (mixture of SO2 and SO3) |
| SO2 | Sulphur Dioxide |
| SO3 | Sulphur Trioxide |
| SRU | Sulphur Recovery Unit |
| SSC | Sector Specific Calculation |
| SSI | Sewage Sludge Incineration |
| SWS | Sour Water Scrubber |
| SPRI | Scottish Pollutant Release Inventory |
| TOC | Total Organic Carbon |
| TPM | Total Particulate Matter |
| TSS | Total Suspended Solids |
| UKSIC | United Kingdom Standard Industry Classification |
| UNECE | United Nations Economic Commission for Europe |
| UK ETS | UK Emissions Trading Scheme |
| UNFCCC | United Nations Framework Convention on Climate Change |
| USEPA | United States Environmental Protection Agency |
| UWWTP | Urban Wastewater Treatment Plant |
| V | Vanadium |
| VC | Vinyl Chloride |
| VDU | Vacuum Distillation Unit |
| VOCs | Volatile Organic Compounds |
| WESP | Wet Electrostatic Precipitator |
| WEWS | Water Environment and Water Services (Scotland) Act 2003 |
| WHO | World Health Organisation |
| WHO-TEF | WHO Toxicity Equivalency Factor |
| WHO-TEQ | Toxicity Equivalents of Dioxins |
| WML | Waste Management Licence |
| WWTP | Wastewater Treatment Plant |
| Zn | Zinc |

## Disclaimer

This guidance is based on the law as it stood when the guidance was published.

Whilst every effort has been made to ensure the accuracy of this guidance, SEPA gives no warranty, covenant or undertaking (express or implied) regarding the fitness for purpose of, or any error, omission or discrepancy in this guidance. Reliance on its contents and the contents of any websites that are linked to or from this guidance is entirely at the user’s own risk. SEPA is not liable for any loss or damage that may come from using this guidance. This includes:

- any direct, indirect and consequential losses

- any loss or damage caused by civil wrongs, breach of contract or otherwise.

SEPA reserves the right to depart from this guidance and take appropriate action as it considers necessary or appropriate. Operators are responsible for ensuring that they are compliant with the law. If necessary, independent legal / specialist advice should be sought.