Managing waste institutional mattresses containing Persistent Organic Pollutants (POPs)

## Version 1.0 April 2026

# What is this guidance and who is it for

This guidance provides information on waste institutional mattresses that contain Persistent Organic Pollutants (POPs) and should be read in conjunction with the SEPA guidance document WAS-G-POPS-01, which outlines the full regulatory framework around POPs wastes. This guidance will support waste holders in classifying and assessing their POPs contaminated institutional mattresses (POPs IMs) and help ensure that they are managed in an appropriate manner.

# POPs in institutional mattresses

The mattresses used in institutional settings tend to have a ‘protective’ PVC coated textile or polyurethane outer cover over a foam base, which tends to be either combustion modified foam, polyurethane foam or polyether foam. A recent study concluded that the mattress covers found on some Institutional Mattresses contain high concentrations of brominated flame-retardant POPs. The concentration levels present in the POPs contaminated mattress covers are such that they are classed as a POPs waste and are subject to the POPs regulations.

If the covers are removed, the mattress will not be classed a POPs waste, however if the POPs contaminated mattress covers are not removed from the mattress, the concentration levels are sufficiently high enough to deem a covered mattress as POPs waste.

# Identifying waste institutional mattresses containing POPs

For the purpose of this guidance, ‘Institutional Mattresses’ are taken to mean mattresses commonly used in the following settings:

Health and Social Care:

  - Healthcare – mattresses identified as originating from either hospital or care home settings and hospital wards, including those classified as clinical waste.

  - Mental health – mattresses identified as originating from mental health institutions, including those from medium and high-risk environments.

Prison – mattress samples provided by the Ministry of Justice and those identified as originating from prisons.

Defence – mattresses using in military installations.

Border Control – mattresses used in border control facilities.

The majority of POPs contaminated institutional mattresses or mattress covers are found in health and social care mattresses sector. However, if you are unsure about institutional mattresses from other sectors you should check if the cover contains POPs.

Evidence suggests that the waterproof, flexible covers of Crib 5 (BS6807:2006) used in health and social care (healthcare/mental health) mattresses may contain persistent organic pollutants (POPs):

Healthcare – Decabromodiphenyl ether (DecaBDE) present in high concentrations (>10,00 mg/kg) in up to 20% of the covers of some Crib 5 mattresses.

Mental health – Decabromodiphenyl ether (DecaBDE) present in high concentrations (>10,00 mg/kg) in up to 30% of the covers of some Crib 5 mattresses.

There is no evidence that the mattress foam contains this POP.

Similar Crib 5 mattress covers used in other sectors might also contain POPs. You should check if the cover contains POPs, if unsure.

Waste mattresses from prisons are not expected to contain POPs.

You can use x-ray fluorescence (XRF) to scan for bromine, as the presence of bromine indicates that POPs are likely to be present. However, you must:

have the right training for using XRF and use a suitable device

contact SEPA to check if you need to make any changes to your permit and management system

be able to show that your process works and is accurate, as SEPA may ask you for this evidence.

If no bromine is found to be present after multiple scans of the front and rear of the cover then brominated POPs will not be present.

Classification and description

Under Section 34 of the Environmental Protection Act 1990 waste holders have a duty to describe their waste fully, to ensure that it is handled appropriately. Where the presence of POPs has been confirmed or should be assumed, the waste institutional mattresses must be classified and described in the waste transfer note as the following:

20 03 07 – ‘mattresses containing POPs’

Although it may contain POPs, waste under a 20 03 07 code is classified as absolute non-hazardous.

You must make the recipient aware that DecaBDE is present. This should be done on the waste transfer note, but if this is not possible then it can be provided separately.

# Collection and transportation

Producers and holders of waste are legally required to undertake all reasonable efforts to avoid, where feasible, contamination of non-POPs waste with POPs waste. By avoiding contamination of non-POPs waste, you will minimise the impact of handling, storing and disposing of any POPs waste and the associated costs. Non-compliance with this duty is an offence.

Where separate collection is not feasible, you can collect waste institutional mattresses containing POPs on the same vehicle as other waste items if they are:

not mixed with other waste

collected in a way that does not contaminate the other waste (for example without compaction)

separated from the other waste when unloaded from the vehicle.

Where mixing occurs, the whole load of ‘mixed waste’ should be considered a POPs contaminated waste.

If the POPs waste cannot be removed, the ‘mixed waste’ should be deemed a POPs waste and sent for destruction even if the mixing has diluted the POPs content to below the concentration limit.

# Sort and store

You must sort and store waste institutional mattresses containing POPs separately from other waste and in a way that prevents:

damage to the cover

release of POPs

contamination with other waste

You must avoid mixing waste institutional mattresses containing POPs with other waste during waste:

production

storage

collection

treatment

Where you meet these requirements, waste institutional mattresses can be stored together with segregated waste upholstered domestic seating at household waste recycling centres. You must separate them before any pre-treatment of either waste stream, as the pre-treatment requirements differ.

# Treatment

## Removing covers containing POPs from mattresses

You may pretreat the waste mattress by removing the cover from the mattress.

If you operate a waste management facility you must be authorised to carry out this activity.

If you operate a health or social care facility, where the waste was produced, you do not need an authorisation to do this at that premises.

If you separate, you must make sure:

you store separated material containing POPs inside a building, under cover, or in a sealed container

that no POPs are released to the sewer or surface water

all cover material is sent for destruction.

Other forms of treatment, before destruction, are not allowed. This waste may only be mixed with other waste during the loading of the incinerator.

Once the cover that contains the POPs has been removed, you do not have to manage the foam part of the mattress as POPs waste.

# Disposal and destruction

## Requirements for landfill operators

Landfill operators must not accept any waste institutional mattresses, or their covers, containing POPs. This includes:

segregated institutional mattresses or their covers

mixed or treated waste containing institutional mattresses or their covers.

Your waste acceptance procedures must screen out waste institutional mattresses and their covers. You must quarantine this waste type and send it to an appropriately authorised incinerator for destruction.

## Disposal requirements for waste institutional mattresses containing POPs

You must incinerate the covers of waste institutional mattresses containing POPs.

The municipal or hazardous waste incinerator (or cement kiln) must be authorised to accept POPs waste.

You must not use any other type of incinerator, for example a clinical waste incinerator, or small waste incineration plant (with a capacity below that set out in Section 5.2 of Annex I to the EU Industrial Emission Directive (2010/75/EU) unless:

the operator has implemented bi-annual polybrominated dibenzo-dioxins and furans monitoring to air

the relevant environment agency has approved evidence of destruction efficiency of brominated POPs submitted by the operator.

You must not use any other waste treatment, recycling or disposal methods, for example landfill or clinical waste treatment plant.

## Contingency plan for incinerator shutdown

When the incinerator cannot accept the waste, you can:

store the waste until the incinerator is able accept it

send it to another incinerator or cement kiln authorised to accept and destroy it.

You must not send it for any other type of disposal or recovery operation, for example landfill.

Incinerator operators need to put contingency plans in place to manage accepting waste institutional mattresses from customers. They cannot send this waste to landfill in the event of a planned or unplanned shutdown.

# Export and import

## Exporting waste institutional mattresses for incineration

You must only export waste institutional mattresses to European Union or European Free Trade Association countries for incineration with energy recovery (R1).

You must make sure that the presence of POPs is made clear in block 12 ‘designation and composition’ of your notification.

## Reusing waste institutional mattresses

You must not reuse waste institutional mattresses containing POPs either in the UK or by exporting them overseas.

As the holder of the waste, you must send the POPs for destruction.

If the mattress is not waste, you may reuse them or export them for reuse.

You should inform the person reusing the mattresses that POPs are present as the mattress will require specialist management at end of life. If you are exporting the mattress, you should consider that some countries may have limited technical capacity to identify and manage POPs waste in a manner that protects the environment and human health.

## Waste from manufacturing of institutional mattresses

When you manufacture waste institutional mattresses you must know what chemicals are in the covers, foams and other materials used.

Suppliers from some countries that have not fully implemented the Stockholm Convention may supply materials containing chemicals, for example flame retardants, that are POPs. You should ask the manufacturer for evidence of which flame retardant is present. If the manufacturer cannot provide evidence of what chemicals it contains and in what quantities (either via a supplier’s specification or analytical testing) you must assume that it contains POPs.

You must describe and classify any waste you produce from manufacturing institutional mattresses correctly. This is so that your waste contractor can manage it appropriately.

You should clearly label your new products with the chemicals that are present in their components. This will make sure that they are managed appropriately in the future, including when they are:

reused

recycled

disposed of

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk

Crib 5 also known as Ignition Source 5 refers to material or furniture that’s been tested to pass UK Fire Regulations. The test is designed to determine the possible risk of ignition, ensure that a material or furnishing won’t easily catch fire and save lives.