Environmental benchmarks

by substance in Scotland: Air

Version 3.0 September 2026

IND-G-027

Contents

Permit requirements for emissions to air	2

How the risk assessment works	2

Air Quality Standards	3

Ecology Critical Levels	5

Ecology Critical Loads	6

Environmental Assessment Levels (EALs)	7

Disclaimer	16

This guidance has been updated to meet accessibility standards and to replace certain references to legislation with references to the Environmental Authorisations (Scotland) Regulations 2018. It has not been reviewed beyond this. We are aware that sections of this guidance may need to be updated, and this work will be completed in due course.

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk

# Permit requirements for emissions to air

In your permit application/variation for a site in Scotland you need to include the following technical information:

- A description of the activities or proposed changes to your existing activities.

- Identify releases to air, including the nature, quantity, source and location of pollutants.

- Identify all sensitive receptors (people and protected sites) within the relevant screening distance from your site.

- Assess whether there is a likely significant effect or whether it can be screened out as insignificant (following the steps and using the screening tools detailed below).

- Submit your air emissions risk assessment as part of your permit application/variation.

Further detail and information are provided on the SEPA webpage for Air Emissions Risk Assessment for Environmental Permits which also contains links to more guidance and advice.

## How the risk assessment works

You need to compare the impact of your emissions to air to the following environmental benchmarks, and where relevant odour thresholds:

- Air Quality Standards (Scotland) Regulations (which give effect to EU Limit/Target Values).

- UK Air Quality Strategy Objectives for Scotland.

- Critical loads and critical levels for ecological receptors.

- Environmental Assessment Levels (EALs).

See environmental benchmarks by substance in this document for more information. The information in these tables are subject to amendments as new benchmarks are identified or additional benchmarks are added.

# Air Quality Standards

| Substance | Long Term concentration (µg/m3) | Short Term concentration (µg/m3) | Derivation method or information source |
| --- | --- | --- | --- |
| 1,3 Butadiene | 2.25 |  | UK Air Quality Strategy Objectives |
| Arsenic and compounds (as As) | 0.006 |  | The Air Quality Standards (Scotland) Regulations 2010 |
| Benzene (24h mean) | 3.25 | 30 | LT: The Air Quality (Scotland) Amendment Regulations 2002/ST: HSE EH40 (2001) |
| Benzo-a-pyrene (PAH) | 0.00025 |  | The Air Quality Standards (Scotland) Regulations 2010 |
| Cadmium and its compounds (as Cd) | 0.005 |  | The Air Quality Standards (Scotland) Regulations 2010 |
| Carbon monoxide (8h mean) |  | 10000 | The Air Quality Standards (Scotland) Regulations 2010 |
| Lead | 0.25 |  | The Air Quality (Scotland) Regulations 2000. |
| Nickel | 0.02 |  | The Air Quality Standards (Scotland) Regulations 2010 |
| Nitrogen Dioxide | 40 | 200 | The Air Quality Standards (Scotland) Regulations 2010 |
| Ozone (Running 8 Hour Mean) |  | 100 | The Air Quality Standards (Scotland) Regulations 2010 |
| Particulates (PM10) (24 hr Mean) |  | 50 | The Air Quality Standards (Scotland) Regulations 2010 |
| Particulates (PM10) | 18 |  | The Air Quality (Scotland) Amendment Regulations 2002 |
| Particulates (PM2.5) | 10 |  | The Air Quality (Scotland) Amendment Regulations 2016 |
| Sulphur Dioxide (15 Min Mean) |  | 266 | The Air Quality Standards (Scotland) Regulations 2010 |
| Sulphur Dioxide (24 Hour Mean) |  | 125 | The Air Quality Standards (Scotland) Regulations 2010 |

# Ecology Critical Levels

For more details see: Critical Loads and Critical Levels - a guide to the data provided in APIS | Air Pollution Information System

| Substance | Long Term concentration (µg/m3) | Short Term concentration (µg/m3) | Derivation method or information source |
| --- | --- | --- | --- |
| Ammonia (ecological receptor - Sensitive Lichens) | 1 |  | UN ECE Convention on Long-range Transboundary Air Pollution (2007) |
| Ammonia (ecological receptor - other vegetation) | 3 |  | UN ECE Convention on Long-range Transboundary Air Pollution (2007) |
| Hydrogen fluoride (as F) (Ecological - Weekly mean/ Daily Mean) | 0.5 | 5 | WHO, 2002 Environmental Health Criteria 227: Fluorides pp 230 |
| Nitrogen Dioxide (Ecological - Daily Mean) | 30 | 75 | LT: The Air Quality Standards (Scotland) Regulations 2010/ST: WHO Air quality Guidelines for Europe 2000, 2nd edition |
| Sulphur Dioxide (Ecological - Sensitive Lichens) | 10 |  | WHO Air quality Guidelines for Europe 2000, 2nd edition |
| Sulphur Dioxide (Other Ecology) | 20 |  | The Air Quality Standards (Scotland) Regulations 2010 |
| Sulphur Dioxide (Ecological - Crops) | 30 |  | UN ECE Convention on Long-range Transboundary Air Pollution (2007) |

# Ecology Critical Loads

For more details see: Critical Loads and Critical Levels - a guide to the data provided in APIS | Air Pollution Information System

| Substance | Long Term concentration (µg/m3) | Short Term concentration (µg/m3) | Derivation method or information source |
| --- | --- | --- | --- |
| Nutrient nitrogen deposition (habitats specific) |  |  | For more details see: Site Relevant Critical Loads and Source Attribution ¦ Air Pollution Information System (apis.ac.uk) |
| Acid deposition for Nitrogen and sulphur (Site specific) |  |  | For more details see: Site Relevant Critical Loads and Source Attribution ¦ Air Pollution Information System (apis.ac.uk) |

# Environmental Assessment Levels (EALs)

| Substance | Long Term concentration (µg/m3) | Short Term concentration (µg/m3) | Derivation method or information source |
| --- | --- | --- | --- |
| 1,1,1-Trichloroethane (Methylchloroform) (24h mean long term) | 5000 |  | EA Hazard characterisation method for determining TCA (2021) |
| 1,2-Dichloroethane (Ethylene dichloride) | 3 |  | EA Hazard characterisation method for determining TCA (2021) |
| 1,2,4-Trichlorobenzene | 76 | 2280 | HSE EH40 (2001) |
| 1,2-Dibromoethane | 7.8 | 234 | HSE EH40 (2001) |
| 1,3 Butadiene (24h mean) |  | 2.25 | EA Hazard characterisation method for determining TCA (2021) |
| 1,4-Dichlorobenzene | 120 | 6000 | EH40/2005 (4th ed 2020) |
| 1,4-Dioxane | 730 | 36600 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| Acetaldehyde | 370 | 9200 | EH40/2005 (4th ed 2020) |
| Acetic acid | 250 | 3700 | HSE EH40 (2001) |
| Acetic anhydride | 1 | 40 | HSE EH40 (2001) |
| Acetone | 12100 | 362000 | EH40/2005 (4th ed 2020) |
| Acetonitrile | 680 | 10200 | EH40/2005 (4th ed 2020) |
| Acrylamide | 0.05 |  | EA Hazard characterisation method for determining TCA (2021) |
| Acrylic acid | 290 | 5900 | EH40/2005 (4th ed 2020) |
| Acrylonitrile | 8.8 | 264 | HSE EH40 (2001) |
| Allyl alcohol | 48 | 970 | EH40/2005 (4th ed 2020) |
| Ammonia (human health receptor) | 180 | 2500 | EH40/2005 (4th ed 2020) |
| Aniline | 8 | 240 | HSE EH40 (2001) |
| Antimony and compounds (as Sb) except antimony | 5 | 150 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| Arsine | 1.6 | 48 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| Benzylchloride | 5.2 | 158 | HSE EH40 (2001) |
| Beryllium and compounds (as Be) | 0.0002 |  | EPAQS Metals and Metalloids (2009) |
| Boron trifluoride |  | 280 | HSE EH40 (2001) |
| Bromine |  | 70 | EPAQS Halogen and Hydrogen Halides (2006) |
| Bromomethane | 200 | 5900 | EH40/2005 (4th ed 2020) |
| Butan-2-one (Methylethylketone) | 6000 | 89900 | EH40/2005 (4th ed 2020) |
| Butane | 14500 | 181000 | EH40/2005 (4th ed 2020) |
| Cadmium (24h mean) |  | 0.03 | Agency for Toxic Substances and Disease Registry (2012) |
| Carbon disulphide (24h mean) | 64 | 100 | LT: HSE EH40 (2001)/ST: WHO Air quality Guidelines 2000 |
| Carbon tetrachloride | 64 | 3200 | EH40/2005 (4th ed 2020) |
| Chlorine |  | 290 | EPAQS Halogen and Hydrogen Halides (2006) |
| Chlorobenzene | 47 | 1400 | EH40/2005 (4th ed 2020) |
| Chloroform (24h mean long term) | 100 |  | EA Hazard characterisation method for determining TCA (2021) |
| Chloromethane (Methyl chloride) (24h mean long term) | 18 |  | World Health Organization International programme for Chemical Safety (2000) |
| Chromium (VI) compounds (as Cr) | 0.00025 |  | EA Hazard characterisation method for determining TCA (2021) |
| Chromium (III) compounds (as Cr) (24h mean long term) | 2 |  | EA Hazard characterisation method for determining TCA (2021) |
| Copper and compounds (as Cu) (24h mean long term) | 0.05 |  | EA Hazard characterisation method for determining TCA (2021) |
| Dibutyl phthalate (DBP) | 50 | 1000 | EH40/2005 (4th ed 2020) |
| Dichloromethane (Methylene Chloride) (24h mean) | 770 | 2100 | LT: Hazard characterisation method for determining TCA (2021)/ST: Agency for Toxic Substances and Disease Registry (2000) |
| Diethyl ether | 3100 | 62000 | EH40/2005 (4th ed 2020) |
| Di-i-propyl ether | 10600 | 131000 | EH40/2005 (4th ed 2020) |
| Diisobutyl phthalate | 50 | 1500 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| Dimethyl sulphate | 0.52 | 15.6 | HSE EH40 (2001) |
| Dimethylformamide | 300 | 6100 | HSE EH40 (2001) |
| Ethyl acrylate | 210 | 4200 | EH40/2005 (4th ed 2020) |
| Ethylbenzene | 4410 | 55200 | EH40/2005 (4th ed 2020) |
| Ethylene oxide | 0.002 |  | EA Hazard characterisation method for determining TCA (2021) |
| Formaldehyde (30min mean) | 5 | 100 | LT: HSE EH40 (2001)/ST: WHO Air quality Guidelines for Europe 2000 |
| Hydrazine | 0.06 | 2.6 | HSE EH40 (2001) |
| Hydrogen bromide |  | 700 | EPAQS Halogen and Hydrogen Halides (2006) |
| Hydrogen chloride |  | 750 | EPAQS Halogen and Hydrogen Halides (2006) |
| Hydrogen cyanide (24h mean long term) | 2 |  | EA Hazard characterisation method for determining TCA (2021) |
| Hydrogen fluoride (as F) (Monthly Mean) | 16 | 160 | LT: EPAQS Addendum to Halogens and Hydrogen Halides Report (2009)/ST: EPAQS Halogen and Hydrogen Halides (2006) |
| Hydrogen iodide (Monthly Mean) | 5 | 520 | LT: EPAQS Addendum to Halogens and Hydrogen Halides Report (2009)/ST: EPAQS Halogen and Hydrogen Halides (2006) |
| Hydrogen sulphide (24h mean) | 70 | 150 | LT: EH40/2005 (4th ed 2020)/ST: WHO Air quality Guidelines for Europe 2000 |
| Manganese and compounds (as Mn) | 0.15 | 1500 | LT: WHO Air quality Guidelines for Europe 2000/ST: HSE EH40 (2001) |
| Mercury and its inorganic compounds (as Hg) (24h mean long term) | 0.06 | 0.6 | LT: EA Hazard characterisation method for determining TCA (2021)/ST: California Office of Environmental Health Hazard Assessment (2008) |
| Methanol | 2660 | 33300 | EH40/2005 (4th ed 2020) |
| Mono-ethanolamine (MEA) (24h mean long term) | 100 | 400 | EA Hazard characterisation method for determining TCA (2021) |
| Naphthalene (24h mean long term) | 3 |  | EA Hazard characterisation method for determining TCA (2021) |
| n-Hexane | 720 | 21600 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| N-nitrosodimethylamine (NDMA) | 0.0002 |  | EA Hazard characterisation method for determining TCA (2021) |
| Nickel and compounds (as Ni) except nickel carbonyl |  | 0.7 | EA Hazard characterisation method for determining TCA (2021) |
| Nitric acid | 52 | 260 | LT: HSE EH40 (2001) /ST: EH40/2005 (4th ed 2020) |
| Nitrogen monoxide (Nitric oxide) | 25 | 4400 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| Orthophosphoric acid | 10 | 200 | EH40/2005 (4th ed 2020) |
| Pentan-2-one (Methylpropylketone) | 7160 | 89500 | EH40/2005 (4th ed 2020) |
| Pentan-3-one (Diethylketone) | 7160 | 89500 | EH40/2005 (4th ed 2020) |
| Phenol | 78 | 1600 | EH40/2005 (4th ed 2020) |
| Phosgene | 0.8 | 25 | EH40/2005 (4th ed 2020) |
| Phosphine | 1.4 | 28 | EH40/2005 (4th ed 2020) |
| Polychlorinated biphenyls (PCB) | 0.2 | 6 | HSE EH40 (2001) |
| Propan-1-ol | 5000 | 62500 | EH40/2005 (4th ed 2020) |
| Propan-2-ol | 9990 | 125000 | EH40/2005 (4th ed 2020) |
| Propionic acid | 310 | 4600 | EH40/2005 (4th ed 2020) |
| Propylene oxide | 24 | 720 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| Selenium and compounds, except hydrogen selenide (as Se) (24h mean long term) | 2 |  | EA Hazard characterisation method for determining TCA (2021) |
| Sodium hydroxide |  | 200 | EH40/2005 (4th ed 2020) |
| Styrene (1 week average long term) | 260 | 800 | LT: WHO air quality guidelines for Europe (WHO 2000)/ST:HSE EH40 (2001) |
| Sulphur hexafluoride | 60700 | 759000 | EH40/2005 (4th ed 2020) |
| Sulphuric acid | 10 | 300 | HSE EH40 (2001) |
| Tetrachloroethylene (24h mean long term) | 40 |  | EA Hazard characterisation method for determining TCA (2021) |
| Tetrahydrofuran | 1500 | 30000 | EH40/2005 (4th ed 2020) |
| Toluene |  | 8000 | WHO air quality guidelines for Europe (WHO 2000) |
| Toluene (1 week average) |  | 260 | WHO air quality guidelines for Europe (WHO 2000) |
| Trichloroethylene | 2 |  | EA Hazard characterisation method for determining TCA (2021) |
| Trimethylbenzenes, all isomers or mixtures | 1250 | 37500 | LT: EH40/2005 (4th ed 2020)/ST: HSE EH40 (2001) |
| Vanadium (24h mean) |  | 1 | WHO Air quality guidelines for Europe 2000 |
| Vinyl acetate | 176 | 3520 | EH40/2005 (4th ed 2020) |
| Vinyl chloride (24h mean) | 10 | 1300 | EA Hazard characterisation method for determining TCA (2021) |
| Xylene, o-, m-, p- or mixed isomers | 2200 | 44100 | EH40/2005 (4th ed 2020) |
| Zinc oxide | 50 | 1000 | HSE EH40 (2001) |

This guidance has been updated to meet accessibility standards and to replace certain references to legislation with references to the Environmental Authorisations (Scotland) Regulations 2018. It has not been reviewed beyond this. We are aware that sections of this guidance may need to be updated, and this work will be completed in due course.

## Disclaimer

This guidance is based on the law as it stood when the guidance was published.

Whilst every effort has been made to ensure the accuracy of this guidance, SEPA gives no warranty, covenant or undertaking (express or implied) regarding the fitness for purpose of, or any error, omission or discrepancy in this guidance. Reliance on its contents and the contents of any websites that are linked to or from this guidance is entirely at the user’s own risk. SEPA is not liable for any loss or damage that may come from using this guidance. This includes:

- any direct, indirect and consequential losses

- any loss or damage caused by civil wrongs, breach of contract or otherwise.

SEPA reserves the right to depart from this guidance and take appropriate action as it considers necessary or appropriate. Operators are responsible for ensuring that they are compliant with the law. If necessary, independent legal / specialist advice should be sought.