Version 2.0: August 2025

IND-LF-G-005

SEPA Guidance:

The disposal of gypsum wastes in non-hazardous waste landfills

This guidance has been updated to meet accessibility standards and to replace certain references to legislation with references to the Environmental Authorisations (Scotland) Regulations 2018. It has not been reviewed beyond this. We are aware that sections of this guidance may need to be updated, and this work will be completed in due course.

# Background

In relation to the disposal of gypsum wastes, COUNCIL DECISION of 19 December 2002 establishing criteria and procedures for the acceptance of waste at landfills pursuant to Article 16 of and Annex II to Directive 1999/31/EC states that, “non-hazardous gypsum-based materials should be disposed of only in landfills for non-hazardous waste in cells where no biodegradable waste is accepted”.

This is because the disposal of gypsum wastes with biodegradable wastes can lead to the evolution of hydrogen sulphide, which can be harmful to health.

# Disposal of Gypsum Waste

Producers of gypsum waste should separate it for recovery and recycling wherever possible. Where this is not possible and wastes containing gypsum are sent to landfill, they must be deposited in a separate cell in which no biodegradable waste is accepted.

The design of a separate cell for gypsum wastes must be risk assessed and addressed on a site-specific basis. The following principles apply:

- No contact with other wastes except suitable inert wastes used as cover material.

- The generation of leachate within the gypsum cell should be minimised.

- Contact should be minimised between leachate arising from biodegradable wastes and that arising from gypsum wastes. and

- The gypsum waste cell must not compromise the landfill gas and leachate management of the biodegradable waste cells.

# Limits on Biodegradability of Wastes Deposited with Gypsum

The following limit values in table 1 below apply to wastes landfilled together with Gypsum based materials. The value for Dissolved Organic Carbon applies to a liquid to solid ratio of 10 litres/kilogram using the 2 stage CEN leaching test EN 12457/3.

Table 1

| Component | Limit Value |
| --- | --- |
| Dissolved Organic Carbon* | 800 mg/kg dry substance |
| Total Organic Carbon | 5% |

* If the waste does not meet these values for dissolved organic carbon (DOC) at its own pH, it may be tested at L/S = 10 l/kg and pH of 7.5 to 8.0. If the result of this determination does not exceed 800 mg/kg, the waste may be considered as complying with the acceptance criteria for DOC. (A method based on BSEN 14429 is available).

This guidance has been updated to meet accessibility standards and to replace certain references to legislation with references to the Environmental Authorisations (Scotland) Regulations 2018. It has not been reviewed beyond this. We are aware that sections of this guidance may need to be updated, and this work will be completed in due course.

## Disclaimer

This guidance is based on the law as it stood when the guidance was published.

Whilst every effort has been made to ensure the accuracy of this guidance, SEPA gives no warranty, covenant or undertaking (express or implied) regarding the fitness for purpose of, or any error, omission or discrepancy in this guidance. Reliance on its contents and the contents of any websites that are linked to or from this guidance is entirely at the user’s own risk. SEPA is not liable for any loss or damage that may come from using this guidance. This includes:

- any direct, indirect and consequential losses

- any loss or damage caused by civil wrongs, breach of contract or otherwise.

SEPA reserves the right to depart from this guidance and take appropriate action as it considers necessary or appropriate. Operators are responsible for ensuring that they are compliant with the law. If necessary, independent legal / specialist advice should be sought.

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk