The Scottish Pollutant Release Inventory (SPRI) Schedule 2025

This Schedule covers the year 2025. Annual returns must be submitted by 28 February 2026

FOR REFERENCE ONLY: DO NOT COMPLETE THIS SCHEDULE

This Schedule should be used to identify the information on pollutant emissions and waste transfers which must be reported to the Scottish Pollutant Release Inventory.

To make your annual return, you should complete the operator form supplied to the SPRI contact by email, unless you have an agreed alternative arrangement to supply SEPA with this data. Note that parts of the form will be pre-populated with your information.

Sections A to G – Show the information you are required to report to SPRI

Section H – Identifies the Activities which require emissions and waste transfers to be reported for any Facility carrying them out.

Updates for 2025:

Administrative updates to reflect the new reporting method are included.

Section A – Address and contact information

Information in this section is carried over annually. You should inform us if any part of it has changed since your 2024 SPRI return was submitted.

Table 1: Address and contact information

| NIC number | The unique identifier for SPRI facilities |
| --- | --- |
| Facility Details: | SEPA-regulated facility |
| Facility Name |  |
| Facility Address |  |
| Authorisation, Licence or Permit Number(s) |  |
| Company Details: | “Company” refers to the company that has been granted an authorisation by SEPA to operate under a particular regulation at a particular facility. |
| Company Name |  |
| Registered Company Number | Where applicable. |
| Proper Address | The Proper Address is the address to which SEPA sends all formal correspondence such as Notices, Licences and Variations. This is the address to which SEPA will serve the Notice under Regulation 37(1) of the Environmental Authorisations (Scotland) Regulations 2018 (the “Regulations”). |
| Company email address |  |
| SPRI Contact Details: | The contact details of the individual within the company who is responsible for ensuring the company meets its obligations to SPRI. (This may not be the person who completes the return). |
| Contact Name |  |
| Contact Telephone Number |  |
| Contact Email Address |  |

Section B – Activity information

Information in this section is carried over annually. You should inform us if any part of it has changed since your 2024 SPRI return was submitted.

Further information on SPRI activity codes is available at the SPRI activity and SIC codes  webpage.

Table 2: Activity Information

| Activity | Code type | Code | Description |
| --- | --- | --- | --- |
| Main/Primary Activity at Installation | SPRI |  |  |  |
| Sub-activity 1 | SPRI |  |  |  |
| Sub-activity 2 | SPRI |  |  |  |
| Sub-activity 3 | SPRI |  |  |  |
| Etc. | SPRI |  |  |  |

Section C – Pollutant releases

Please check the guidance listed on the SPRI operator guidance webpage for detailed information on completing your return.

For all pollutants emitted you must provide:

Table 3: Pollutants - Field information and data requirements

| Name of field | Field type | What you should provide |
| --- | --- | --- |
| Measurement type (M, C or E) | Drop down | State whether your emission value is Measured, Calculated or Estimated. |
| Method type | Drop down | Use the option which best matches the method you used to produce your emission value. Where available the method must include the UK or international quality standard reference number(s). |
| Method description | Free text (unlimited) | Please explain how you produced the emission value. Include calculations where appropriate. The more information you provide, the less likely we will contact you for clarification. |
| Total Emission (kg) | Number | Enter your annual total emission value. This value should be the total emission of the identified pollutant to the specified media for the reporting year. It should include any accidental releases if reportable to SEPA. If there was no emission this year but there is likely to be emissions in future years, then input zero (0) for Not Emitted (NE). Give values in kilograms, to three significant figures. NB: If you need to report accidental emission of a pollutant, ensure the value is included in the total emission and complete the Accidental release of pollutants section on the Return information tab. |
| ART/BRT/NE | Drop Down | The reporting threshold for each pollutant (specific to the medium) is shown in column C. Please indicate whether your total emission value was above the reporting threshold (ART), below the reporting threshold (BRT) or no emission (NE). |
| Qualification | Free text (unlimited) | Explain any differences from the previous year's data, especially if flagged by check fields. More information here helps us validate your data and reduces the likelihood of us having to contact you for clarification. |

If your facility has released additional pollutants to those pre-populated on your form in 2025, you should check the tables below to see whether they should be reported. If so, let us know on the Additional pollutants section of the Return information tab, and we will contact you for further information.

Pollutants and thresholds are given in the tables below.

Pollutant releases to air

Table 4: Pollutant releases to air - Inorganics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 7664-41-7 | NH3 | Ammonia | 1,000 |
| 1332-21-4 | - | Asbestos | 1 |
| 124-38-9 | CO2 | Carbon dioxide | 10,000,000 |
| 630-08-0 | CO | Carbon monoxide | 100,000 |
| 7647-01-0 | HCl | Hydrogen chloride | 10,000 |
| 74-90-8 | HCN | Hydrogen cyanide | 100 |
| 10024-97-2 | N2O | Nitrous oxide | 10,000 |
| 2551-62-4 | SF6 | Sulphur hexafluoride | 10 |

Table 5: Pollutant releases to air - Organics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 309-00-2 | - | Aldrin | 1 |
| 120-12-7 | - | Anthracene | 10 |
| 71-43-2 | - | Benzene | 1,000 |
| 50-32-8 | - | Benzo(a)pyrene | 1 |
| 106-99-0 | 1,3-Butadiene | Butadiene | 100 |
| 56-23-5 | Tetrachloromethane | Carbon tetrachloride | 10 |
| 57-74-9 | - | Chlordane | 1 |
| 143-50-0 | - | Chlordecone | 1 |
| 67-66-3 | Trichloromethane | Chloroform | 100 |
| 218-01-9 | - | Chrysene | 10 |
| 117-81-7 | DEHP | Di(2-ethylhexyl) phthalate | 10 |
| – | DDT – all isomers | Dichlorodiphenyltrichloroethane – all isomers | 1 |
| 60-57-1 | - | Dieldrin | 1 |
| 72-20-8 | - | Endrin | 1 |
| 107-06-2 | 1,2-Dichloroethane | Ethylene dichloride | 1,000 |
| 75-21-8 | 1,2-Epoxyethane | Ethylene oxide | 1,000 |
| 50-00-0 | Methanal | Formaldehyde | 10 |
| 76-44-8 | - | Heptachlor | 1 |
| 36355-01-8 | - | Hexabromobiphenyl | 0.1 |
| 118-74-1 | - | Hexachlorobenzene | 1 |
| 608-73-1 | - | Hexachlorocyclohexane – all isomers | 1 |
| 193-39-5 | - | Indeno (1, 2, 3-cd) pyrene | 1 |
| 58-89-9 | - | Lindane | 1 |
| 74-82-8 | - | Methane | 10,000 |
| 74-87-3 | Chloromethane | Methyl chloride | 1,000 |
| 71-55-6 | 1,1,1-Trichloroethane | Methyl chloroform | 10 |
| 75-09-2 | Dichloromethane (DCM) | Methylene chloride | 1,000 |
| 2385-85-5 | - | Mirex | 1 |
| 91-20-3 | - | Naphthalene | 100 |
| 87-86-5 | - | Pentachlorophenol | 1 |
| 608-93-5 | - | Pentachlorobenzene | 1 |
| 100-42-5 | - | Styrene | 100 |
| 79-34-5 | 1,1,2,2- Tetrachloroethane | Tetrachloroethane | 10 |
| 127-18-4 | - | Tetrachloroethylene | 100 |
| 108-88-3 | - | Toluene | 100 |
| 8001-35-2 | - | Toxaphene | 1 |
| 12002-48-1 | - | Trichlorobenzene – all isomers | 1 |
| 79-01-6 | - | Trichloroethylene | 1,000 |
| 75-01-4 | - | Vinyl chloride | 1,000 |

Table 6: Pollutant releases to air – Metal Compounds

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 7440-36-0 | Sb | Antimony | 1 |
| 7440-38-2 | As | Arsenic | 1 |
| 7440-43-9 | Cd | Cadmium | 1 |
| 7440-47-3 | Cr | Chromium | 10 |
| 7440-50-8 | Cu | Copper | 10 |
| 7439-92-1 | Pb | Lead | 100 |
| 7439-96-5 | Mn | Manganese | 10 |
| 7439-97-6 | Hg | Mercury | 1 |
| 7440-02-0 | Ni | Nickel | 10 |
| 7782-49-2 | Se | Selenium | 100 |
| 7440-62-2 | V | Vanadium | 10 |
| 7440-66-6 | Zn | Zinc | 100 |

Table 7: Pollutant releases to air – Other Pollutant Groups

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| - | - | Chlorine and total inorganic chlorine compounds – as HCl | 10,000 |
| - | CFCs | Chlorofluorocarbons (CFCs) | 1 |
| - | PCDDs + PCDFs | Dioxins and furans – as ITEQ | 0.00001 |
| - | PCDDs + PCDFs | Dioxins and furans – as WHO TEQ | 0.00001 |
| 7782-41-4 | - | Fluorine and total inorganic fluorine compounds – as HF | 1,000 |
| - | - | Halons | 1 |
| - | HCFCs | Hydrochlorofluorocarbons (HCFCs) | 1 |
| - | HFCs | Hydrofluorocarbons (HFCs) | 100 |
| - | NOx | Nitrogen oxides, NO and NO2 as NO2 | 100,000 |
| - | NMVOCs | Non-methane volatile organic compounds (NMVOCs) | 10,000 |
| - | PM10 | Particulate matter – PM10 and smaller | 10,000 |
| - |  | Particulate matter – total | 50,000 |
| - | - | Particulates – PM2.5 and smaller only | 1,000 |
| - | PFCs | Perfluorocarbons (PFCs) | 10 |
| 1336-36-3 | PCBs as WHO TEQ | Polychlorinated biphenyls – total as WHO TEQ | 0.00001 |
| 1336-36-3 | PCBs | Polychlorinated biphenyls (PCBs) | 0.1 |
| - | PAHs | Polycyclic aromatic hydrocarbons (PAH’s) | 1 |
| - | SOx | Sulphur oxides, SO2 and SO3 as SO2 | 100,000 |

Radionuclide emissions to air for premises with nuclear and non-nuclear authorisations

Table 8: Radionuclide emissions to air for premises with nuclear and non-nuclear authorisations

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold MBq/yr |
| --- | --- | --- | --- |
| - | - | Americium-241 | 0 |
| - | - | Argon-41 | 0 |
| - | - | Caesium-137 | 0 |
| - | - | Carbon-14 | 0 |
| - | - | Fluorine-18 | 0 |
| - | - | Iodine-125 | 0 |
| - | - | Iodine-129 | 0 |
| - | - | Iodine-131 | 0 |
| - | - | Krypton-85 | 0 |
| - | - | Lead-210 | 0 |
| - | - | Plutonium-alpha | 0 |
| - | - | Polonium-210 | 0 |
| - | - | Radium-226 | 0 |
| - | - | Radium-228 | 0 |
| - | - | Radon-222 | 0 |
| - | - | Ruthenium-106 | 0 |
| - | - | Sulphur-35 | 0 |
| - | - | Technetium-99m | 0 |
| - | - | Tritium | 0 |
| - | - | Uranium-alpha | 0 |
| - | - | Xenon-133 | 0 |
| - | - | Other Alpha-emitting Radionuclides | 0 |
| - | - | Other Non Alpha-emitting Radionuclides | 0 |
| - | - | Group of Two or More Specified Radionuclides | 0 |
| - | - | Other Radionuclides Not Listed | 0 |

Pollutant releases to water

Table 9: Pollutant releases to water - Inorganics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 1332-21-4 | - | Asbestos | 0.1 |

Table 10: Pollutant releases to water - Organics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 15792-60-8 | - | Alachlor | 0.1 |
| 309-00-2 | - | Aldrin | 0.0005 |
| 7664-41-7 | - | Ammonia (total) | 20 |
| 120-12-7 | - | Anthracene | 0.1 |
| 1912 24-9 | - | Atrazine | 0.05 |
| 35575-96-3 | - | Azamethiphos | 0.001 |
| 71-43-2 | - | Benzene | 10 |
| 191-24-2 | - | Benzo(g, h, i) perylene | 0.1 |
| 50-32-8 | - | Benzo(a)pyrene | 1 |
| 85-68-7 | BBP | Benzyl butyl phthalate | 0.1 |
| 80-05-7 | - | Bisphenol-A | 0.1 |
| 56-23-5 | Tetrachloromethane | Carbon tetrachloride | 1 |
| 57-74-9 | - | Chlordane | 0.1 |
| 143-50-0 | - | Chlordecone | 0.1 |
| 470-90-8 | - | Chlorfenvinphos | 0.1 |
| 67-66-3 | Trichloromethane | Chloroform | 5 |
| 2921-88-2 | - | Chlorpyrifos | 0.1 |
| 52315-07-8 | - | Cypermethrin | 0.005 |
| 52918-63-5 | - | Deltamethrin | 0.002 |
| 117-81-7 | DEHP | Di(2-ethylhexyl) phthalate | 0.1 |
| 333-41-5 | - | Diazinon | 0.01 |
| 50-29-3 | DDT | Dichlorodiphenyltrichloroethane – all isomers | 0.0005 |
| 120-83-2 | - | 2,4-Dichlorophenol | 0.1 |
| 94-75-7 | - | 2,4-Dichlorophenoxyacetic acid (2,4-D) – ester and non-ester | 0.1 |
| 62-73-7 | - | Dichlorvos | 0.0005 |
| 60-57-1 | - | Dieldrin | 0.0005 |
| 60-51-5 |  | Dimethoate | 0.01 |
| 330-541 | - | Diuron | 0.05 |
| 137512-74-4 | - | Emamectin Benzoate | 0.001 |
| 115-29-7 | - | Endosulfan | 0.0005 |
| 72-20-8 | - | Endrin | 0.0005 |
| 100-41-4 | - | Ethylbenzene | 10 |
| 107-06-2 | 1,2-Dichloroethane | Ethylene dichloride | 10 |
| 75-21-8 | 1, 2-epoxyethane | Ethylene oxide | 1 |
| 206-44-0 | - | Fluoranthene | 0.1 |
| 76-44-8 | - | Heptachlor (and Heptachlor epoxide) | 0.1 |
| 36355-1-8 | - | Hexabromobiphenyl | 0.1 |
| 25637-99-4 | - | Hexabromocyclododecane | 0.1 |
| 118-74-1 | - | Hexachlorobenzene | 0.01 |
| 87-68-3 | - | Hexachlorobutadiene | 0.1 |
| 608-73-1 | - | Hexachlorocyclohexane – all isomers | 0.01 |
| 465-73-6 | - | Isodrin | 0.0005 |
| 34123-59-6 | - | Isoproturon | 0.01 |
| 58-89-9 | - | Lindane | 0.1 |
| 330-55-2 | - | Linuron | 0.01 |
| 93-65-2 | - | Mecoprop | 1 |
| 75-09-2 | Dichloromethane (DCM) | Methylene chloride | 10 |
| 2385-85-5 | - | Mirex | 0.1 |
| 91-20-3 | - | Naphthalene | 1 |
| 608-93-5 | - | Pentachlorobenzene | 0.1 |
| 87-86-5 | - | Pentachlorophenol | 0.05 |
| - | - | Perfluoro octanyl sulphate (PFOS) | 0.1 |
| 52645-53-1 | - | Permethrin | 0.001 |
| 122-34-9 | - | Simazine | 0.01 |
| 83121-18-0 | - | Teflubenzuron | 0.001 |
| 140-66-9 | - | 4-tert-octylphenol | 1 |
| 127-18-4 | - | Tetrachloroethylene | 1 |
| 108-88-3 | - | Toluene | 10 |
| 8001-35-2 | - | Toxaphene | 0.1 |
| 12002-48-1 | - | Trichlorobenzene – all isomers | 0.01 |
| 79-01-6 | - | Trichloroethylene | 1 |
| 1582-09-8 | - | Trifluralin | 0.001 |
| 75-01-4 | - | Vinyl chloride | 1 |
| 1330-20-7 | Dimethylbenzene | Xylene – all isomers | 10 |

Table 11: Pollutant releases to water – Metal and compounds (expressed as mass of metal)

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 7440-38-2 | As | Arsenic | 5 |
| 7440-43-9 | Cd | Cadmium | 1 |
| 7440-47-3 | Cr | Chromium (total) | 20 |
| - | - | Chromium (III) | 20 |
| 18540-29-9 | - | Chromium (VI) | 20 |
| 7440-50-8 | Cu | Copper | 20 |
| 7439-89-6 | Fe | Iron | 1,000 |
| 7439-92-1 | Pb | Lead | 20 |
| 7439-97-6 | Hg | Mercury | 0.1 |
| 7439-96-5 | Mn | Manganese | 200 |
| 7440-02-0 | Ni | Nickel | 20 |
| 7440-66-6 | Zn | Zinc | 100 |

Table 12: Pollutant releases to water – Other pollutant groups (report as mass unless otherwise stated)

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| - | - | Brominated diphenylethers – total as Br | 0.1 |
| - | - | Chlorides – total as Cl | 2,000,000 |
| - | - | Cyanides – total as CN | 50 |
| - | F | Fluorides – total as F | 2,000 |
| - | AOX | Halogenated organic compounds – total as AOX | 1,000 |
| - | - | Nitrogen – total as N | 50,000 |
| - | - | Nonylphenol ethoxylates | 1 |
| - | - | Nonylphenols | 1 |
| - | - | Nonylphenol ethoxylates and nonylphenols | 1 |
| - | - | Octylphenol ethoxylates | 1 |
| - | - | Octylphenols | 1 |
| - | - | Octylphenol ethoxylates and octylphenols | 1 |
| - | Sn | Organotin compounds – total as Sn | 5 |
| - | PCDDs + PCDFs | Dioxins and Furans as ITEQ | 0.0001 |
| - | PCDDs + PCDFs | Dioxins and Furans as WHO TEQ | 0.0001 |
| 108-95-2 | - | Phenols – total as C | 20 |
| - | - | Phosphorus – total as P | 5,000 |
| 1336-36-3 | PCBs | Polychlorinated biphenyls | 0.001 |
| 1336-36-3 | PCBs as WHO TEQ | Polychlorinated biphenyls – total as WHO TEQ | 0.002 |
| - | PAHs | Polycyclic aromatic hydrocarbons (PAHs) | 1 |
| - | TOC | Total organic carbon or COD/3 | 50,000 |
| 85535-84-8 | - | Chloro-alkanes (C10-C13) | 1 |
| 3380-34-5 | - | Triclosan | 0.1 |
| 56573-85-4 | TBT compounds | Tributyltin compounds | 0.005 |
| 668-38-8 | TPT compounds | Triphenyltin compounds | 0.1 |

Radionuclide emissions to water for premises with nuclear and non-nuclear authorisations

Table 13: Radionuclide emissions to water for premises with nuclear and non-nuclear authorisations

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold MBq/yr |
| --- | --- | --- | --- |
| - | - | Americium-241 | 0 |
| - | - | Antimony-125 | 0 |
| - | - | Caesium-134 | 0 |
| - | - | Caesium-137 | 0 |
| - | - | Carbon-14 | 0 |
| - | - | Cerium-144 | 0 |
| - | - | Cobalt-60 | 0 |
| - | - | Curium-242 | 0 |
| - | - | Iodine-129 | 0 |
| - | - | Lead-210 | 0 |
| - | - | Neptunium-237 | 0 |
| - | - | Niobium-95 | 0 |
| - | - | Polonium-210 | 0 |
| - | - | Plutonium-alpha | 0 |
| - | - | Plutonium-241 | 0 |
| - | - | Radium-226 | 0 |
| - | - | Radium-228 | 0 |
| - | - | Ruthenium-106 | 0 |
| - | - | Strontium-90 | 0 |
| - | - | Sulphur-35 | 0 |
| - | - | Technetium-99m | 0 |
| - | - | Thorium-230 | 0 |
| - | - | Thorium-232 | 0 |
| - | - | Tritium | 0 |
| - | - | Uranium-alpha | 0 |
| - | - | Yttrium-90 | 0 |
| - | - | Zirconium-95 | 0 |
| - | - | Other Alpha-emitting Radionuclides | 0 |
| - | - | Other Non Alpha-emitting Radionuclides | 0 |
| - | - | Group of Two or More Specified Radionuclides | 0 |
| - | - | Other Radionuclides Not Listed | 0 |

Pollutant releases to land

The reportable substances and reportable thresholds shown in the table below apply only to pollutants in waste which is subject to the disposal operations of land treatment or deep injection.

Table 14: Pollutant releases to land – Inorganics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 1332-21-4 | - | Asbestos | 0.1 |

Table 15: Pollutant releases to land – Organics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 15972-60-8 | - | Alachlor | 1 |
| 309-00-2 | - | Aldrin | 1 |
| 120-12-7 | - | Anthracene | 1 |
| 1912-24-9 | - | Atrazine | 1 |
| 71-43-2 | - | Benzene | 200 (as BTEX) |
| 57-74-9 | - | Chlordane | 1 |
| 143-50-0 | - | Chlordecone | 1 |
| 470-90-8 | - | Chlorfenvinphos | 1 |
| 2921-88-2 | - | Chlorpyrifos | 1 |
| 50-29-3 | DDT | Dichlorodiphenyltrichloroethane – all isomers | 1 |
| 75-09-2 | Dichloromethane (DCM) | Methylene chloride | 10 |
| 60-57-1 | - | Dieldrin | 1 |
| 117-81-7 | DEHP | Di(2-ethylhexyl)phthalate | 1 |
| 330-54-1 | - | Diuron | 1 |
| 115-29-7 | - | Endosulfan | 1 |
| 72-20-8 | - | Endrin | 1 |
| 100-41-4 | - | Ethylbenzene | 200 (as BTEX) |
| 107-06-2 | 1,2-Dichloroethane | Ethylene dichloride | 10 |
| 75-21-8 | 1,2-Epoxyethane | Ethylene oxide | 10 |
| 76-44-8 | - | Heptachlor | 1 |
| 36355-01-8 | - | Hexabromobiphenyl | 0.1 |
| 118-74-1 | - | Hexachlorobenzene | 1 |
| 87-68-3 | - | Hexachlorobutadiene | 1 |
| 608-73-1 | - | Hexachlorocyclohexane – all isomers | 1 |
| 34123-59-6 | - | Isoproturon | 1 |
| 58-89-9 | - | Lindane | 1 |
| 2385-85-5 | - | Mirex | 1 |
| 91-20-3 | - | Naphthalene | 10 |
| 87-86-5 | - | Pentachlorophenol | 1 |
| 608-93-5 | - | Pentachlorobenzene | 1 |
| 122-34-9 | - | Simazine | 1 |
| 108-88-3 | - | Toluene | 200 (as BTEX) |
| 8001-35-2 | - | Toxaphene | 1 |
| 1582-09-8 | - | Trifluralin | 1 |
| 75-01-4 | - | Vinyl chloride | 10 |
| 1330-20-7 | Dimethylbenzene | Xylene – all isomers | 200 (as BTEX) |

Table 16: Pollutant releases to land – Metal and compounds (express as mass of metal)

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 7440-38-2 | As | Arsenic | 5 |
| 7440-43-9 | Cd | Cadmium | 5 |
| 7440-47-3 | Cr | Chromium | 50 |
| 7440-50-8 | Cu | Copper | 50 |
| 7439-92-1 | Pb | Lead | 20 |
| 7439-97-6 | Hg | Mercury | 1 |
| 7440-02-0 | Ni | Nickel | 20 |
| 7440-66-6 | Zn | Zinc | 100 |

Table 17: Pollutant releases to land – Other pollutant groups (report as total mass unless otherwise stated)

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| - | Br | Brominated diphenylethers – Total as Br | 1 |
| 16887-00-6 | Cl | Chlorides – total as Cl | 2,000,000 |
| - | - | Cyanides – as Total CN | 50 |
| 16984-48-8 | F | Fluorides – as Total F | 2,000 |
| - | AOX | Halogenated organic compounds – as total AOX | 1,000 |
| - | - | Nitrogen - as Total N | 50,000 |
| - | - | Nonylphenols | 1 |
| - | - | Nonylphenol ethoxylates | 1 |
| - | - | Nonylphenol ethoxylates and nonylphenols | 1 |
| - | - | Octylphenols | 1 |
| - | - | Octylphenol ethyoxylates | 1 |
| - | - | Octylphenols and octylphenol ethyoxylates | 1 |
| - | Sn | Organo-tin compounds – as Total Sn | 50 |
|  | PCDDs + PCDFs | Dioxins and furans as I-TEQ | 0.0001 |
| - | PCDDs + PCDFs | Dioxins and furans as WHO-TEQ | 0.0001 |
| 108-95-2 | - | Phenols – as Total C | 20 |
| - | P | Phosphorus – as Total P | 5,000 |
| 1336-36-3 | PCBs | Polychlorinated biphenyls | 0.1 |
| - | - | Polychlorinated biphenyls as WHO-TEQ | 0.0002 |
| - | PAHs | Polycyclic aromatic hydrocarbons (PAHs) | 5 |
| 85535-84-8 | - | Chloro-alkanes (C10-C13) | 1 |
| 56573-85-4 | TBT compounds | Tributyltin compounds | 1 |
| 668-38-8 | TPT | Triphenyltin compounds | 1 |

As BTEX – single pollutants are reported if the threshold for BTEX (the sum parameter of benzene, toluene, ethyl benzene, xylenes) is exceeded.

Pollutant releases to wastewater

Table 18: Pollutant releases to wastewater - Inorganics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 1332-21-4 | - | Asbestos | 0.1 |

Table 19: Pollutant releases to wastewater - Organics

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 15792-60-8 | - | Alachlor | 0.1 |
| 309-00-2 | - | Aldrin | 0.0005 |
| 7664-41-7 | - | Ammonia (total) | 20 |
| 120-12-7 | - | Anthracene | 0.1 |
| 1912-24-9 | - | Atrazine | 0.05 |
| 35575-96-3 | - | Azamethiphos | 0.001 |
| 71-43-2 | - | Benzene | 10 |
| 50-32-8 | - | Benzo(a)pyrene | 1 |
| 191-24-2 | - | Benzo (g,h,i) perylene | 0.1 |
| 85-68-7 | BBP | Benzyl butyl phthalate | 0.1 |
| 80-05-7 | - | Bisphenol-A | 0.1 |
| 56-23-5 | Tetrachloromethane | Carbon tetrachloride | 1 |
| 57-74-9 | - | Chlordane | 0.1 |
| 143-50-0 | - | Chlordecone | 0.1 |
| 470-90-8 | - | Chlorfenvinphos | 0.1 |
| 67-66-3 | Trichloromethane | Chloroform | 5 |
| 2921-88-2 | - | Chlorpyrifos | 0.1 |
| 52315-07-8 | - | Cypermethrin | 0.005 |
| 52918-63-5 | - | Deltamethrin | 0.002 |
| 117-81-7 | DEHP | Di(2-ethylhexyl) phthalate | 0.1 |
| 333-41-5 | - | Diazinon | 0.01 |
| 50-29-3 | DDT | Dichlorodiphenyltrichloroethane – all isomers | 0.0005 |
| 120-83-2 | - | 2,4-Dichlorophenol | 0.1 |
| 94-75-7 | - | 2,4-Dichlorophenoxyacetic acid (2,4-D) – ester and non-ester | 0.1 |
| 62-73-7 | - | Dichlorvos | 0.0005 |
| 60-57-1 | - | Dieldrin | 0.0005 |
| 60-51-5 |  | Dimethoate | 0.01 |
| 330-54-1 | - | Diuron | 0.05 |
| 137512-74-4 | - | Emamectin benzoate | 0.001 |
| 115-29-7 | - | Endosulfan | 0.0005 |
| 72-20-8 | - | Endrin | 0.0005 |
| 100-41-4 | - | Ethylbenzene | 10 |
| 107-06-2 | 1,2-Dichloroethane | Ethylene dichloride | 10 |
| 75-21-8 | 1, 2-epoxyethane | Ethylene oxide | 1 |
| 206-44-0 | - | Fluoranthene | 0.1 |
| 76-44-8 | - | Heptachlor | 0.1 |
| 36355-1-8 | - | Hexabromobiphenyl | 0.1 |
| 25637-99-4 | - | Hexabromocyclododecane | 0.1 |
| 118-74-1 | - | Hexachlorobenzene | 0.01 |
| 87-68-3 | - | Hexachlorobutadiene | 0.1 |
| 608-73-1 | - | Hexachlorocyclohexane – all isomers | 0.01 |
| 465-73-6 | - | Isodrin | 0.0005 |
| 34123-59-6 | - | Isoproturon | 0.01 |
| 58-89-9 | - | Lindane | 0.1 |
| 330-55-2 | - | Linuron | 0.01 |
| 93-65-2 | - | Mecoprop | 1 |
| 75-09-2 | Dichloromethane (DCM) | Methylene chloride | 10 |
| 2385-85-5 | - | Mirex | 0.1 |
| 91-20-3 | - | Naphthalene | 1 |
| 608-93-5 | - | Pentachlorobenzene | 0.1 |
| 87-86-5 | - | Pentachlorophenol | 0.05 |
| - | - | Perfluoro octanyl sulphate (PFOS) | 0.1 |
| 52645-53-1 | - | Permethrin | 0.001 |
| 122-34-9 | - | Simazine | 0.01 |
| 83121-18-0 | - | Teflubenzuron | 0.001 |
| 140-66-9 | - | 4-tert-octylphenol | 1 |
| 127-18-4 | - | Tetrachloroethylene | 1 |
| 108-88-3 | - | Toluene | 10 |
| 8001-35-2 | - | Toxaphene | 0.1 |
| 12002-48-1 | - | Trichlorobenzene – all isomers | 0.01 |
| 79-01-6 | - | Trichloroethylene | 1 |
| 3380-34-5 | - | Triclosan | 0.1 |
| 1582-09-8 | - | Trifluralin | 0.001 |
| 75-01-4 | - | Vinyl chloride | 1 |
| 1330-20-7 | Dimethylbenzene | Xylene – all isomers | 10 |

Table 20: Pollutant releases to wastewater – Metals and compounds (express as mass of metal)

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| 7440-38-2 | As | Arsenic | 5 |
| 7440-43-9 | Cd | Cadmium | 1 |
| 7440-47-3 | Cr | Chromium (total) | 20 |
| - | - | Chromium (III) | 20 |
| 18540-29-9 | - | Chromium (VI) | 20 |
| 7440-50-8 | Cu | Copper | 20 |
| 7439-89-6 | Fe | Iron | 1,000 |
| 7439-92-1 | Pb | Lead | 20 |
| 7439-97-6 | Hg | Mercury | 0.1 |
| 7439-96-5 | Mn | Manganese | 200 |
| 7440-02-0 | Ni | Nickel | 20 |
| 7440-66-6 | Zn | Zinc | 100 |

Table 21: Pollutant releases to wastewater – Other pollutant groups (report as total mass unless otherwise stated)

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold kg/yr |
| --- | --- | --- | --- |
| - | - | Brominated diphenylethers – total as Br | 0.1 |
| - | - | Chlorides – total as Cl | 2,000,000 |
| - | - | Cyanides – total as CN | 50 |
| - | F | Fluorides – total as F | 2,000 |
| - | AOX | Halogenated organic compounds – total as AOX | 1,000 |
| - | - | Nitrogen – total as N | 50,000 |
| - | - | Nonylphenol | 1 |
| - | - | Nonylphenol ethoxylates | 1 |
| - | - | Nonylphenol ethoxylates and nonylphenols | 1 |
| - | - | Octylphenol ethoxylates | 1 |
| - | - | Octylphenols | 1 |
| - | - | Octylphenol ethoxylates and octylphenols | 1 |
| - | Sn | Organic tin compounds – total as Sn | 5 |
| - | PCDDs + PCDFs | Dioxins and furans as I-TEQ | 0.0001 |
| - | PCDDs + PCDFs | Dioxins and furans as WHO-TEQ | 0.0001 |
| - | - | Phenols – total as C | 20 |
| - | - | Phosphorus – total as P | 5,000 |
| 1336-36-3 | PCBs | Polychlorinated biphenyls | 0.001 |
| - | - | Polychlorinated biphenyls as WHO-TEQ | 0.002 |
| - | PAHs | Polycyclic aromatic hydrocarbons (PAHs) | 1 |
| 85535-84-8 | - | Chloro-alkanes | 1 |
| - | TOC | Total organic carbon or COD/3 | 50,000 |
| 56573-85-4 | TBT compounds | Tributyltin compounds | 0.005 |
| 668-38-8 | TPT compounds | Triphenyltin compounds | 0.1 |

Radionuclide emissions to wastewater for premises with nuclear and non-nuclear authorisations

Table 22: Radionuclide emissions to wastewater for premises with nuclear and non-nuclear authorisations

| CAS No. | Pollutant (Systemic name) | Pollutant (common name) | Threshold MBq/yr |
| --- | --- | --- | --- |
| - | - | Americium-241 | 0 |
| - | - | Antimony-125 | 0 |
| - | - | Caesium-134 | 0 |
| - | - | Caesium-137 | 0 |
| - | - | Carbon-14 | 0 |
| - | - | Cerium-144 | 0 |
| - | - | Chromium-51 | 0 |
| - | - | Cobalt-57 | 0 |
| - | - | Cobalt-58 | 0 |
| - | - | Cobalt-60 | 0 |
| - | - | Curium-242 | 0 |
| - | - | Erbium-169 | 0 |
| - | - | Fluorine-18 | 0 |
| - | - | Gallium-67 | 0 |
| - | - | Indium-111 | 0 |
| - | - | Iodine-123 | 0 |
| - | - | Iodine-125 | 0 |
| - | - | Iodine-129 | 0 |
| - | - | Iodine-131 | 0 |
| - | - | Lead-210 | 0 |
| - | - | Neptunium-237 | 0 |
| - | - | Niobium-95 | 0 |
| - | - | Phosphorus-32 | 0 |
| - | - | Phosphorus-33 | 0 |
| - | - | Plutonium-alpha | 0 |
| - | - | Plutonium-241 | 0 |
| - | - | Polonium-210 | 0 |
| - | - | Radium-226 | 0 |
| - | - | Radium-228 | 0 |
| - | - | Ruthenium-106 | 0 |
| - | - | Samarium-153 | 0 |
| - | - | Selenium-75 | 0 |
| - | - | Sodium-22 | 0 |
| - | - | Strontium-89 | 0 |
| - | - | Strontium-90 | 0 |
| - | - | Sulphur-35 | 0 |
| - | - | Technetium-99m | 0 |
| - | - | Thallium-201 | 0 |
| - | - | Thorium-230 | 0 |
| - | - | Thorium-232 | 0 |
| - | - | Tritium | 0 |
| - | - | Uranium-alpha | 0 |
| - | - | Yttrium-90 | 0 |
| - | - | Zirconium-95 | 0 |
| - | - | Other Alpha-emitting Radionuclides | 0 |
| - | - | Other Non Alpha-emitting Radionuclides | 0 |
| - | - | Group of Two or More Specified Radionuclides | 0 |
| - | - | Other Radionuclides Not Listed | 0 |

Section D – Off-site transfers of waste

You should report the mass of waste which is removed from the facility by pipe, tanker or lorry, where the annual total tonnage for that type of waste is above a threshold value:

Table 23: Threshold values for Hazardous and Non-Hazardous waste

| Type of waste | Threshold value (tonnes) |
| --- | --- |
| Hazardous | 2 |
| Non-hazardous | 2,000 |

For each type of waste, the deciding factor on whether to report is the total mass transferred off site during the reporting year; the waste may have been split between different treatment facilities or countries and been handled in different ways.

Where your waste water is treated by an independently operated wastewater treatment plant (IOWWTP), such as an industrial wastewater treatment plant covered by Schedule 20 of the Environmental Authorisations (Scotland) Regulations, you should record the transfer as waste in tonnes in this section of the form.

For both hazardous waste (where the total transferred off-site exceeds 2t) and non-hazardous waste (where the total transferred off-site exceeds 2,000t) you must tell us the total tonnage which was sent for disposal and the total sent for recovery. In each case you should tell us:

Table 24: Waste - Field information and data requirements

| Name of field | Field type | What you should provide |
| --- | --- | --- |
| Measurement type (M, C or E) | Drop down | Indicate whether your emission value is Measured, Calculated or Estimated. Where your data is obtained using weighing records report “measured”. |
| Method type | Drop down | Use the option which best matches the method you used to produce your total waste value. Where available the method must include the UK or international quality standard reference number(s). |
| Method description | Free text (unlimited) | Explain how you produced the emission value, including any calculations. More detail helps validate your data and reduces follow-up inquiries. |
| Value (tonnes) | Number | Give values as normal wet waste in tonnes, to 3 significant figures. |
| Qualification | Free text (unlimited) | Explain any differences from the previous year's data, especially if flagged by check fields. More information here helps us validate your data and reduces the likelihood of us having to contact you for clarification. |

If any hazardous waste was transferred outside the UK, you must provide details of the waste destination (name and address of recoverer/disposer and the address of the actual site of recovery/disposal). Complete the Outside UK waste transfers section of the Return information and we will contact you to collect the detailed information.

Section E – Waste input data					This section is not in use.

Section F – Large Combustion Plant Directive (LCPD)	This section is not in use.

Section G – Voluntary information				This section is not in use.

Section H – Activities which require emissions and waste transfers to be reported for any Facility carrying them out.

Table 25: Activities – Energy Sector

| SPRI Code | Activity | PRTR Capacity Threshold |
| --- | --- | --- |
| 1 (a) | Mineral oil and gas refineries | None |
| 1 (b) | Installations for gasification and liquefaction | None |
| 1 (c) | Thermal power stations and other combustion installations | With a heat input of 50 megawatts (MW) |
| 1 (d) | Coke ovens | None |
| 1 (e) | Coal rolling mills | With capacity of 1 tonne per hour |
| 1 (f) | Installations for the manufacture of coal products and sole smokeless fuel | None |

Table 26: Activities – Production and processing of metals

| SPRI Code | Activity | PRTR Capacity Threshold |
| --- | --- | --- |
| 2 (a) | Metal ore (including sulphide ore) roasting or sintering installations | None |
| 2 (b) | Installations for the production of pig iron or steel (primary or secondary melting) including continuous casting | With a capacity of 2.5 tonnes per hour |
| 2 (c) | Installations for the processing of ferrous metals: |  |
| 2 (c) (i) | Hot rolling mills | With a capacity of 20 tonnes of crude steel per hour |
| 2 (c) (ii) | Smitheries with hammers | With an energy of 50 kilojoules per hammer, where the calorific power used exceeds 20 MW |
| 2 (c) (iii) | Application of protective fused metal coats | With an input of 2 tonnes of crude steel per hour |
| 2 (d) | Ferrous metal foundries | With a production capacity of 20 tonnes per day |
| 2 (e) | Installations: |  |
| 2 (e) (i) | For the production of non-ferrous crude metals from ore, concentrates or secondary raw materials by metallurgical, chemical or electrolytic processes | None |
| 2 (e) (ii) | For the smelting, including the alloying, of non-ferrous metals, including recovered products (refining, foundry casting, etc.) | With a melting capacity of 4 tonnes per day for lead and cadmium or 20 tonnes per day for all other metals |
| 2 (f) | Installations for surface treatment of metals and plastic materials using an electrolytic or chemical process | Where the volume of the treatment vats equals 30 m3 |

Table 27: Activities – Mineral industry

| SPRI Code | Activity | PRTR Capacity Threshold | Scotland-specific requirements |
| --- | --- | --- | --- |
| 3 (a) | Underground mining and related operations | None | Note while this activity is not directly regulated by SEPA, other related activities are e.g. water discharges, mobile crushing plants |
| 3 (b) | Opencast mining and quarrying | Where the surface of the area being mined equals 25 hectares | Note while this activity is not directly regulated by SEPA, other related activities are e.g. water discharges, mobile crushing plants |
| 3 (c) | Installations for the production of: |  |  |
| 3 (c) (i) | Cement clinker in rotary kilns | With a production capacity of 500 tonnes per day | None |
| 3 (c) (ii) | Lime in rotary kilns | With a production capacity of 50 tonnes per day | None |
| 3 (c) (iii) | Cement clinker or lime in other furnaces | With a production capacity of 50 tonnes per day | None |
| 3 (d) | Installations for the production of asbestos and the manufacture of asbestos-based products | None | None |
| 3 (e) | Installations for the manufacture of glass, including glass fibre | With a melting capacity of 20 tonnes per day | None |
| 3 (f) | Installations for melting mineral substances, including the production of mineral fibres | With a melting capacity of 20 tonnes per day | None |
| 3 (g) | Installations for the manufacture of ceramic products by firing, in particular roofing tiles, bricks, refractory bricks, tiles, stoneware or porcelain | With a production capacity of 75 tonnes per day, or with a kiln capacity of 4 m3 and with a setting density per kiln of 300 kg/m3 | None |

Table 28: Activities – Chemical industry

| SPRI Code | Activity |
| --- | --- |
| 4 (a) | Chemical installations for the production on an industrial scale of basic organic chemicals, such as: |
| 4 (a) (i) | Simple hydrocarbons (linear or cyclic, saturated or unsaturated, aliphatic or aromatic) |
| 4 (a) (ii) | Oxygen-containing hydrocarbons such as alcohols, aldehydes, ketones, carboxylic acids, esters, acetates, ethers, peroxides, epoxy resins |
| 4 (a) (iii) | Sulphurous hydrocarbons |
| 4 (a) (iv) | Nitrogenous hydrocarbons such as amines, amides, nitrous compounds, nitro compounds or nitrate compounds, nitriles, cyanates, isocyanates |
| 4 (a) (v) | Phosphorus-containing hydrocarbons |
| 4 (a) (vi) | Halogenic hydrocarbons |
| 4 (a) (vii) | Organometallic compounds |
| 4 (a) (viii) | Basic plastic materials (polymers, synthetic fibres and cellulose-based fibres) |
| 4 (a) (ix) | Synthetic rubbers |
| 4 (a) (x) | Dyes and pigments |
| 4 (a) (xi) | Surface-active agents and surfactants |
| 4 (b) | Chemical installations for the production on an industrial scale of basic inorganic chemicals, such as: |
| 4 (b) (i) | Gases, such as ammonia, chlorine or hydrogen chloride, fluorine or hydrogen fluoride, carbon oxides, sulphur compounds, nitrogen oxides, hydrogen, sulphur dioxide, carbonyl chloride |
| 4 (b) (ii) | Acids, such as chromic acid, hydrofluoric acid, phosphoric acid, nitric acid, hydrochloric acid, sulphuric acid, oleum, sulphurous acids |
| 4 (b) (iii) | Bases, such as ammonium hydroxide, potassium hydroxide, sodium hydroxide |
| 4 (b) (iv) | Salts, such as ammonium chloride, potassium chlorate, potassium carbonate, sodium carbonate, perborate, silver nitrate |
| 4 (b) (v) | Non-metals, metal oxides or other inorganic compounds such as calcium carbide, silicon, silicon carbide |
| 4 (c) | Chemical installations for the production on an industrial scale of phosphorous-, nitrogen- or potassium-based fertilisers (simple or compound fertilisers) |
| 4 (d) | Chemical installations for the production on an industrial scale of basic plant health products and of biocides |
| 4 (e) | Installations using a chemical or biological process for the production on an industrial scale of basic pharmaceutical product |
| 4 (f) | Installations for the production on an industrial scale of explosives and pyrotechnic products |

Table 29: Activities – Waste and wastewater management

| SPRI Code | Activity | PRTR Capacity Threshold | Scotland-specific requirements |
| --- | --- | --- | --- |
| 5 (a) | Installations for the recovery or disposal of hazardous waste | Receiving 10 tonnes per day | All facilities with a capacity to accept at least 10 tonnes per day for the recovery and disposal of hazardous waste |
| 5 (b) | Installations for the incineration of non-hazardous waste in the scope of Directive 2000/76/EC of the European Parliament and of the Council of 4 December 2000 on the incineration of waste | With a capacity of 3 tonnes per hour | None |
| 5 (c) | Installations for the disposal of non-hazardous waste | With a capacity of 50 tonnes per day | All facilities with a capacity to accept at least 50 tonnes per day for the recovery and disposal of non-hazardous waste |
| 5 (d) | Landfills (excluding landfills of inert waste and landfills, which were definitely closed before 16.7.2001 or for which the after-care phase required by the competent authorities according to Article 13 of Council Directive 1999/31/EC of 26 April 1999 on the landfill of waste has expired) | Receiving 10 tonnes per day or with a total capacity of 25,000 tonnes | None |
| 5 (d) (i) | Landfills of inert waste |  | Receiving 10 tonnes per day or with a total capacity of 25,000 tonnes |
| 5 (e) | Installations for the disposal or recycling of animal carcasses and animal waste | With a treatment capacity of 10 tonnes per day |  |
| 5 (f) | Urban waste-water treatment plants: |  |  |
| 5 (f) (i) | Urban waste-water treatment plants | With a capacity of 100,000 population equivalents | None |
| 5 (f) (ii) | Urban waste-water treatment plants with a capacity of between 15,000 and 100,000 population equivalents |  | All sewage treatment works and equivalent industrial discharges with a design population equivalent of 15,000 or more (where population equivalent has the meaning given in the Urban Waste Water Treatment (Scotland) Regulations 1994). |
| 5 (g) | Independently operated industrial waste-water treatment plants which serve one or more activities of this list | With a capacity of 10,000 m3 per day | ​With a capacity of 2,000 m3 per day |
| 5 (h) | Recovery, or a mix of recovery and disposal, of non-hazardous waste |  | With a capacity exceeding 75 tonnes per day |
| 5 (h) (i) | Disposal of non-hazardous waste: biological treatment; |  |  |
| 5 (h) (ii) | Disposal of non-hazardous waste: physico-chemical treatment; |  |  |
| 5 (h) (iii) | Disposal of non-hazardous waste: pre-treatment of waste for incineration or co-incineration; |  |  |
| 5 (h) (iv) | Disposal of non-hazardous waste: treatment of slags and ashes; |  |  |
| 5 (h) (v) | When the only waste treatment activity carried out is anaerobic digestion. |  | With a capacity of 100 tonnes per day. |

Table 30: Activities – Paper and wood production and processing

| SPRI Code | Activity | PRTR Capacity Threshold |
| --- | --- | --- |
| 6 (a) | Industrial plants for the production of pulp from timber or similar fibrous materials | None |
| 6 (b) | Industrial plants for the production of paper and board and other primary wood products (such as chipboard, fibreboard and plywood) | With a production capacity of 20 tonnes per day |
| 6 (c) | Industrial plants for the preservation of wood and wood products with chemicals | With a production capacity of 50m3 per day |

Table 31: Activities – Intensive livestock production and aquaculture

| SPRI Code | Activity | PRTR Capacity Threshold |
| --- | --- | --- |
| 7 (a) | Installations for the intensive rearing of poultry or pigs: |  |
| 7 (a) (i) | Installations for the intensive rearing of poultry | With 40,000 places for poultry |
| 7 (a) (ii) | Installations for the intensive rearing of pigs | With 2,000 places for production pigs (over 30 kg) |
| 7 (a) (iii) | Installations for the intensive rearing of pigs | With 750 places for sows |
| 7 (b) | Intensive aquaculture: |  |
| 7 (b) (i) | Intensive aquaculture | With a production capacity of 1,000 tonnes of fish or shellfish per year |
| 7 (b) (ii) | Intensive aquaculture | With a production capacity below 1,000 tonnes of fish or shellfish per year |

Table 32: Activities – Animal and vegetable products from the food and beverage sector

| SPRI Code | Activity | PRTR Capacity Threshold |
| --- | --- | --- |
| 8 (a) | Slaughterhouses | With a carcass production capacity of 50 tonnes per day |
| 8 (b) | Treatment and processing intended for the production of food and beverage products from: |  |
| 8 (b) (i) | Animal raw materials (other than milk) | With a finished product production capacity of 75 tonnes per day |
| 8 (b) (ii) | Vegetable raw materials | With a finished product production capacity of 300 tonnes per day (average value on a quarterly basis) |
| 8 (c) | Treatment and processing of milk | With a capacity to receive 200 tonnes of milk per day (average value on an annual basis) |

Table 33: Activities – Other activities

| SPRI Code | Activity | PRTR Capacity Threshold |
| --- | --- | --- |
| 9 (a) | Plants for the pre-treatment (operations such as washing, bleaching, mercerisation) or dyeing of fibres or textiles | With a treatment capacity of 10 tonnes per day |
| 9 (b) | Plants for the tanning of hides and skins | With a treatment capacity of 12 tonnes of finished product per day |
| 9 (c) | Installations for the surface treatment of substances, objects or products using organic solvents, in particular for dressing, printing, coating, degreasing, waterproofing, sizing, painting, cleaning or impregnating | With a consumption capacity of 150 kg per hour or 200 tonnes per year |
| 9 (d) | Installations for the production of carbon (hard-burnt coal) or electro-graphite by means of incineration or graphitisation | None |
| 9 (e) | Installations for the building of, and painting or removal of paint from ships | With a capacity for ships 100 m long |

Table 34: Radioactive substances activities

| SPRI Code | Activity | PRTR Capacity Threshold |
| --- | --- | --- |
| 10 (a) | All nuclear installations (including plants undergoing decommissioning) and all non-nuclear installations holding authorisation for air, water and waste water releases: Radioactive substances activity - nuclear | None |
| 10 (b) | All nuclear installations (including plants undergoing decommissioning) and all non-nuclear installations holding authorisation for air, water and waste water releases: Radioactive substances activity - non-nuclear | None |

<Report date here (month, year)>

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk